3GPP TSG-RAN WG4 Meeting #115	R4-2508010

# Malta, Malta,  19th May – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0026 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the new NTN L-bands to TR 38.863 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Aalyria, THALES, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_combinedLband-Core |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of the new NTN L-bands to TR 38.863 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of relevant changes and new sections to TR 38.863 to introduce the new NTN Combined L-bands n253, n251, n250, including corresponding regulatory aspects, RF requirements and a new appendix C.2 capturing power back-off (A-MPR) simulations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information pertaining the new Combined L-bands n253, n251 and n250 will not be captured in the TR, causing a significant gap in the background information surrounding Combined L-bands and the outcomes of the WI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2.5 (new), 7.2.2, 7.2.3, 7.2.3.1 (new), 7.2.4.1, 7.2.4.2, 7.4.1.1 (new), 7.4.2.2.1, 7.4.2.2.2, 7.4.2.2.11 (new), 7.4.2.2.11.1 (new), 7.4.2.2.11.2 (new), 7.4.2.2.12 (new), 7.4.2.2.12.1 (new), Annex C.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** FIRST CHANGED SECTION **********

## 5.1	ITU-R

The following services are among those defined in the ITU-R Radio Regulations [3]:

-	Fixed (1.20): A radiocommunication service between specified fixed points.

-	Fixed satellite (1.21) : A radiocommunication service between earth stations at given positions, when one or more satellites are used; the given position may be a specified fixed point or any fixed point within specified areas; in some cases this service includes satellite-to-satellite links, which may also be operated in the inter-satellite service; the fixed-satellite service may also include feeder links for other space radiocommunication services.

-	Mobile (1.24): radiocommunication service between mobile and land stations, or between mobile stations (CV).

-	Mobile satellite (1.25): A radiocommunication service:

-	between mobile earth stations and one or more space stations, or between space stations used by this service; or

-	between mobile earth stations by means of one or more space stations.

This service may also include feeder links necessary for its operation.

Based on the ITU-R Radio Regulations [3], the following frequency ranges are allocated to MSS and have been identified as first candidate bands for NTN satellite operations:

-	S-band: UL: 1980–2010 MHz / DL: 2170–2200 MHz.

-	L-band: UL: 1626.5-1660.5 MHz / DL: 1525-1559 MHz.

- For providing mobile services through HAPS, current ITU-R Radio Regulations [3] allow the use of frequency ranges 1885–1980 MHz, 2010-2025 MHz and 2110–2170 MHz by HAPS. Additional spectrum may be allocated for HAPS in 2023 (see clause 5.3).

- The following additional frequency ranges allocated to MSS based on the ITU-R Radio Regulations [3] have been identified as additional bands for NTN satellite operations:

- Extended L-band: UL: 1668-1675 MHz / DL: 1518-1525 MHz.

********** NEXT CHANGED SECTION **********

### 5.2.5	NTN Satellite band: UL 1668-1675 MHz / DL 1518-1525 MHz)

The purpose of this section is to collect information from existing regulatory documents concerning regulatory rules applicable to this NTN Extended L-band.

NOTE1:	As per agreement in RAN4#108, the following assumption is captured:

For the time being, re-use TN-NTN coexistence assumptions from TR 38.863, which would yield a total separation distance of 5250 m from TN BS site and NTN UE, at least for low-gain land-based NTN UE, compatible with smartphone and IoT use cases.

NOTE2:	At RAN#108-bis Xiamen, an LS Reply was received from ETSI SES SCN (R4-2317932), with the following content: 

“ETSI TC SES would like to thank 3GPP RAN4 for its LS on the 3GPP NTN requirements and the potential impact on the specification work in ETSI TC SES.

ETSI TC SES want to clarify that the requirements in the ETSI harmonised standards for earth stations are defined based on and referenced in CEPT regulatory framework or in accordance with ITU recommendations. Current TC-SES ETSI harmonised standards have not yet  considered 3GPP NTN access technology and updates may be needed.

Therefore, it is proposed to create a new work item (NWI) in ETSI in order  to accommodate NTN requirements in harmonised standards. There are already ongoing discussions between ETSI TC SES and ETSI TC MSG groups to find a way forward to capture the 3GPP NTN specifications in ETSI standards. 

ETSI TC SES would remain at the disposal of 3GPP RAN4 for any additional information.”

NOTE3:	ETSI requirements from EN 301 681 and ECC requirements from ECC Report 263 are captured for information, but have not presently been included in the specification. 
As per the agreements in RAN4#109, the introduction of additional blocking requirements from ETSI EN 301 681 and recommendations from ECC Report 263 has been put on hold.  Any ETSI- or ECC-specific requirements will be addressed after further feedback from ETSI is received.

#### 5.2.5.1 ITU-R

The ITU-R Radio Regulations [3] specify the following service allocation for the frequency ranges 1518 – 1525 MHz and 1668 – 1675 MHz, shown in Table 5.2.5.1-1:

Table 5.2.5.1-1: Allocation of 1518-1525 MHz and 1668-1675 MHz in the ITU-R Radio Regulations

| Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |
| 1 518-1 525 FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (space-to-Earth) 5.348 5.348A 5.348B 5.351A |  | 1 518-1 525 FIXED MOBILE 5.343 MOBILE-SATELLITE (space-to-Earth) 5.348 5.348A 5.348B 5.351A |  | 1 518-1 525 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.348 5.348A 5.348B 5.351A |
| 5.341 5.342 |  | 5.341 5.344 |  | 5.341 |
| 1 668-1 668.4 MOBILE-SATELLITE (Earth-to-space) 5.351A 5.379B 5.379C RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile 5.149 5.341 5.379 5.379A |  |  |  |  |
| 1 668.4-1 670 METEOROLOGICAL AIDS FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (Earth-to-space) 5.351A 5.379B 5.379C RADIO ASTRONOMY 5.149 5.341 5.379D 5.379E |  |  |  |  |
| 1 670-1 675 METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.379B 5.341 5.379D 5.379E 5.380A |  |  |  |  |
| 5.149 In making assignments to stations of other services to which the bands: 13 360-13 410 kHz, 25 550-25 670 kHz, 37.5-38.25 MHz, 73-74.6 MHz in Regions 1 and 3, 150.05-153 MHz in Region 1, 322-328.6 MHz, 406.1-410 MHz, 608-614 MHz in Regions 1 and 3, 1 330-1 400 MHz, 1 610.6-1 613.8 MHz, 1 660-1 670 MHz, 1 718.8-1 722.2 MHz, 2 655-2 690 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 4 825-4 835 MHz,; 4 950-4 990 MHz, 4 990-5 000 MHz, 6 650-6 675.2 MHz, 10.6-10.68 GHz, 14.47-14.5 GHz, 22.01-22.21 GHz, 22.21-22.5 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 31.5-31.8 GHz in Regions 1 and 3, 36.43-36.5 GHz, 42.5-43.5 GHz, 48.94-49.04 GHz, 76-86 GHz, 92-94 GHz, 94.1-100 GHz,; 102-109.5 GHz, 111.8-114.25 GHz, 128.33-128.59 GHz, 129.23-129.49 GHz, 130-134 GHz, 136-148.5 GHz, 151.5-158.5 GHz, 168.59-168.93 GHz, 171.11-171.45 GHz, 172.31-172.65 GHz, 173.52-173.85 GHz, 195.75-196.15 GHz, 209-226 GHz, 241-250 GHz, 252-275 GHz. are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from spaceborne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. 4.5 and 4.6 and Article 29). (WRC-07) 5.341 In the bands 1 400-1 727 MHz, 101-120 GHz and 197-220 GHz, passive research is being conducted by some countries in a programme for the search for intentional emissions of extraterrestrial origin. 5.342 Additional allocation: in Armenia, Azerbaijan, Belarus, the Russian Federation, Uzbekistan, Kyrgyzstan and Ukraine, the frequency band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis, exclusively for the purposes of aeronautical telemetry within the national territory. As of 1 April 2007, the use of the frequency band 1 452-1 492 MHz is subject to agreement between the administrations concerned. (WRC-15) 5.348 The use of the band 1 518-1 525 MHz by the mobile-satellite service is subject to coordination under No. 9.11A. In the band 1 518-1 525 MHz stations in the mobile-satellite service shall not claim protection from the stations in the fixed service. No. 5.43A does not apply. (WRC-03) 5.348A In the band 1 518-1 525 MHz, the coordination threshold in terms of the power flux-density levels at the surface of the Earth in application of No. 9.11A for space stations in the mobile-satellite (space-to-Earth) service, with respect to the land mobile service use for specialized mobile radios or used in conjunction with public switched telecommunication networks (PSTN) operating within the territory of Japan, shall be –150 dB(W/m2) in any 4 kHz band for all angles of arrival, instead of those given in Table 5-2 of Appendix 5. In the band 1 518-1 525 MHz stations in the mobile-satellite service shall not claim protection from stations in the mobile service in the territory of Japan. No. 5.43A does not apply. (WRC-03) 5.348B In the band 1 518-1 525 MHz, stations in the mobile-satellite service shall not claim protection from aeronautical mobile telemetry stations in the mobile service in the territory of the United States (see Nos. 5.343 and 5.344) and in the countries listed in No. 5.342. No. 5.43A does not apply. (WRC-03) 5.351A For the use of the frequency bands 1 518-1 544 MHz, 1 545-1 559 MHz, 1 610-1 645.5 MHz, 1 646.5-1 660.5 MHz, 1 668-1 675 MHz, 1 980-2 010 MHz, 2 170-2 200 MHz, 2 483.5-2 520 MHz and 2 670-2 690 MHz by the mobile-satellite service, see Resolutions 212 (Rev.WRC-23) and 225 (Rev.WRC-23). (WRC-23) 5.379 Additional allocation: in Bangladesh, India, Indonesia, Nigeria and Pakistan, the band 1 660.5-1 668.4 MHz is also allocated to the meteorological aids service on a secondary basis. 5.379A Administrations are urged to give all practicable protection in the band 1 660.5-1 668.4 MHz for future research in radio astronomy, particularly by eliminating air-to-ground transmissions in the meteorological aids service in the band 1 664.4-1 668.4 MHz as soon as practicable. 5.379B The use of the frequency band 1 668-1 675 MHz by the mobile-satellite service is subject to coordination under No. 9.11A. (WRC-23) 5.379C In order to protect the radio astronomy service in the band 1 668-1 670 MHz, the aggregate power flux-density values produced by mobile earth stations in a network of the mobile-satellite service operating in this band shall not exceed –181 dB(W/m2) in 10 MHz and 194 dB(W/m2) in any 20 kHz at any radio astronomy station recorded in the Master International Frequency Register, for more than 2% of integration periods of 2 000 s. (WRC-03) 5.379D For sharing of the frequency band 1 668.4-1 675 MHz between the mobile-satellite service and the fixed and mobile services, Resolution 744 (Rev.WRC-23) shall apply. (WRC-23) 5.379E In the band 1 668.4-1 675 MHz, stations in the mobile-satellite service shall not cause harmful interference to stations in the meteorological aids service in China, Iran (Islamic Republic of), Japan and Uzbekistan. In the band 1 668.4-1 675 MHz, administrations are urged not to implement new systems in the meteorological aids service and are encouraged to migrate existing meteorological aids service operations to other bands as soon as practicable. (WRC-03) 5.380A In the band 1 670-1 675 MHz, stations in the mobile-satellite service shall not cause harmful interference to, nor constrain the development of, existing earth stations in the meteorological-satellite service notified before 1 January 2004. Any new assignment to these earth stations in this band shall also be protected from harmful interference from stations in the mobile-satellite service. (WRC-07) |

#### 5.2.5.2 FCC

The FCC Rules [23] specify the following service allocations and technical rules for the frequency ranges 1518 – 1525 MHz and 1668 – 1675 MHz.

| Table of Frequency Allocations 1626.5-2110 MHz (UHF) |  |  |  |  |  | Page 35 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| International Table |  |  | United States Table |  |  | FCC Rule Part(s) |  |
| Region 1 Table | Region 2 Table | Region 3 Table | Federal Table | Non-Federal Table |  |  |  |
| 1668-1668.4 MOBILE-SATELLITE (Earth-to-space) 5.351A 5.379B 5.379C RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile 5.149 5.341 5.379 5.379A |  |  | 1660.5-1668.4 RADIO ASTRONOMY US74 SPACE RESEARCH (passive) |  |  |  |  |
| 1668.4-1670 METEOROLOGICAL AIDS FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (Earth-to-space) 5.351A 5.379B 5.379C RADIO ASTRONOMY 5.149 5.341 5.379D 5.379E |  |  | 5.341 US246 |  |  |  |  |
| 1670-1675 METEOROLOGICAL AIDS FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.379B 5.341 5.379D 5.379E 5.380A |  |  | 1668.4-1670 METEOROLOGICAL AIDS (radiosonde) RADIO ASTRONOMY US74 5.341 US99 US342 |  |  |  |  |

47 CFR 2.106(b)(341)

5.341 In the bands 1400-1727 MHz, 101-120 GHz and 197-220 GHz, passive research is being conducted by some countries in a programme for the search for intentional emissions of extraterrestrial origin.

47 CFR 2.106(b) (348)

5.348 The use of the band 1518-1525 MHz by the mobile-satellite service is subject to coordination under No. 9.11A. In the band 1518-1525 MHz stations in the mobile-satellite service shall not claim protection from the stations in the fixed service. No. 5.43A does not apply.

(i) 5.348A In the band 1518-1525 MHz, the coordination threshold in terms of the power flux-density levels at the surface of the Earth in application of No. 9.11A for space stations in the mobile-satellite (space-to-Earth) service, with respect to the land mobile service use for specialized mobile radios or used in conjunction with public switched telecommunication networks (PSTN) operating within the territory of Japan, shall be −150 dB(W/m2) in any 4 kHz band for all angles of arrival, instead of those given in Table 5-2 of Appendix 5. In the band 1518-1525 MHz stations in the mobile-satellite service shall not claim protection from stations in the mobile service in the territory of Japan. No. 5.43A does not apply.

(ii) 5.348B In the band 1518-1525 MHz, stations in the mobile-satellite service shall not claim protection from aeronautical mobile telemetry stations in the mobile service in the territory of the United States (see paragraphs (b)(343) and (344) of this section) and in the countries listed in paragraph (b)(342) of this section. No. 5.43A does not apply.

47 CFR 2.106(b)(351)

(i) 5.351A For the use of the bands 1518-1544 MHz, 1545-1559 MHz, 1610-1645.5 MHz, 1646.5-1660.5 MHz, 1668-1675 MHz, 1980-2010 MHz, 2170-2200 MHz, 2483.5-2520 MHz and 2670-2690 MHz by the mobile-satellite service, see Resolutions 212 (Rev.WRC-19) and 225 (Rev.WRC-12).

47 CFR 2.106(b)(379)

5.379 Additional allocation: in Bangladesh, India, Indonesia, Nigeria and Pakistan, the band 1660.5-1668.4 MHz is also allocated to the meteorological aids service on a secondary basis.

(i) 5.379A Administrations are urged to give all practicable protection in the band 1660.5-1668.4 MHz for future research in radio astronomy, particularly by eliminating air-to-ground transmissions in the meteorological aids service in the band 1664.4-1668.4 MHz as soon as practicable.

(ii) 5.379B The use of the band 1668-1675 MHz by the mobile-satellite service is subject to coordination under No. 9.11A. In the band 1668-1668.4 MHz, Resolution 904 (WRC-07) shall apply.

(iii) 5.379C In order to protect the radio astronomy service in the band 1668-1670 MHz, the aggregate power flux-density values produced by mobile earth stations in a network of the mobile-satellite service operating in this band shall not exceed −181 dB(W/m2) in 10 MHz and −194 dB(W/m2) in any 20 kHz at any radio astronomy station recorded in the Master International Frequency Register, for more than 2% of integration periods of 2000 s.

(iv) 5.379D For sharing of the band 1668.4-1675 MHz between the mobile-satellite service and the fixed and mobile services, Resolution 744 (Rev.WRC-07) shall apply.

(v) 5.379E In the band 1668.4-1675 MHz, stations in the mobile-satellite service shall not cause harmful interference to stations in the meteorological aids service in China, Iran (Islamic Republic of), Japan and Uzbekistan. In the band 1668.4-1675 MHz, administrations are urged not to implement new systems in the meteorological aids service and are encouraged to migrate existing meteorological aids service operations to other bands as soon as practicable.

47 CFR 2.106(b)(380)

5.380A In the band 1670-1675 MHz, stations in the mobile-satellite service shall not cause harmful interference to, nor constrain the development of, existing earth stations in the meteorological-satellite service notified before 1 January 2004. Any new assignment to these earth stations in this band shall also be protected from harmful interference from stations in the mobile-satellite service.

US211:   In the bands 1670-1690, 5000-5250 MHz and 10.7-11.7, 15.1365-15.35, 15.4-15.7, 22.5-22.55, 24-24.05, 31.0-31.3, 31.8-32.0, 40.5-42.5, 116-122.25, 123-130, 158.5-164, 167-168, 191.8-200, and 252-265 GHz, applicants for airborne or space station assignments are urged to take all practicable steps to protect radio astronomy observations in the adjacent bands from harmful interference; however, US74 applies.

US362:   The band 1670-1675 MHz is allocated to the meteorological-satellite service (space-to-Earth) on a primary basis for Federal use. Earth station use of this allocation is limited to Wallops Island, VA (37° 56' 44'' N, 75° 27' 37'' W), Fairbanks, AK (64° 58' 22'' N, 147° 30' 04'' W), and Greenbelt, MD (39° 00' 02'' N, 76° 50' 29'' W). Applicants for non-Federal stations within 100 kilometers of the Wallops Island or Fairbanks coordinates and within 65 kilometers of the Greenbelt coordinates shall notify NOAA in accordance with the procedures specified in 47 CFR 1.924.

#### 5.2.5.3 CEPT/ECC

##### 5.2.5.3.1 CEPT

This section is reserved.

##### 5.2.5.3.2 ECC Report 263

NOTE:	The coexistence assumptions used in ECC Report 263 are different compared to the assumptions used by 3GPP in the TN-NTN coexistence analysis captured in TR 38.863.

The following section contains some excerpts from ECC Report 263.

The report has established the technical characteristics of the IMT and the MSS system and determined the relevant scenarios. It has also determined the appropriate propagation models for these scenarios according to the environment where the equipment is used and further the protection criteria has been established.

The report has, from these characteristics and parameters, developed an MCL analysis with the resulting required separation distances for the 3 different frequency separations (1 MHz, 3 MHz and 6 MHz frequency separations).

The frequency separation allocation is not addressed by this report.

Further the report makes use of the MCL to establish interference arising in the first MES channel above the 3 different frequency separations investigated for an area with IMT coverage.

The results of the simulations show that there will be some interference irrespective of the selected frequency separation.

MES terminals currently on the market which have characteristics similar to those selected for this study, may experience interference problems because of the susceptibility of the MES receiver to the wanted signals from the IMT systems. As there are currently no available technical characteristics which outline at what frequency this effect starts to occur, blocking may also be experienced from IMT transmitters more than 6 MHz away into the IMT band below 1518 MHz.

Based on the findings of this report, the following mitigation techniques could further improve the compatibility between IMT and MSS around 1518 MHz:

-	The interference due to IMT OOB emissions can be reduced by improved filtering on the IMT base station.

-	The interference due to blocking can be reduced by improving the MES resilience to LTE blocking signals in the adjacent band.

-	Either adding location based frequency allocation to MSS to avoid the use the lower couple of MHz and/or, implementing interference avoidance which would in addition allow for a better frequency utilisation of the lower part of the 1518-1559 MHz frequency band for MSS. The feasibility and impact of these techniques have not been assessed.

The following values have been used to examine the impact of enhanced MES receiver performance.

Table 7.1.2.2-1: Assumed blocking level for enhanced MES receivers

| Frequency separation between channel edges | Interference level (at output of receiving antenna) |
| --- | --- |
| 1 MHz | –55 to –45 dBm |
| 3 MHz | –35 to –30 dBm |
| 6 MHz | –30 to –25 dBm |

Based on the final results of its compatibility studies, it is concluded that:

-	the minimum in-band blocking characteristic for land mobile earth stations receivers from a 5 MHz broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm above 1520 MHz (Note: when the MES operates above 1520 MHz),

-	the base station unwanted emission limits EIRP for a broadband signal interferer (LTE) operating below 1518 MHz shall be −30dBm/MHz above 1520 MHz. This figure is 10 dB more stringent than ECC Decision (13)03 due to a different service in the adjacent band.

It is noted that the IMT block ends at 1517 MHz.

#### 5.2.5.4 ETSI

##### 5.2.5.4.1 ETSI EN 301 681

##### 5.2.5.4.1.1 In-band and Out-of-band Emission Requirements

The following section includes excerpts from ETSI EN 301 681.

ETSI EN 301 681 is applicable to Mobile Earth Stations (SES) with both transmit and receive capabilities for operation in  Satellite Personal Communications Networks (S-PCN) in any combination of all or any part of the Mobile Satellite Service (MSS) frequency bands sub-band 1 and sub-band 2 defined in table 5.2.5.4.1.1-1.

Table 5.2.5.4.1.1-1: Mobile Satellite Service (MSS) frequency band

| Sub-band | Transmission path | MSS frequency band |
| --- | --- | --- |
| 1 | MESs transmit 1 | 1 626,5 MHz to 1 660,5 MHz |
|  | MESs receive 1 | 1 525 MHz to 1 559 MHz |
| 2 | MESs transmit 2 | 1 668,0 MHz to 1 675,0 MHz |
|  | MESs receive 2 | 1 518,0 MHz to 1 525,0 MHz |

Unwanted emissions from Mobile Earth Stations (LMESs) outside the band 1 626,5 MHz to 1 660,5 MHz and outside the band 1 668 MHz to 1 675 MHz shall be below the following limits.

Unless otherwise stated the specification in this clause shall apply to all types of MESs notwithstanding their transmitting capabilities within the frequency bands as defined in table 5.2.5.4.1.1-1:

-	For MES that are capable of transmitting within only the sub-band 1 frequency band as defined in table 5.2.5.4.1.1-1, the maximum EIRP spectral density of the unwanted emissions from the MES outside the band 1 626,5 MHz to 1 660,5 MHz shall not exceed the limits in either table 5.2.5.4.1.1-2 or table 5.2.5.4.1.1-3. The applicant shall declare which alternative shall be used.

-	For MES that are capable of transmitting within only sub-band 2 frequency band or within both the sub-band 1 and sub-band 2 frequency bands, the maximum EIRP spectral density of the unwanted emissions from the MES outside the bands 1 626,5 MHz to 1 660,5 MHz and 1 668,0 MHz to 1 675,0 MHz shall not exceed the limits in table 5.2.5.4.1.1-3.

Table 5.2.5.4.1.1-2: Unwanted emissions outside the band 1 626,5 MHz to 1 660,5 MHz for MES only capable of transmitting within sub-band 1 frequency band as defined in table 5.2.5.4.1.1-1

| Frequency (MHz) | Carrier - on state |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method (see note 6) |
| 30 to 1 000 | -66 | 100 kHz | Peak Hold |
| 1 000 to 1 559 | -61 | 1 MHz | Average |
| 1 559 to 1 605,0 | -70 | 1 MHz (see note 3) | Average (see note 2) |
| 1 605,0 to 1 612,5 | -70 to -58,5 (see note 4) | 1 MHz (see note 3) | Average |
| 1 612,5 to 1 616,5 | -55 to -50 (see note 4) | 1 MHz | Average |
| 1 616,5 to 1 621,5 | -50 to -46 (see note 4) | 1 MHz | Average |
| 1 621,5 to 1 624,5 | -60 | 30 kHz | Average |
| 1 624,5 to 1 625,0 | -60 to -57,5 (see notes 4, 5, 7) | 30 kHz | Average |
| 1 625,0 to 1 625,125 | -57,5 to -57,2 (see notes 4, 5, 7) | 30 kHz | Average |
| 1 625,125 to 1 625,8 | -57,2 to -50 (see notes 4, 5, 7) | 30 kHz | Average |
| 1 625,8 to 1 626,0 | -50 to -47 (see notes 4, 5, 7) | 30 kHz | Average |
| 1 626,0 to 1 626,2 | -47 to -40 (see notes 4, 5, 7) | 30 kHz | Average |
| 1 626,2 to 1 626,5 | -40 (see notes 5, 7) | 30 kHz | Average |
| 1 626,5 to 1 660,5 | NOT APPLICABLE | NOT APPLICABLE | NOT APPLICABLE |
| 1 660,5 to 1 662,5 | The levels in table 4a shall apply from 1 660,5 MHz to 1 662,5 MHz |  |  |
| 1 662,5 to 1 665,5 | -60 | 30 kHz | Average |
| 1 665,5 to 1 670,5 | -60 | 100 kHz | Average |
| 1 670,5 to 1 680,5 | -60 | 300 kHz | Average |
| 1 680,5 to 1 690,5 | -60 | 1 MHz | Average |
| 1 690,5 to 2 250 | -60 | 3 MHz | Average |
| 2 250 to 12 750 (see note 1) | -60 | 3 MHz | Peak Hold |
| NOTE 1: In the band 3 253,0 MHz to 3 321,0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -38 dBW. Elsewhere in this band the power limit in table 3 shall be applied. In each of the bands 4 879,5 MHz to 4 981,5 MHz, 6 506,0 MHz to 6 642,0 MHz and 8 132,5 MHz to 8 302,5 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -48 dBW. Elsewhere in this band the power limit in table 3 shall be applied. NOTE 2: The average measurement method defined in clause 5.2.2.3 shall apply except that an averaging period of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 3: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 4: Linearly interpolated in dBW vs. Frequency. NOTE 5: The power limits specified in the band 1 624,5 MHz to 1 626,5 MHz require further study. This study is important to determine whether less stringent limits may enhance spectrum efficiency and utilization immediately above 1 626,5 MHz. NOTE 6: Peak Hold and Average measurements shall be performed as specified in clauses 5.2.2.2 and 5.2.2.3. NOTE 7: For systems employing CDMA, the EIRP limits shall be decreased by 10 log (N) dB, where N is the maximum number of MESs in the receive beam of the satellite to which these MESs are communicating and which are expected to transmit simultaneously in the same frequency band within that same beam. This number shall be declared by the manufacturer (N = 1 in a TDMA system). |  |  |  |

Table 5.2.5.4.1.1-3: Unwanted emissions outside the bands 1 626,5 MHz to 1 660,5 MHz and 1 668,0 MHz to 1 675,0 MHz for MES capable of transmitting within all or any part of sub-band 1 and sub-band 2 frequency bands as defined in table 5.2.5.4.1.1-1

| Frequency (MHz) | Carrier - on state |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method (see note 8) |
| 30 to 1 000 | -66 | 100 kHz | Peak Hold |
| 1 000 to 1 559 | -61 | 1 MHz | Average |
| 1 559 to 1 605,0 | -70 | 1 MHz (see note 5) | Average (see note 2) |
| 1 605,0 to 1 612,5 | -70 to -58,5 (see note 6) | 1 MHz (see note 5) | Average |
| 1 612,5 to 1 616,5 | -55 to -50 (see note 6) | 1 MHz | Average |
| 1 616,5 to 1 621,5 | -50 to -46 (see note 6) | 1 MHz | Average |
| 1 621,5 to 1 624,5 | -60 | 30 kHz | Average |
| 1 624,5 to 1 625,0 | -60 to -57,5 (see notes 6, 7, 9) | 30 kHz | Average |
| 1 625,0 to 1 625,125 | -57,5 to -57,2 (see notes 6, 7, 9) | 30 kHz | Average |
| 1 625,125 to 1 625,8 | -57,2 to -50 (see notes 6, 7, 9) | 30 kHz | Average |
| 1 625,8 to 1 626,0 | -50 to -47 (see notes 6, 7, 9) | 30 kHz | Average |
| 1 626,0 to 1 626,2 | -47 to -40 (see notes 6, 7, 9) | 30 kHz | Average |
| 1 626,2 to 1 626,5 | -40 (see notes 7, 9) | 30 kHz | Average |
| 1 626,5 to 1 660,5 | NOT APPLICABLE | NOT APPLICABLE | NOT APPLICABLE |
| 1 660,5 to 1 662,5 | note 1 | note 1 | note 1 |
| 1 662,5 to 1 666,0 | -55 | 30 kHz | Average |
| 1 666,0 to 1 668,0 | note 2 | note 2 | note 2 |
| 1 668,0 to 1 675,0 | NOT APPLICABLE | NOT APPLICABLE | NOT APPLICABLE |
| 1 675,0 to 1 677,0 | note 2 | note 2 | note 2 |
| 1 677,0 to 1 680,0 | -60 | 30 kHz | Average |
| 1 680,0 to 1 685,0 | -60 | 100 kHz | Average |
| 1 685,0 to 1 695,0 | -60 | 300 kHz | Average |
| 1 695,0 to 1 705,0 | -60 | 1 MHz | Average |
| 1 705,0 to 2 250 | -60 | 3 MHz | Average |
| 2 250 to 12 750 (see note 3) | -60 | 3 MHz | Peak Hold |
| NOTE 1: For an MES transmitting in sub-band 1 (as defined in table 1) the limits defined in table 4a shall apply for the band 1 660,5 MHz to 1 662,5 MHz. For an MES transmitting in sub-band 2 (as defined in table 1) a limit of -55 dBW in 30 kHz shall apply for the band 1 660,5 MHz to 1 662,5 MHz. NOTE 2: For an MES transmitting in sub-band 2 (as defined in table 1) the limits defined in table 4a shall apply for the bands 1 666,0 MHz to 1 668,0 MHz and 1 675,0 MHz to 1 677,0 MHz. For an MES transmitting in sub-band 1 (as defined in table 1) a limit of -55 dBW in 30 kHz shall apply for the bands 1 666,0 MHz to 1 668,0 MHz and 1 675,0 MHz to 1 677,0 MHz. NOTE 3: In the bands 3 253,0 MHz to 3 321,0 MHz and 3 336,0 MHz to 3 350,0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -38 dBW. Elsewhere in this band the power limit in table 3a shall be applied. In the bands 4 879,5 MHz to 4 981,5 MHz and 5 004,0 MHz to 5 025,0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -48 dBW. Elsewhere in this band the power limit in table 3a shall be applied. In the bands 6 506,0 MHz to 6 642,0 MHz and 6 672,0 MHz to 6 700,0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -48 dBW. Elsewhere in this band the power limit in table 3a shall be applied. In the bands 8 132,5 MHz to 8 302,5 MHz and 8 340,0 MHz to 8 375,0 MHz the maximum EIRP in one, and only one, 3 MHz measurement bandwidth shall not exceed -48 dBW. Elsewhere in this band the power limit in table 3a shall be applied. NOTE 4: The average measurement method defined in clause 5.2.2.3 shall apply except that an averaging period of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 5: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 6: Linearly interpolated in dBW vs. Frequency. NOTE 7: The power limits specified in the band 1 624,5 MHz to 1 626,5 MHz require further study. This study is important to determine whether less stringent limits may enhance spectrum efficiency and utilization immediately above 1 626,5 MHz. NOTE 8: Peak Hold and Average measurements shall be performed as specified in clauses 5.2.2.2 and 5.2.2.3. NOTE 9: For systems employing CDMA, the EIRP limits shall be decreased by 10 log (N) dB, where N is the maximum number of MESs in the receive beam of the satellite to which these MESs are communicating and which are expected to transmit simultaneously in the same frequency band within that same beam. This number shall be declared by the manufacturer (N = 1 in a TDMA system). |  |  |  |

Table 5.2.5.4.1.1-4: Maximum unwanted emissions in the bands 1 626,5 MHz to 1 660,5 MHz and 1 660,5 MHz to 1 662,5 MHz caused by S-PCN MESs transmitting in the band 1 626,5 MHz to 1 660,5 MHz; and maximum unwanted emissions in the bands 1 666,0 MHz to 1 668,0 MHz, 1 668,0 MHz to 1 675,0 MHz and 1 675,0 MHz to 1 677,0 MHz caused by S-PCN MESs transmitting in the band 1 668,0 MHz to 1 675,0 MHz

| Frequency | Carrier-on state |  |  |
| --- | --- | --- | --- |
| offset (kHz) (see note 1) | EIRP (dBW) (see note 2) | Measurement bandwidth (kHz) | Measurement method |
| 0 to 25 | 0 to -15 | 3 | Average |
| 25 to 125 | -15 to -50 | 3 | Average |
| 125 to 425 | -50 | 3 | Average |
| 425 to 1 500 | -50 to -65 | 3 | Average |
| 1 500 to 36 000 | -55 | 30 | Average |
| NOTE 1: Frequency offset is determined from the edge of the nominated bandwidth. NOTE 2: Linearly interpolated in dBW vs. Frequency offset. |  |  |  |

Table 5.2.5.4.1.1-5: Maximum unwanted emissions in the bands 1 626,5 MHz to 1 660,5 MHz caused by S-PCN MESs transmitting in the band 1 626,5 MHz to 1 660,5 MHz; and maximum unwanted emissions in the bands 1 668,0 MHz to 1 675,0 MHz caused by S-PCN MESs transmitting in the band 1 668,0 MHz to 1 675,0 MHz

| Frequency offset (kHz) | Carrier-on state |  |  |
| --- | --- | --- | --- |
| (see notes 1 and 3) | EIRP (dBW) (see note 2) | Measurement bandwidth (kHz) | Measurement method |
| 0 to 25 | 0 to -15 | 3 | Average |
| 25 to 55 | -15 to -25 (see note 4) | 3 | Average |
| 55 to AB | -25 (see note 4) | 3 | Average |
| AB to (AB + 0,35 x B3dB) | -25 to -40 (see note 4) | 3 | Average |
| (AB + 0,35 x B3dB) to CD | -40 | 3 | Average |
| CD to (CD + 0,25 x B3dB) | -40 to -50 | 3 | Average |
| (CD + 0,25 x B3dB) to EF | -50 | 3 | Average |
| EF to 1 500 | -50 to -65 | 3 | Average |
| 1 500 to 36 000 | -55 | 30 | Average |
| NOTE 1: Frequency offset is determined from the edge of the nominated bandwidth. NOTE 2: Linearly interpolated in dBW vs. Frequency offset. NOTE 3: The parameters AB, CD, EF are defined below. NOTE 4: The limit of -25 dBW in this table is determined on the assumption that the adjacent channel interference results from a single interferer. This limit shall apply to MESs that are designed for operation in a network where the occurrence of two (or more) interferers, all transmitting with the maximum permitted level of unwanted emissions, does not exceed 0,1 % of the time; otherwise a limit of -30 dBW shall apply. |  |  |  |

The parameters AB, CD and EF are defined as a proportion of the 3 dB Bandwidth as follows:

AB = (55) or (100 % of the B3dB), whichever is the greater; • 	CD = (95) or (200 % of the B3dB), whichever is the greater;

EF = (125) or (300 % of the B3dB), whichever is the greater.

##### 5.2.5.4.1.2 Additional measures to protect Radioastronomy (RA) stations

ETSI EN 301 681 harmonized standard specifies requirements for the protection of radioastronomy bands. In section 4.2.6 of the document, the protection of radioastronomy services is mandated by disabling devices in the vicinity of radioastronomy stations recorded in the ITU master international frequency register.

To protect the radio astronomy service in the 1 660 MHz to 1 660,5 MHz band and the 1 668,0 MHz to 1 670,0 MHz band from emissions produced by MESs the transmissions in these frequency bands shall be capable of being disabled in the vicinity of RA stations recorded in the ITU Master International Frequency Register.

Requirements are also set out for the activation of this disable function:

“An MES which is in the carrier-on state shall not continue transmissions for a period of time longer than 1s after receipt of a transmission disable command.”

Spectrum emission requirements are further specified when a transmission disable command is issued, in the form of carrier-off state emissions:

Table 5.2.5.4.1.2-1 EN 301 681 Carrier-off state requirements

| Frequency (MHz) | EIRP (dBW) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 30 to 1 000 | -87 | 100 kHz | peak hold |
| 1 000 to 1 525 | -87 | 100 kHz | peak hold |
| 1 525 to 1 559 | -97 (see note 3) | 100 kHz | average |
| 1 559 to 1 610 | -70 | 1 MHz (see note 1) | average (see note 2) |
| 1 610 to 12 750 | -87 | 100 kHz | peak hold |
| NOTE 1: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 2: The average measurement method defined in clause 5.2.2.3 shall apply except that a measurement time of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 3: If MES antenna gain is higher than 8 dBi, the unwanted emission limit in the band 1 525 MHz to 1 559 MHz shall be considered to be -90 dBW in 100 kHz. |  |  |  |

The emissions limit for 1610-12750MHz captures the carrier-off requirements for protection of radioastronomy bands within 1660-1660.5MHz and 1668-1670MHz, mandating a limit of -87dBW/100kHz, measured in any time period as a peak hold measurement.

********** NEXT CHANGED SECTION **********

## 7.2	Common issues for satellite access node and NTN UE

### 7.2.1	General

This section aims to present the agreed definitions for the newly introduced NTN operating bands. The possible configurations in terms of channel bandwidth, SCS, channel and synchronization raster for NTN operations are introduced as well.

### 7.2.2	Operating bands

The following bands in Table 7.2.2-1 are agreed as NTN FR1 satellite bands starting with Release 17. Regarding the band numbering for NTN satellite bands, it is agreed to start from the largest band number in FR1 range for NTN satellite bands which fully within FR1 frequency ranges to better differentiate NTN satellite band and TN bands and ensure contiguous band number allocation for NTN satellite bands, the number can be taken in a decreased order with first come, first service.

Table 7.2.2-1: NTN satellite bands in FR1

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n253 | 1668 MHz – 1675 MHz | 1518 MHz – 1525 MHz | FDD |
| n251 | 1626.5 MHz – 1660.5 MHz | 1518 MHz – 1559 MHz | FDD |
| n250 | 1668 MHz – 1675 MHz | 1518 MHz – 1559 MHz | FDD |
| NOTE: NTN bands are numbered in descending order from n256. |  |  |  |

********** NEXT CHANGED SECTION **********

### 7.2.3	Channel bandwidth, SCS and spectral utilization

The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in TS 38.104[16] and TS 38.101-1[17] can be reused for NTN satellite system.

The supported channel bandwidths per operating band should be defined based on NTN operator input. The details are given in Table 7.2.3-1.

Table 7.2.3-1: Channel bandwidths and SCS per NTN satellite operating band in FR1

| NTN satellite band # | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz1 | 30 MHz1 | 35 MHz1 | 40 MHz1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
| n255 |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
| n253 |  | 15 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n251 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  | [Yes] |  | [Yes] |  | [Yes2] |  | [Yes2] |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  | [Yes] |  | [Yes] |  | [Yes2] |  | [Yes2] |
|  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  | [Yes2] |  | [Yes] |  | [Yes2] |  | [Yes2] |
| n250 |  | 15 |  | Yes |  | Yes2 |  | Yes2 |  |  | Yes2 |  | [Yes2] |  | [Yes2] |  | [Yes2] |  | [Yes2] |
|  |  | 30 |  |  |  | Yes2 |  | Yes2 |  |  | Yes2 |  | [Yes2] |  | [Yes2] |  | [Yes2] |  | [Yes2] |
|  |  | 60 |  |  |  | Yes2 |  | Yes2 |  |  | Yes2 |  | [Yes2] |  | [Yes2] |  | [Yes2] |  | [Yes2] |
| NOTE 1: Deployment of 30, 35 and 40 MHz channel bandwidths for NTN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. NOTE 2: This channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.3.1		Asymmetric channel bandwidth

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 7.4.1.1-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 7.2.3-2.

Table 7.2.3-2: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n251 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 | 0 |
|  | 15 | 20 | 0 |
| n250 | 5 | 10, 15, 20 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

********** NEXT CHANGED SECTION **********

### 7.2.4	Channel raster and sync raster

#### 7.2.4.1	Channel raster

In order to be compatible with current design of sync raster below 3 GHz, the ARFCN parameters below 3000 MHz will be reused and 100 kHz was chosen as the channel raster for NR NTN satellite bands below 3 GHz.

The applicable ARFCNs per operating and for satellite access node are defined in table 7.2.4.1-1.

Table 7.2.4.1-1: ARFCNs per operating band satellite access node

| NTN satellite band # | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n253 | 100 | 333600 – <20> – 335000 | 303600 – <20> – 305000 |
| n251 | 100 | 325300 – <20> – 332100 | 303600 – <20> – 311800 |
| n250 | 100 | 333600 – <20> – 335000 | 303600 – <20> – 311800 |

In addition to the 100 kHz channel raster, Enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 7.2.4.1-2 is given as <2>.

Table 7.2.4.1-2: ARFCNs per operating band satellite access node

| NTN satellite band # | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n253 | 10 | 333600 – <2> – 335000 | 303600 – <2> – 305000 |
| n251 | 10 | 325300 – <2> – 332100 | 303600 – <2> – 311800 |
| n250 | 10 | 333600 – <2> – 335000 | 303600 – <2> – 311800 |

#### 7.2.4.2	Sync raster

The applicable SS raster entries per operating band are defined in Table 7.2.4.2-1

Table 7.2.4.2-1: Applicable SS raster entries per operating band

| NTN satellite band # | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n253 | 15 kHz | Case A | 3800 – <1> – 3807 |
| n251 | 15 kHz | Case A | 3800 – <1> – 3892 |
|  | 30 kHz | Case B | 3806 – <1> – 3886 |
| n250 | 15 kHz | Case A | 3800 – <1> – 3892 |
|  | 30 kHz | Case B | 3806 – <1> – 3886 |

********** NEXT CHANGED SECTION **********

## 7.4	NTN UE requirements

### 7.4.1	General

In general, NTN UE products are assumed to meet TN UE requirements as specified in TS 38.101-1[17]. The structure and content of the NTN UE specification TS 38.101-5 should be based on TS 38.101-1[17]. The development of TS 38.101-5 should follow the set of conditions that were agreed at the RAN#94e meeting in RP-213618[27].

#### 7.4.1.1	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 7.4.1.1-1 for FR1-NTN.

Table 7.4.1.1-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n253 | -150 MHz1 -148 to -152 MHz2 |
| n251 | -108.5 MHz1 -72.5 to -137.5 MHz2 |
| n250 | -150 MHz1 -114 to -152 MHz2 |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

********** NEXT CHANGED SECTION **********

### 7.4.2	UE Transmission characteristics for satellite access

#### 7.4.2.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0dBi is assumed. Handheld PC3 UE is assumed in Release 17 for satellite access.

#### 7.4.2.2	Conducted transmitter characteristics

##### 7.4.2.2.1	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 7.4.2.2.1-1: UE Power Class

| NTN satellite band # | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | +2 |
| n253 | 23 | ±2 |
| n251 | 23 | ±2 |
| n250 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

##### 7.4.2.2.2	MPR/AMPR

7.4.2.2.2.1	MPR

UE is allowed to reduce the maximum output power (MPR) due to higher order modulations and transmit bandwidth configurations to meet the general transmission requirements which includes in-band emission, ACLR, out-of-band emission and general spurious emission. As it was agreed those general transmission requirements can be reused from TN, the same requirements specified in Table 6.2.2-1 in TS 38.101-1 [17] can be reused for NTN satellite access UEs.

7.4.2.2.2.2	A-MPR

A-MPR is band specific requirement which is used to meet the additional requirements indicated by network signalling (NS) value.

For n256, based on the regulation conclusion in section 5.2, the A-MPR requirements for n65 could be as starting point. From TS 38.101-1, since the maximum carrier centre frequency of NS_05 and NS_05U is lower than 1960 MHz and the maximum carrier centre frequency of NS_51 is lower than 1980 MHz, it is proposed NS_05, NS_05U and NS_51 don’t need to be considered for n256. The requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 [17] for band n65 can be reused for n256

For n255, based on regulatory requirements, a new NS is required. The following observations were made from the measurement results using an LTE modem and transceiver supporting 1626.5 – 1656.5 MHz:

-	For 5 MHz channel, PA emissions for DFT-s-OFDM are below the spurious emission limits between 1559 and 1606 MHz.

-	For 10 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 5 dB between 1604 and 1606 MHz.

-	For 15 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 9 dB between 1599 and 1606 MHz.

-	For 20 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits by 9 dB between 1584 and 1606 MHz.

-	For 700 Hz MBW, PA emissions for all channel bandwidths are below the spurious emission limits between 1559 and 1606 MHz.

A n255 duplexer with rejection of more than 10 dB will be a sufficient to satisfy the spurious emission limits specified in Table 7.4.2.2.8-1.

Based on this measurement observations, the expected filter attenuation of more than 10 dB below 1610 MHz is going to be adequate to meet the additional emissions requirements between 1559 – 1610 MHz, and therefore, no A-MPR needs to be specified for n255.

In view of the above, the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value shall be specified in Table 7.4.2.2.2.2-1 and the mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 7.4.2.2.2.2-1A.

NS_09N and NS_10N are defined to capture additional spectrum emission requirements from ETSI 301 681.

Table 7.4.2.2.2.2-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.1-1 | 5, 10, 15, 20 | Table 5.3.2-1 in TS 38.101-1 | N/A |
| NS_02N | 6.5.3.3.1 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_24 | 6.5.3.3.13 in TS 38.101-1 | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in TS 38.101-1 | Clause 6.2.3.7 in TS 38.101-12 |
| NS_100 | 6.5.2.4.2 in TS 38.101-1 | n2561 |  |  | Table 6.2.3.1-2 in TS 38.101-1 |
| NS_09N | 6.5.3.3.8 | n250, n253 | 5 |  |  |
| NS_10N | 6.5.3.3.9 | n251 | 5, 10, 15, 20 |  |  |
| NOTE 1: This NS can be signalled for NTN satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz is not specified, and therefore shall be used as a guard band. |  |  |  |  |  |

Table 7.4.2.2.2.2-1A: Mapping of network signalling label

| NTN satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n253 | NS_01 | NS_02N | NS_09N |  |  |  |  |  |
| n251 | NS_01 | NS_02N | NS_10N |  |  |  |  |  |
| n250 | NS_01 | NS_02N | NS_09N |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331. |  |  |  |  |  |  |  |  |

For n256, FFS for how to protect n2, n25, n70 and whether A-MPR required.

When "NS_09N" or "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 7.4.2.2.2.2-2 for any channel bandwidth configured within the corresponding band.

Table 7.4.2.2.2.2-2: Additional requirements for "NS_09N" and "NS_10N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

When "NS_09N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 7.4.2.2.2.2-3. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 7.4.2.2.2.2-1 from the edge of the channel bandwidth.

Table 7.4.2.2.2.2-3: Additional out-of-band requirements for "NS_09N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1666.5 | -25 | 30 kHz | Average |
| 1666.5 to 1667.575 | -20 to -35 | 3 kHz | Average |
| 1667.575 to 1667.875 | -20 | 3 kHz | Average |
| 1667.875 to 1667.975 | 15 to - 20 | 3 kHz | Average |
| 1667.975 to 1668 | 30 to 15 | 3 kHz | Average |
| 1675 to 1675.025 | 30 to 15 | 3 kHz | Average |
| 1675.025 to 1675.125 | 15 to - 20 | 3 kHz | Average |
| 1675.125 to 1675.425 | -20 | 3 kHz | Average |
| 1675.425 to 1676.5 | -20 to -35 | 3 kHz | Average |
| 1676.5 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

When "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 7.4.2.2.2.2-4. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 7.4.2.2.2.2-1 from the edge of the channel bandwidth.

Table 7.4.2.2.2.2-4: Additional out-of-band requirements for "NS_10N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1662.5 | -25 | 30 kHz | Average |
| 1662.5 to 1665.5 | -30 | 30 kHz | Average |
| 1665.5 to 1670.5 | -30 | 100 kHz | Average |
| 1670.5 to 1680.5 | -30 | 300 kHz | Average |
| 1680.5 to 1690.5 | -30 | 1 MHz | Average |
| 1690.5 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

********** NEXT CHANGED SECTION **********

##### 7.4.2.2.10	UE to UE coexistence issues between NTN and TN

In recent RAN4#102 and RAN4#103 meetings, the spurious emissions requirements (-50dBm/MHz) for protected bands UE co-existence (especially for band n256 UL: 1980~2010MHz DL: 2170~2200MHz) were widely discussed. The adjacent/overlapping TN bands to NTN band n256 are shown in Figure 5.2.1-1.

1)	Coexistence protection for band n34 DL

The NS_24 including associated A-MPR value as specified in TS 38.101-1[17] is reused to solve this issue in Release 17. However, other companies pointed out the n256 UL frequency range can’t be leveraged due to 5MHz guard band at the upper of n256 UL frequency range to protect band n34.If NS_24 is used, n256 will be reduced due to the 5 MHz guard band that has been considered with NS_24, frequency range 2005-2010 MHz can not be used anymore. Besides, when NS_24 is indicated to protect the DL frequency range of band n34, satellite UE has to reduce the out power in band n256. It will lead to UL throughput loss and even UL radio link failure for some UL RB configurations.

2)	Coexistence protection for band n39 DL

-50dBm/MHz co-existence requirement can be met considering 20MHz channel bandwidth for band n256 without A-MPR requirement, even if a band n65 duplexer is reused for band n256.

Based on extrapolating the SEM mask to the ACLR4 region which overlaps n39 for the lowest n256 20MHz channel it is feasible to specify -50dBm/MHz protection level for n39 by n256 even when using n65 duplexer (no filter help assumed). This assumes the maximum n256 bandwidth is 20MHz.

Figure 7.4.2.2.10-1: Coexistence analysis based on SEM mask extension for band n256

3)	Coexistence protection for band n2, n25, n70 DL

From RAN4 perspective, UE co-existence requirements can’t be fulfilled (-50dBm/MHz) for the TN bands (i.e., n2, n25 and n70) which are overlapping n256. In Release 17, it is RAN4 understanding that the co-existence issue between n256 and those TN bands shall rely on regional or national regulation.

Other proposals:

Other proposals were submitted to solve the UE to UE coexistence issues between NTN and TN especially for the protected DL frequency range which is overlapping with UL frequency range of band n256 or next to the edge of band n256 UL filter.

For example:

-	Isolation regions could be defined between NTN and TN coverage. And a corresponding signaling could be designed by RAN2.

-	Before transmitting UL band n256 signals, satellite UE should determine (or be informed) whether the IMT services exist for the protected DL frequency range in the vicinity.

-	Other potential mechanisms/solutions are not precluded.

Considering these proposals will have impacts on RAN1/RAN2 specifications, these proposals were not further discussed in Release 17.

********** NEXT CHANGED SECTION **********

##### 7.4.2.2.11	Coexistence with GNSS

7.4.2.2.11.1 Reserved

7.4.2.2.11.2 Bands n253, n251, n250

This section is reserved.

##### 7.4.2.2.12	Coexistence with Radio Astronomy (RA)

7.4.2.2.12.1 Band n253, n251, n250

##### This section is reserved.

********** NEXT CHANGED SECTION **********

#### 7.4.3.2	Conducted receiver characteristics

##### 7.4.3.2.1	General and diversity characteristics

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for satellite access.

The handheld UE for satellite access is required to be equipped with a minimum of two Rx antenna ports in all NTN satellite operating bands in FR1.

##### 7.4.3.2.2	Reference sensitivity

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For below 6GHz, the REFSENS level can be calculated by the equation below in TR 38.817-01[32]:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

It is noted that the Rx BW is identical to the transmission bandwidth configuration, which is determined by the spectrum utilization. The RB values in the analysis of this contribution are based on the agreed SU for NR.

SNR in Nagoya meeting was agreed as -1dB for NR.

The REFSENS for E-UTRA assumes that the receiver is equipped with two Rx port as a baseline. If 2Rx is considered, the diversity gain is 3dB and the Implementation Margin (IM) uses 2.5dB. In the following calculation, same assumptions are used for NR NTN satellite bands.

For band n255, 9dB noise figure is assumed which is aligned with band n24.

Both dedicate 30MHz duplexer and n65 duplexer are allowed to be used for band n256. Finally, 9dB noise figure is assumed for band n256 as well.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 from TS 38.101-1 [17] (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 from TS 38.101-1 [17])  with parameters specified in Table 7.4.3.2.2-1 and Table 7.4.3.2.2-2.

Table 7.4.3.2.2-1: Two antenna port reference sensitivity QPSK REFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # |  | SCS kHz |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) |  | 30 MHz (dBm) |  | 35 MHz (dBm) |  | 40 MHz (dBm) |  |
|  |  | 15 |  | -100.0 |  | -96.8 |  | -95.0 |  | -93.8 |  |  |  |  |  |  |  |  |  |
| n255 |  | 30 |  |  |  | -97.1 |  | -95.1 |  | -94.0 |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | -97.5 |  | -95.4 |  | -94.2 |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | -100.0 |  | -96.8 |  | -95.0 |  | -93.8 |  |  |  |  |  |  |  |  |  |
| n256 |  | 30 |  |  |  | -97.1 |  | -95.1 |  | -94.0 |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | -97.5 |  | -95.4 |  | -94.2 |  |  |  |  |  |  |  |  |  |
| n253 |  | 15 |  | -100.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n251 |  | 15 |  | -100.0 |  | -96.8 |  | -95.0 |  | -93.8 |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | -97.1 |  | -95.1 |  | -94.0 |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | -97.5 |  | -95.4 |  | -94.2 |  |  |  |  |  |  |  |  |  |
| n250 |  | 15 |  | -100.0 |  | -96.8 |  | -95.0 |  | -93.8 |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | -97.1 |  | -95.1 |  | -94.0 |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | -97.5 |  | -95.4 |  | -94.2 |  |  |  |  |  |  |  |  |  |

Table 7.4.3.2.2-2: Uplink configuration for reference sensitivity for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # |  | SCS kHz |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) |  | 30 MHz (dBm) |  | 35 MHz (dBm) |  | 40 MHz (dBm) |  |
|  |  | 15 |  | 25 |  | 50 |  | 75 |  | 100 |  |  |  |  |  |  |  |  |  |
| n255 |  | 30 |  |  |  | 24 |  | 36 |  | 50 |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | 10 |  | 18 |  | 24 |  |  |  |  |  |  |  |  |  |
|  |  | 15 |  | 25 |  | 50 |  | 75 |  | 100 |  |  |  |  |  |  |  |  |  |
| n256 |  | 30 |  |  |  | 24 |  | 36 |  | 50 |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  | 10 |  | 18 |  | 24 |  |  |  |  |  |  |  |  |  |
| n253 | 15 | 25 |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |
| n251 | 15 | 25 | 50 | 75 | 752 503 |  |  |  |  |
|  | 30 |  | 24 | 36 | 362 243 |  |  |  |  |
|  | 60 |  | 10 | 18 | 182 103 |  |  |  |  |
| n250 | 15 | 25 |  |  | 752 503 |  |  |  |  |

Table 7.4.3.2.2-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n253 | 5 MHz | -148MHz, -152MHz |
| n251 | 5 MHz | -72.5 MHz, -137.5 MHz |
|  | 20 MHz | -87.5 MHz, -122.5MHz |
| n250 | 5 MHz | -114 MHz, -152 MHz |
|  | 20 MHz | -129 MHz, -137 MHz |

********** NEXT CHANGED SECTION **********

Annex C: Summary of NR-NTN co-existence study

All NR-NTN co-existence study results have been captured in the list. Please see attachment Summary of NR-NTN co-existence study.

[Editor note: see attachment Summary of NR-NTN coexistence study_r1]

Annex C.2: Summary of n253, n251, n250 co-existence simulations

## C.2.1	Company A power back-off analysis for ETSI L-band sub-ranges

### C.2.1.1 ETSI sub-range 1 (1626.5-1660.5MHz)

This section captures preliminary power back-off results for a UE operating in the ETSI sub-band 1 provided by Company A. The following three major test points are considered:

-	a channel in the middle of the band (Fc=1645MHz)

-	a channel at the lower edge of the band (Fc=1629MHz)

-	a channel at the upper edge of the band (Fc=1658MHz)

#### C.2.1.1.1	5MHz channel bandwidth

As can be seen from the Figure 2.3.1.1-1 below, when the NR channel is in the middle of the band, no additional AMPR is anticipated: the out-of-band emission requirements are not relevant because of the size of the band and the in-band ETSI emission mask does not create any additional restrictions either.

Figure 2.3.1.1-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1645MHz).

As for the channels either at the lower or the upper edge of the band, extra A-MPR is needed for larger allocations with DFT-s-OFDM and the right-most channel edge allocations as presented in Figure 2.3.1.1-2 below. And the same observations can be done for the 5MHz channel at the upper edge of the band: left-most channel allocations will need extra A-MPR to comply with the out-of-band emission requirements.

Figure 2.3.1.1-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1629MHz).

Figure 2.3.1.1-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1658MHz).

One of the reasons why the left- and right-most channel power back-off look identical despite the fact that out-of-band emission requirements are not the same is the fact that the limiting factor is -30dBm/30kHz. Two figures below present the spectrum plot for the 5MHz channels at the left and right edges of the band, from which one can see that the most limiting factor is the -30dBm/30kHz emission limit at the offset of 2MHz from the band edge.

Figure 2.3.1.1-4: Spectrum plot for the 5MHz channel at the left edge (top figure) and right edge (bottom figure) of the band.

#### C.2.1.1.2	10MHz channel bandwidth

In this sub-section we present power back-off simulation results for the 10MHz channel. Figure 2.3.1.2-1 presents results for a case when the 10MHz channel resides in the middle of the band; and as can be seen from the figure, no extra A-MPR is needed, i.e. power back-off values are within the MPR margin.

Figure 2.3.1.2-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1645MHz)

Figure 2.3.1.2-2 and 2.3.1.2-3 present results for channels residing at the left and the right edges of the band. General observations are similar to the 5MHz channel, the only noticeable difference is that the 10MHz channel at the upper edge of the band requires slightly higher power back-off due to more stringent out-of-band emission requirements.

Figure 2.3.1.2-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1631.5MHz).

Figure 2.3.1.2-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1655.5MHz).

#### C.2.1.1.3	15MHz channel bandwidth

Figure 2.3.1.3-1 presents results for the 15MHz channel located in the middle of the band. Unlike previous cases for the 5 and 10MHz channels, this time extra A-MPR is need because the power back-off values exceed the MPR margin for edge RB allocations.

Figure 2.3.1.3-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1645MHz).

Power back-off simulation results for the 15MHz channels residing at the band edges are logically identical to the 10MHz results presented in the previous section. The 15MHz channel at the upper edge of the band requires even higher power back-off for large allocations and the for the right-most allocations within the band.

Figure 2.3.1.3-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1634MHz).

Figure 2.3.1.3-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1653MHz).

#### C.2.1.1.4	20MHz channel bandwidth

The 20MHz channel in the ETSI sub-range 1 will require even higher power back-off. As can be seen from Figure 2.3.1.4-1, when the 20MHz channel is in the middle of the band, only edge RB allocations need extra A-MPR.

Figure 2.3.1.4-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1645MHz).

For the channel at the lower edge of the band, the left-most allocations will need the highest power back-off reaching 5-6.5dB. And similarly, the right-most allocations within the channel at the upper edge of the band will require power back-off in the order of  5.5dB.

Figure 2.3.1.4-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1636.5MHz).

Figure 2.3.1.4-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1650.5MHz).

#### C.2.1.1.5		Summary of the power back-off results for ETSI sub-range 1

Based on the presented power back-off results in the previous sub-section the following preliminary A-MPR values are suggested (subject for further checking and revisions).

Table 2.3.1.5-1: A-MPR regions for the ETSI sub-range 1

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | LCRB*12*SCS | A-MPR |
| 5MHz | 1629 <= Fc < 1630.5 | <= 0.72 |  | A1 |
|  | 1656.5 < Fc <= 1658 | >= 3.6 |  | A1 |
| 10MHz | 1631.5 <= Fc < 1634 | <= 2.16 | < 7.2 | A1 |
|  |  |  | >= 7.2 | A2 |
|  | 1653 < Fc <= 1655.5 | >= 6.84 | < 7.2 | A3 |
|  |  |  | >= 7.2 | A2 |
| 15MHz | 1634 <= Fc < 1639 | <= 2.16 |  | A2 |
|  | 1639 <= Fc <= 1648 | <= 1.8, >= 12.02 | <= 1.08 | A3 |
|  | 1648 < Fc <= 1653 | >= 10.08 | < 10.8 | A4 |
|  |  |  | >= 10.8 | A3 |
| 20MHz | 1636.5 <= Fc < 1641.5 | <= 1.8 | <= 7.2 | A5 |
|  |  | >1.8, <=5.4 | <=0.72 | A1 |
|  |  |  | > 7.2 | A4 |
|  |  | >= 18.0 |  | A1 |
|  | 1641.5 <= Fc <= 1645.5 | <= 1.8, >= 17.28 | <= 1.08 | A4 |
|  | 1645.5 < Fc <= 1650.5 | >=13.32 |  | A4 |
|  |  | <=1.08 |  | A1 |

Table 2.3.1.5-2: A-MPR values for the ETSI sub-range 1

|  | Modulation | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  |  |  |  |  |
|  | QPSK | 1.5 | 2.0 | 2.5 | 3.5 | 5 |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |
| CP-OFDM | QPSK | 3.0 | 3.5 | 4.0 | 5.5 | 6.5 |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |

### C.2.1.2	ETSI sub-range 2 (1668-1675MHz)

This section captures preliminary power back-off results for a UE operating in the ETSI sub-band 2 provided by Company A. Since the total sub-band size is 7MHz, only the 5MHz channel is considered at the following test points:

-	a 5MHz channel at the lower edge of the band (Fc=1670.5MHz)

-	a 5MHz channel in the middle of the band (Fc=1671.5MHz)

-	a 5MHz channel at the upper edge of the band (Fc=1672.5MHz)

#### C.2.1.2.1	5MHz channel

Figure 2.3.2.1-1 presents power back-off values for the 5MHz channel at the lower edge of the band. As can be seen most allocations do not need A-MPR, except the case(s) with a single RB allocated at the right edge of the 5MHz channel. The reason for that is the upper edge out-of-band emission requirements which would require some A-MPR. As presented in Figure 2.3.2.1-2 below, CP-OFDM slightly exceeds the limit of -30dBm/30kHz and DFT-s-OFDM is also hitting that limit.

Figure 2.3.2.1-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1670.5MHz).

Figure 2.3.2.1-2: Spectrum plot for the 5MHz channel and 1RB allocation (Fc=1670.5MHz).

Once the 5MHz channel is shifted farther towards the upper edge of the band, even higher A-MPR might be needed, which is however mostly concerns single RB allocations at the upper edge of the configured channel.

Figure 2.3.2.1-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1671.5MHz).

Figure 2.3.2.1-4: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1672.5MHz).

#### C.2.1.2.2		Summary of the power back-off results for ETSI sub-range 2

Based on the presented power back-off results in the previous sub-section we suggest adopting the following preliminary A-MPR values (subject for further checking and revisions).

Table 2.3.2.2-1: A-MPR regions for the ETSI sub-range 2

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | LCRB*12*SCS | A-MPR |
| 5MHz | 1670.5 <= Fc <= 1671.5 | >= 3.6 |  | A1 |
|  | 1671.5 < Fc <= 1672.5 | >= 3.6 |  | A1 |
|  |  |  | >= 3.6 | A1 |

Table 2.3.2.2-2: A-MPR values for the ETSI sub-range 2

|  | Modulation | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  |  |  |  |  |
|  | QPSK | 1.5 |  |  |  |  |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |
| CP-OFDM | QPSK | 3.0 |  |  |  |  |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |