3GPP TSG-WG4 Meeting #116	R4-2511627

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

# Title:	On removing 5MHz CBW from Bands n41, n90

# Agenda Item:	6.12.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution proposes to remove the 5MHz channel bandwidth (CBW) for Bands n41 and n90 based on [1] and way-forward [2].

- Discussion

The smallest DL CBW for Bands n41 and n90 is 5MHz which has been specified since Release 17 as an optional CBW for operation at SCS 15kHz. This situation raises several questions:

- A new CBW is usually introduced as an optional CBW in the current release of its introduction. The status may change in the following Release, i.e. at the end of Rel-18 in this case; however the 5MHz CBW remained optional in Rel-18. With only a few meetings left before the end of Rel-19, one may have expected a status change now that we are nearly three Releases after its introduction. One may consider three ways to handle the status of this CBW:

  - Shall the 5MHz CBW for Bands n41/n90 change from optional to mandatory support in Rel-19?

  - Shall the 5MHz CBW remain optional in Rel-19 and remain optional in Rel-20, i.e. for 4 consecutive releases? If so, what would justify keeping this CBW optional for so long?

  - If no commercial deployment has occurred over 3 Releases, shall the 5MHz CBW be removed from Bands n41/n90?

- Each scenario triggers additional questions/challenges:

  - In case this CBW becomes mandatory, and

    - For band combinations where n41/n90 are the DL affected bands of a two-band or three-band DL CA combination; and

    - Such DL CA combination is specified with BCS4/5.

    - Then, most of the band n41 MSD requirements would need to be re-evaluated because:

      - Most Band n41 MSD requirements are specified for 10MHz CBW; and

      - Yet, based on the RAN4 PRD guidelines, MSD requirements shall be specified for the smallest supported DL CBW.

This scenario would therefore create a significant workload for RAN4.

  - In case this CBW remains optional, RAN4 may decide to ignore this CBW, and the Band n41 MSD requirements would not need to be re-evaluated. However, in this scenario the specifications would remain ambiguous.

  - In case this CBW is removed,

    - For most MSD test points, RAN4 would not need to revisit the Band n41 MSD requirements. There are a few exception cases, like CA_n39-n41 where the Band n41 MSD is specified for UL4/DL3 5MHz CBW.

    - A relatively small clean-up CR would be needed, and the ambiguities would be removed.

As explained in [1], and to avoid the consequences of scenarios a) and b) hereabove, we believe that the 5MHz CBW should be removed from the CBW specified for Bands n41/n90 because this CBW can only deployed using SCS 15kHz. Also, it is our understanding that Band n41 has been deployed throughout several regions of the world using SCS 30kHz only. Hence, there are good reasons to believe that this CBW will not be used in commercial networks and should be removed from TS 38.101-1.

Proposal: Remove the 5MHz CBW support from Band n41 and Band n90. Revisit the legacy Band n41 MSD requirements to ensure that the band n41 MSD requirements are specified for 10MHz CBW.

- Conclusion

This contribution proposes to remove the 5MHz CBW support from Bands n41 and n90. A companion CR is proposed at this meeting.

Proposal: Remove the 5MHz CBW support from Band n41 and Band n90. Revisit the legacy band n41 MSD requirements to ensure that the band n41 MSD requirements are specified for 10MHz CBW.

# References

- R4-2507700 Impact of Rel-19 BCS4_5 new greatest-smallest CBW on MSD, 3GPP TSG RAN Meeting #115, St Julians, Malta, Skyworks Solutions, Inc.

- R4-2508050 WF on optional CHBW across releases, 3GPP TSG RAN Meeting #115, St Julians, Malta, Skyworks Solutions, Inc.