3GPP TSG-WG4 Meeting #116	R4-2511630

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

# Title:	NS_05 A-MPR Considerations

# Agenda Item:	6.5.3

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution shares our views on A-MPR region definition, specifically on ensuring that the total allowed back-off for PC2 (i.e., the maximum of MPR and A-MPR) does not exceed the total back-off allowance for PC3 for any RB allocation, and on the additional allowance to account for PC2 2Tx reverse-IMD. These considerations are based on a small set of power amplifier (PA) emission measurements.

- Discussion

  - General considerations on PC2 vs PC3 A-MPR requirements

With the emergence of PC2 support for most FR1 FDD bands in Rel-19, several A-MPR requirements have been carefully studied. However, A-MPR requirements for Band n1 were the first requirements to be completed and were specified in the last meetings of Rel-17. Due to the lack of extensive studies on the impact of PC2 achieved with 2Tx (“PC22tx”) reverse-IMD, RAN4 agreed to allow a temporary allowance of [2]dB for PC22tx. This relaxation is specified in Note 7 in Table 6.2.3.1-1.

Unfortunately, Note 7 has not been re-visited in Rel-18 and was carried in Rel-19. Recently RAN4 has started to study ways to removing this note from NS_05, NS_48 and NS_49 [1]. The following guideline was proposed in [1], where:

- A-MPR denotes the difference of A-MPR between PC2 and PC3, i.e. A-MPR = PC2A-MPR – PC3A-MPR, and

- In all cases, ensure that A-MPR ≤3 dB, and,

- In cases where A-MPR ≤2.5 dB, allow a [0.5]dB PC22Tx A-MPR relaxation.

However, RAN4 agreed in WF [2] to allow a 1 dB relaxation to account for PC22Tx reverse IMD. RAN4 may need to further discuss general guidelines to ensure that the same rules are applied across all FDD band PC2 A-MPR requirements.

Observation 1: RAN4 is discussing two different sets of assumptions to account for PC22Tx reverse IMD: 1 dB was agreed for new NS_05, and 0.5 dB is discussed for NS_48 / NS_49.

In addition,

- At meeting #115, one leading company commented that the simple check of [1] is not sufficient because there may be cases where a PC2 A-MPR requirement is specified for certain RB allocations/Channel Bandwidth (CBW)/Carrier Frequencies (Fc), and for the same configuration, no PC3 A-MPR is specified, leading to cases where A-MPR exceeds 3 dB.

- It was also observed in [3] that checking A-MPR ≤3 dB was not sufficient and that RAN4 should more generally verify that the total allowed output back-off (OBO) does not differ by more than 3 dB between PC2 (1Tx or 2Tx) operation and PC3, where:

  - The UE total OBO allowance is defined as the maximum of MPRPCx, A-MPRPCx, where here PCx denotes PC2 or PC3,

  - OBOtotal = max(MPRPC2, A-MPRPC2(2Tx)) – max(MPRPC3, A-MPRPC3), and

  - In all cases, ensure that OBOtotal ≤3 dB, and

  - In cases where OBOtotal ≤2.5dB, allow an extra 0.5 dB PC22Tx A-MPR relaxation.

For example, for 20 MHz CBW, SCS = 15 kHz and Fc <1945.7 MHz and RB allocation LCRB = 36(RBstart = 20),

- For PC3: only MPR applies for this inner allocation since the PC3 A3 region is specified for LCRB*12*SCS <5.76 MHz,

- For PC2: The A2 A-MPR applies to this inner allocation, since the A2 region is specified for LCRB*12*SCS <6.48 MHz.

- Table 3 shows that OBOtotal may exceed 3 dB for

  - DFT-s-OFDM Pi.2 BPSK, QPSK and 16QAM

  - CP OFDM QPSK and 16QAM.

  - This delta may increase by 1 dB if the PC22Tx reverse-IMD extra allowance is applied to all modulation orders,

Table 1: New NS_05 20MHz PC2 total OBO vs. PC3 total OBO for LCRB=36(RBstart=20) SCS 15kHz, Fc<1947.5MHz.

| Modulation/ Waveform |  | PC3 MPR inner (dB) | PC3 A-MPR inner (dB) | PC3 total OBO max(MPR,A-MPR) (dB) | PC2 MPR inner (dB) | PC21Tx A2 A-MPR (dB) | PC2 total OBO max(MPR,A-MPR) (dB) | PC21Tx OBOtotal (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 0 | n-a | 0 | 0 | ≤ 4.0 | ≤4.0 | 4 |
|  | QPSK | 0 | n-a | 0 | 0 | ≤ 4.5 | ≤4.5 | 4.5 |
|  | 16 QAM | ≤1 | n-a | ≤1 | ≤1 | ≤ 5.0 | ≤5.0 | 4 |
|  | 64 QAM | ≤2.5 | n-a | ≤2.5 | ≤2.5 | ≤ 5.5 | ≤5.5 | 3 |
|  | 256 QAM | ≤4.5 | n-a | ≤4.5 | ≤4.5 | ≤ 5.5 | ≤5.5 | 1 |
| CP-OFDM | QPSK | ≤1.5 | n-a | ≤1.5 | ≤1.5 | ≤ 6.5 | ≤6.5 | 5 |
|  | 16 QAM | ≤2 | n-a | ≤2 | ≤2 | ≤ 6.5 | ≤6.5 | 4.5 |
|  | 64 QAM | ≤3.5 | n-a | ≤3.5 | ≤3.5 | ≤ 6.5 | ≤6.5 | 3 |
|  | 256 QAM | ≤6.5 | n-a | ≤6.5 | ≤6.5 | ≤ 7.0 | ≤7.0 | 0.5 |

Observation 2: When specifying the PC2 A-MPR, it is not always sufficient to ensure that the difference of PC3/PC2 A-MPR does not exceed 3 dB. Rather, RAN4 should verify that, for a given RB allocation/CBW/Fc, the difference in total output back-off (OBO) does not exceed 3 dB between PC2 and PC3 operation. For a given power class “PCx,” the total OBO = max(MPRPCx, A-MPRPCx).

Proposal 1: It is proposed that RAN4 defines a set of guidelines on specifying the PC2 A-MPR requirements. As a starting point, the guidelines may consider the following:

- Agree upon the PC2 2Tx reverse-IMD extra relaxation, candidates are [0.5 dB for all modulation orders], and [1 dB for certain modulation orders]. Other options are not precluded,

- Agree that RAN4 PC2 requirements should be specified to ensure that:

  - In all cases, OBOtotal ≤3 dB, and

    - Option 1: in cases where OBOtotal ≤2.5 dB, allow an extra [0.5]dB PC22Tx A-MPR relaxation [for all modulation orders],

    - Option 2: in cases where OBOtotal ≤2.0 dB, allow an extra [1.0]dB PC22Tx A-MPR relaxation [for certain modulation orders],

    - Other options are not precluded at this stage and companies are encouraged to provide PC22Tx measurements to further evaluate this relaxation.

Where, OBOtotal = max(MPRPC2, A-MPRPC2(2Tx)) - max(MPRPC3, A-MPRPC3).

  - These rules should be enforced for all RB allocations of a given uplink carrier configuration.

  - Considerations specific to new NS_05 requirements

The Table 1 below uses the example of DFT-s-OFDM QPSK to highlight the NS_05 key differences between the PC3 and the PC2 A-MPR requirements, where:

- The last column entitled “A-MPR” is the difference of A-MPR between PC2 and PC3 for example case of DFT-s-OFDM QPSK. We highlight in red cases where A-MPR may exceed 3 dB, (i.e., cases where PC2 A-MPR > PC3 A-MPR +3 dB),

- Cells highlighted in yellow indicate cases where A-MPR regions may need to be redefined.

Table 1: NS_05 PC3 A-MPR vs PC2 A-MPR comparison for DFT-s-OFDM QPSK based on [2].

| CBW | Fc (MHz) | PC3 A-MPR WF [2] |  |  |  | PC2 A-MPR WF [2] |  |  |  | A-MPR for PC21Tx (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Regions |  | A-MPR Region/ DFTs QPSK A-MPR |  | Regions |  | A-MPR Region/ DFT QPSK A-MPR |  |  |
|  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |  |  |
| 5 MHz | < 1925 | MPR applies. For DFT-s-OFDM QPSK: - Edge/Outer: ≤1.0 - Inner: 0 |  |  |  | ≤ 0.18 | ≥ 3.24 | A1 | ≤ 4.0 | Up to 3 |
|  |  |  |  |  |  | n-a | n-a | A2 |  |  |
|  |  |  |  |  |  | n-a | n-a | A3 |  |  |
| 10 MHz | < 1925 |  |  |  |  | ≥0 | ≥ 1.35*RBstart + 4.32MHz/12/scs | A1 | ≤ 5.5 | Up to 4.5dB |
|  |  |  |  |  |  | ≤ 1.8MHz/12/scs | ≤ 2.16MHz/12/scs | A2 | ≤ 4.0 | Up to 4.0dB |
|  |  |  |  |  |  | ≥0 | < 1.35* RBstart*12*SCS + 4.32 ≥ 1.2* RBstart*12*SCS + 3.6 | A3 | ≤ 4.0 | Up to 4.0dB |
| 10 MHz | 1925 < fc < 1930.7 |  |  |  |  | ≥0 | ≥ 1.35*RBstart + 4.32MHz/12/scs | A1 | ≤ 5.5 | Up to 4.5dB |
|  |  |  |  |  |  | ≤ 1.8MHz/12/scs | ≤ 2.16MHz/12/scs | A2 | ≤ 4.0 | Up to 4.0dB |
|  |  |  |  |  |  | n-a | n-a | A3 | n-a |  |
| 15 MHz | 1927.5 ≤ fc <1932.5 | ≥ 0 | ≥ RBstart*1.2 + 4.68MHz/12/scs | A1 | ≤ 4 | ≥0 | ≥ RBstart*1.2 + 3.96MHz/12/scs | A1 | ≤ 5.5 | 1.5 |
|  |  | ≥ 0 | ≤ 3.96MHz/12/scs | A2 | ≤ 3 | ≤ 3.6MHz/12/scs | ≤ RBstart*1.2 + 3.96MHz/12/scs ≤ 4.5MHz/12/scs | A2 | ≤ 4.5 | 1.5 |
|  |  | ≤ 3.6MHz/12/scs; > 0 | ≤ 3.96MHz/12/scs | A3 | ≤ 1.5 | ≥0 | ≤ RBstart*1.2 + 3.96MHz/12/scs ≥ RBstart*12*SCS * + 3.6 > 4.5MHz/12/scs | A3 | ≤ 4.0 | 2.5 |
| 15 MHz | 1932.5 ≤ fc < 1938.2 | ≤ 0.54/12/scs | ≥ 8.28/12/scs | A1 | ≤ 3 | ≤ 1.8MHz/12/scs | ≥ 8.28MHz/12/ scs | A1 | ≤ 4.5 | 1.5 |
|  |  | ≤ 1.62MHz/12/scs | ≤ 2.16MHz/12/scs | A2 | ≤ 2.5 | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | A2 | ≤ 4.0 | 1.5 |
|  |  | > 1.62MHz/12/scs ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | A3 | ≤ 1.0 | n-a | n-a | A3 |  |  |
| 20 MHz | < 1945.7 | ≥0 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | A1 | ≤ 4 | ≥0 | ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs | A1 | ≤ 5.5 | 1.5 |
|  |  | ≤ 1.8MHz/12/scs | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | A2 | ≤ 3.5 | ≤ 4.68MHz/12/scs | ≤ 6.48MHz/12/scs; < RBstart*1.0 + 4.14MHz/12/scs | A2 | ≤ 4.5 | 1 |
|  |  | > 1.8MHz/12/scs ≤ 4.68MHz/12/scs | ≤ 5.76MHz/12/scs | A3 | ≤ 2.5 | n-a | n-a | A3 |  |  |

We identify 2 cases on the summary Table 1:

- For CBW of 5 MHz and 10 MHz, cases where OBOtotal may exceed 3 dB due to PC2 A-MPR vs PC3 MPR,

- For CBW of 15 MHz and 20 MHz, cases where PC3 and PC2 have different A-MPR region contours. This latter case has already been discussed with the example of observation 2 for 20 MHz CBW

  - Case 1: OBOtotal>3dB for 5MHz and 10MHz CBW

For 5 MHz or 10 MHz CBW and for PC2 outer RB allocations fulfilling the A1 region criterium {RBend*12*SCS>=0; LCRB*12*SCS >1.35*RBstart + 4.32 MHz/12/scs},

- For PC3, only MPR applies, with outer MPR for example,

- For PC2, the A1 A-MPR applies,

- OBOtotal may exceed 3 dB for DFT-s-OFDM Pi.2 BPSK, QPSK, 16QAM and 64QAM, and for CP-OFDM QPSK, 16QAM— see Table 2. This delta may increase due to reverse IMD extra relaxation.

Table 2: NS_05 5,10MHz PC2 A1 A-MPR vs. PC3 MPR comparison, {RBend*12*SCS>=0; LCRB*12*SCS > 1.35*RBstart + 4.32MHz/12/scs}

| Modulation/Waveform |  | PC3 MPR outer (dB) | PC2 MPR outer (dB) | PC2 A1 A-MPR (dB) | PC2 max(MPR,A-MPR) (dB) | PC21Tx OBOtotal (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 0 | ≤ 0.5 | ≤ 4.5 | ≤4.5 | 4.5 |
|  | QPSK | ≤1 | ≤ 1 | ≤ 5.5 | ≤5.5 | 4.5 |
|  | 16 QAM | ≤2 | ≤ 2 | ≤ 6.0 | ≤6 | 4 |
|  | 64 QAM | ≤2.5 | ≤ 2.5 | ≤ 6.0 | ≤6 | 3.5 |
|  | 256 QAM | ≤4.5 | ≤4.5 | ≤ 6.0 | ≤6 | 1.5 |
| CP-OFDM | QPSK | ≤3 | ≤ 3 | ≤ 6.5 | ≤6.5 | 3.5 |
|  | 16 QAM | ≤3 | ≤ 3 | ≤ 6.5 | ≤6.5 | 3.5 |
|  | 64 QAM | ≤3.5 | ≤3.5 | ≤ 6.5 | ≤6.5 | 3 |
|  | 256 QAM | ≤6.5 | ≤6.5 | ≤ 7.0 | ≤7 | 0.5 |

Proposal 2: For 5, 10, 15, and 20 MHz CBW, FFS NS_05 region contour and A-MPR requirements to ensure that OBOtotal does not exceed 3 dB.

  - Simplification for PC2 10 MHz

Observation 4: Based on Table 1, we note that the PC2 A2 and A3 A-MPR requirements are identical for all modulations/waveforms for 10 MHz CBW Fc <1925 MHz.

Proposal 3: Consider merging the PC2 A2 and A3 A-MPR requirements for 10 MHz CBW Fc < 1925 MHz.

  - PC2 Region A3 for 20 MHz

Observation 5: Based on Table 1, we note region the PC3 A3 is not specified for 20 MHz CBW where the PC2 A2 overlays both the PC3 A2 and A3 regions. The PC2 performance for this CBW could be enhanced by at least 1 dB by introducing A3.

  - PC21Tx A-MPR checks

Table 1 summarizes the measured PC21Tx OBO using a band n1 PA calibrated using the usual PC2 calibration assumptions (i.e., PA linearity adjusted to meet –31dBc ACLR at MPR 1 using a DFT-s-OFDM QPSK SCS15kHz LCRB=100(RBSTART=0) waveform, and seven waveforms with the following RF transceiver impairments):

- Carrier leakage: 28 dBc

- Image rejection: 28 dBc

- CIM3: 60 dBc

- CIM5: 70 dBc

Table 5: PC21Tx PA measurement results for 20MHz SCS15kHz DFT-s-OFDM QPSK waveforms

|  | Region | PC21Tx measured OBO (dB) | WF PC21Tx A-MPR (dB) | Minimum Margin to WF A-MPR(dB) |
| --- | --- | --- | --- | --- |
| D4_20M0_1RB0_15 | A2 | 4.2 | 4.5 | 0.3 |
| D4_20M0_6RB0_15 | A2 | 3.2 | 4.5 |  |
| D4_20M0_20RB0_15 | A2 | 2.2 | 4.5 |  |
| D4_20M0_25RB0_15 | A1 | 1.6 | 5.5 | 0.9 |
| D4_20M0_50RB0_15 | A1 | 3.6 | 5.5 |  |
| D4_20M0_75RB0_15 | A1 | 0.4 | 5.5 |  |
| D4_20M0_100RB0_15 | A1 | 4.6 | 5.5 |  |

Observation 6: For 20 MHz CBW region A2, the agreed PC2 A-MPR provides a small 0.3 dB margin. This A-MPR may need to be increased to account for 2Tx reverse IMD. The same observation applies to region A1 in case a 1 dB allowance is applied to some modulation orders for 2Tx.

- Conclusion

This contribution summarizes our views on evaluating the PC2 A-MPR requirements for FDD bands. Some PA measurements indicate that A-MPR may need to be increased for certain regions, while still ensuring that the total allowed back-off for PC2 does not differ by more than 3 dB from the PC3 total back-off.

Observation 1: RAN4 is discussing two different set of assumptions to account for PC22Tx reverse IMD: 1 dB was agreed for new NS_05, and 0.5 dB is discussed for NS_48/NS_49.

Observation 2: When specifying the PC2 A-MPR, it is not always sufficient to ensure that the difference of PC3/PC2 A-MPR does not exceed 3 dB. RAN4 should rather verify that for a given RB allocation/CBW/Fc the difference in total output back-off (OBO) does not exceed 3 dB between PC2 and PC3 operation, where for a given power class “PCx,” the total OBO = max(MPRPCx, A-MPRPCx).

Proposal 1: It is proposed that RAN4 defines a set of guidelines on specifying the PC2 A-MPR requirements. As a starting point, the guidelines may consider the following:

- Agree upon the PC2 2Tx reverse-IMD extra relaxation, candidates are [0.5 dB for all modulation orders], and [1 dB for certain modulation orders]. Other options are not precluded,

- Agree that RAN4 PC2 requirements should be specified to ensure that:

  - In all cases, OBOtotal ≤3 dB, and

    - Option 1: in cases where OBOtotal ≤2.5 dB, allow an extra [0.5]dB PC22Tx A-MPR relaxation [for all modulation orders],

    - Option 2: in cases where OBOtotal ≤2.0 dB, allow an extra [1.0]dB PC22Tx A-MPR relaxation [for certain modulation orders],

    - Other options are not precluded at this stage, and companies are encouraged to provide PC22Tx measurements to further evaluate this relaxation.

Where, OBOtotal = max(MPRPC2, A-MPRPC2(2Tx)) - max(MPRPC3, A-MPRPC3).

  - These rules should be enforced for all RB allocations of a given uplink carrier configuration.

Proposal 2: For 5, 10, 15, and 20 MHz CBW, FFS NS_05 region contour and A-MPR requirements to ensure that OBOtotal does not exceed 3 dB.

Observation 4: Based on Table 1, we note that the PC2 A2 and A3 A-MPR requirements are identical for all modulations/waveforms for 10 MHz CBW Fc <1925 MHz.

Proposal 3: Consider merging the PC2 A2 and A3 A-MPR requirements for 10 MHz CBW Fc <1925 MHz.

Observation 5: Based on Table 1, we note the PC3 A3 region is not specified for 20 MHz CBW where the PC2 A2 overlays both the PC3 A2 and A3 regions. The PC2 performance for this CBW could be enhanced by at least 1 dB by introducing A3.

Observation 6: For 20 MHz CBW region A2, the agreed PC2 A-MPR provides a small 0.3 dB margin. This A-MPR may need to be increased to account for 2Tx reverse IMD. The same observation applies to region A1 in case a 1 dB allowance is applied to some modulation orders for 2Tx.

# References

- R4-2501984 NS_05 NS_48 NS_49 PC2 A-MPR clean-up, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc.

- R4-2508023 WF on PHS protection update, 3GPP TSG-RAN WG4 Meeting #115, St Julians, Malta, KDDI, OPPO, Ericsson.

- R4-2511629 NS_48 A-MPR clean-up to remove HPUE note 7, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Skyworks Solutions, Inc.