3GPP TSG-RAN WG4 Meeting #116	R4-2511632

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.5.2

Source:	Qualcomm Incorporated

Title: 		RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

RAN4 has been discussing various scenarios and issues related to RRM requirements for LP WUR/WUS over the past few meetings and achieved many agreements [2]-[9]. In this paper, we present our views on some open issues.

# LP-WUR operation in FR2

Since other working groups, including RAN4 RF, are actively working on specifying requirements for LP-WUR operation in FR2, we propose that RAN4 also specify the corresponding RRM requirements to ensure consistency and completeness across the specification.

LP-WUR operation introduces two key impacts on RRM requirements:

- Measurement and Evaluation Requirements for LP-WUR

- Relaxed Serving and Neighbor Cell Measurement Requirements for the MR

Measurement and Evaluation Criteria for LP-WUR in FR2

To determine the measurement and evaluation requirements for LP-WUR in FR2, we propose building upon the FR1 requirements by introducing the Rx beam sweeping scaling factor (N1). In FR1, this factor is assumed to be 1 due to omnidirectional reception. However, in FR2, Rx beam sweeping is likely to be enabled on LP-WUR to ensure coverage comparable to the MR.

The key question is: How many Rx beams can be assumed for LP-WUR in FR2?
A straightforward approach is to align the number of Rx beams with those used by the MR.

Proposal 1: Specify LP-WUR measurement and evaluation requirements in FR2 by scaling the FR1 requirements using beam sweeping factor N1.

Proposal 2: For SSB-based measurements, assume N1 is equal to the number of Rx beams used by the MR.

Proposal 3: For OOK-based measurements, assume N1 = 8.

For measurement accuracy, assume the baseband measurement accuracy is same as FR1 and only difference would be in RF inaccuracy which is about 1.5 dB.

Proposal 4: The target measurement accuracy requirements for FR2 is 1.5 dB worse than FR1, to account for the additional RF inaccuracy.

Relaxed Serving and Neighbor Cell Measurement Requirements

In FR1, a measurement relaxation factor of 16 was agreed upon for relaxed serving and neighbor cell measurements for MR when LP-WUR is enabled. We propose reusing this factor for FR2, given the similar constraints and objectives in minimizing power consumption while maintaining basic connectivity.

Proposal 5: For MR-based relaxed serving and neighbor cell measurements in FR2, reuse the measurement relaxation factor of 16 as agreed for FR1.

# LP-WUR operation with eDRX

RAN2 has agreed to support LP-WUS with eDRX. Although RAN4 didn’t receive a formal LS from RAN2 on this topic, given the limited time remaining for this WI completion, RAN4 should discuss whether RRM requirements need to be specified for LP-WUR operation with eDRX.

Our understanding is that when a UE is configured with an eDRX cycle longer than 10.24s, it monitors Paging Occasions (POs) only within a Paging Time Window (PTW). RAN4 currently specifies serving and neighbor cell measurements and cell reselection evaluation periods under the assumption that all measurements are performed within the PTW, even when measurements are performed at a relaxed rate. With full offloading, RAN4 may need to discuss how frequently measurements are performed by the WUR especially outside the PTW and whether the WUR can maintain synchronization and measurement accuracy over long eDRX cycles.

Furthermore, the power savings gain may not be substantial with long eDRX cycles, and in some cases may see negative power savings if the WUR is required to perform measurements more frequently. Even in Rel-16 and Rel-17, RAN4 limited the applicability of neighbor cell measurement relaxations to eDRX cycles up to 163.84s, acknowledging diminishing returns beyond that point.

Given the increased complexity and limited power savings associated with LP-WUS operation during long eDRX cycles (>10.24s), we propose limiting the scope of RRM requirements accordingly.

Proposal 6: RAN4 to specify RRM requirements for LP-WUR operation only for eDRX without PTW (i.e., eDRX cycles <=10.24s).

# Frequency separation between SSB and LP-WUS

RAN1 has agreed that, in IDLE and INACTIVE modes, the LP-WUS/LP-SS frequency resources may be located outside the initial DL BWP, and that there is no restriction on the maximum frequency bandwidth covering both LP-WUS/LP-SS and the SSB. However, RAN1 also noted that it is within RAN4’s scope to introduce such restrictions if deemed necessary.

From an implementation perspective, LP-WUR is expected to be a low-cost receiver, and it is likely that the UE uses a shared RF front-end for both the main receiver (MR) and LP-WUR. In this context, if the LP-WUR is OFDM-based and required to measure SSBs and monitor LP-WUS, then transmitting LP-WUS/LP-SS within the initial DL BWP can significantly simplify the UE design.

Specifically:

- It avoids the need for RF frequency retuning before and after SSB reception.

- It enables simpler signal processing and synchronization, especially when LP-WUR shares hardware resources with the MR.

- It reduces implementation complexity and power consumption, which are critical for low-power devices.

Therefore, the frequency placement of LP-WUS/LP-SS relative to the initial DL BWP has a direct impact on UE complexity and cost. To support simpler UE designs, we propose that LP-WUS/LP-SS be transmitted within the initial DL BWP, preferably within CORESET#0, which is already used for initial control signaling.

Observation: A large frequency separation between SSB and LP-WUS may require frequent RF retuning on the LP-WUR, increasing complexity and power consumption.

Proposal 7: RAN4 requirements on LP-WUR are applicable only when LP-WUS/LP-SS is transmitted within the initial DL BWP of the UE.

# Conclusion

Proposal 1: Specify LP-WUR measurement and evaluation requirements in FR2 by scaling the FR1 requirements using a beam sweeping factor N1.

Proposal 2: For SSB-based measurements, assume N1 is equal to the number of Rx beams used by the MR.

Proposal 3: For OOK-based measurements, assume N1 = 8.

Proposal 4: The target measurement accuracy requirements for FR2 is 1.5 dB worse than FR1, to account for the additional RF inaccuracy.

Proposal 5: For MR-based relaxed serving and neighbor cell measurements in FR2, reuse the measurement relaxation factor of 16 as agreed for FR1.

Proposal 6: RAN4 to specify RRM requirements for LP-WUR operation only for eDRX without PTW (i.e., eDRX cycles <=10.24s).

Proposal 7: RAN4 requirements on LP-WUR are applicable only when LP-WUS/LP-SS is transmitted within the initial DL BWP of the UE.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103

[2] R4-2406367, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#110bis

[3] R4-2410296, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#111

[4] R4-2413899, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112

[5] R4-2416861, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112bis

[6] R4-2420100, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#113

[7] R4-2502600, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#114

[8] R4-2504908, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#114bis

[9] R4-2508280, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#115