3GPP TSG-RAN WG4 Meeting #116	R4-2511638

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.4.1

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# Introduction

In Release 19, the primary objective of the 3GPP work item on the Evolution of NR Duplex Operation: Sub-band Full Duplex (SBFD) is to specify SBFD operation in TDD bands. In addition to this, enhancements to the UE-to-UE Cross-Link Interference (CLI) handling scheme are also part of the work item [1], with expected implications for Radio Resource Management (RRM) requirements.

As we know, RAN1 has agreed to define L1-SRS-RSRP and L1-CLI-RSSI as two new measurement report quantities and has finalized upon method #1, #2 and#3 to be used to perform CLI measurements.

RAN4 has already reached several agreements towards defining RRM requirements for the above three methods, including the measurement delays, scheduling restrictions, measurement restrictions etc., in recent meetings. This paper focuses on a few remaining open issues related to Layer 1 CLI (L1-CLI) requirements.

# L1-CLI measurements with ‘invalid’ symbol type

RAN1 has agreed that for CSI reports associated with periodic or semi-persistent L1-SRS-RSRP and L1-CLI-RSSI measurement resources, the “valid” symbol type configured in the CSI-ReportConfig applies to L1 UE-to-UE CLI measurements when SBFD symbols are configured for the UE. This means that the UE shall perform L1 CLI measurements only during symbols marked as “valid” in the configuration.

RAN4 has also reached the following agreements regarding the number of measurement samples (M) for L1-CLI measurements:

- L1-SRS-RSRP:

  - For periodic/semi-persistent SRS resources:

    - If timeRestrictionForChannelMeasurement is configured: M = [FFS: 1 or 2]

    - Otherwise: M = 3

  - For aperiodic SRS resources: M = 1

- L1-CLI-RSSI:

  - Measurement period is based on M = 1

For CSI-RS-based measurements, RAN4 has already agreed to extend measurement delays to account for missed measurement opportunities due to:

- Collision between CSI-RS resources and dynamic UL transmissions

- Symbols marked as “invalid” by the network

We propose that similar delay extensions should apply to L1-CLI measurements when measurement symbols are marked as “invalid” in the CSI-ReportConfig.

Proposal 1: For a UE configured with SBFD symbols, the measurement delay for L1-CLI-RSSI and/or L1-SRS-RSRP should be extended if one or more of the M measurement occasions fall within symbols marked as “invalid” in the CSI-ReportConfig.

Next, we need to determine the exact delay extension for various cases. For instance, when M=1 (either for L1-CLI-RSSI or L1-RSR-RSRP), the delays are extended until the first ‘valid’ measurement symbol. For M=3, the measurement delays can be extended until UE receives M ‘valid’ symbols.

Proposal 2: The measurement delay for L1-CLI should be extended until the UE has received M ‘valid’ measurement symbols.

# SBFD UE supporting basic feature

RAN1 has defined a basic feature for SBFD-aware UEs, which supports downlink (DL) reception in only one DL subband during SBFD symbols. It is currently under discussion whether these UEs should also support DL reception in both DL subbands. If RAN1 decides against dual-subband DL reception, RAN4 must evaluate whether this limitation should also apply to L1-CLI-RSSI measurement requirements.

For example:

- In Resource Type #1, where L1-CLI-RSSI measurement resources are configured in a single DL subband, the UE should be required to perform measurements only if the resources are located in the same subband where the PDSCH is scheduled.

- In Resource Type #2, where measurement resources span both DL subbands, the UE should be required to measure only the resources that fall within the subband used for PDSCH scheduling.

Observation: For an SBFD UE supporting the basic feature, it is unclear whether the UE is required to measure L1-CLI-RSSI resources configured in a DL subband not used for PDSCH scheduling.

Proposal 3: For an SBFD UE supporting the basic feature, the UE should not be required to measure L1-CLI-RSSI resources configured in a DL subband that is not used for PDSCH scheduling.

# L1-CLI measurement delay requirements

There are two key open issues currently under discussion regarding the determination of L1-CLI measurement delay requirements:

- The number of samples required to determine L1-SRS-RSRP measurement delay when timeRestrictionForChannelMeasurement is configured.

- The number of samples required to determine L1-CLI-RSSI measurement delay when timeRestrictionForChannelMeasurement is/isn’t configured.

We think, for L1-SRS-RSRP measurements, when timeRestrictionForChannelMeasurement is configured, RAN4 should base the delay requirement on a single sample. This aligns with the previously agreed measurement accuracy requirement, which is also based on a single sample.

Similarly, L1-CLI-RSSI measurements delay requirement should be based on a single sample, regardless of whether timeRestrictionForChannelMeasurement is configured or not.

Proposal 4: L1-SRS-RSRP measurements delay requirements are based on a single SRS sample (i.e., M=1) when timeRestrictionForChannelMeasurement is configured

Proposal 5: L1-CLI-RSSI measurement delay should be specified based on a single sample irrespective of whether timeRestrictionForChannelMeasurement is configured or not.

# Conclusion

Proposal 1: For a UE configured with SBFD symbols, the measurement delay for L1-CLI-RSSI and/or L1-SRS-RSRP should be extended if one or more of the M measurement occasions fall within symbols marked as “invalid” in the CSI-ReportConfig.

Proposal 2: The measurement delay for L1-CLI should be extended until the UE has received M ‘valid’ measurement symbols.

Proposal 3: For an SBFD UE supporting the basic feature, the UE should not be required to measure L1-CLI-RSSI resources configured in a DL subband that is not used for PDSCH scheduling.

Proposal 4: L1-SRS-RSRP measurements delay requirements are based on a single SRS sample (i.e., M=1) when timeRestrictionForChannelMeasurement is configured

Proposal 5: L1-CLI-RSSI measurement delay should be specified based on a single sample irrespective of whether timeRestrictionForChannelMeasurement is configured or not.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104