# 3GPP TSG-RAN WG4 Meeting #116 　　　　　　　　　　　　　　　　　	R4-2511835

Bengaluru, India, 25th  – 29th August, 2025

Source: 	NTT DOCOMO INC., Skyworks, Qualcomm

Title:	TP for TR 38.746 adding HPUE CA_n28A-n78A-n79A with 2UL

Agenda item:	6.8.3

Document for:	Approval

- Introduction

This paper provides TP for HPUE CA_n28A-n78A-n79A with dual ul for TR 38.786. All PC2 fallbacks have been specified.

2 Text Proposal

< Start of Text Proposal>

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.X	CA_n28-n78-n79

### 6.X.1	Configurations

Table 6.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n78A-n79A | CA_n28A-n78A7 CA_n28A-n79A7 CA_n78A-n79A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  | CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.X.2	Maximum output power

Table 6.X.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n78-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.X.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.X.3.1	Power class 2 case a, b, c, d

#### Based on calculation,

- IMD2/IMD3/IMD5 due to UL CA_n28A-n78A fall into n79 DL. The IMD2 value is calculated by adding 6dB to the PC3 MSD value, resulting in 32.2dB. PC2 IMD5 value is 5.7dB.

- IMD2/IMD3 due to UL CA_n28A-n79A fall into n78 DL. The IMD2 value is calculated by adding 6dB to the PC3 MSD value, resulting in 32.9dB. PC2 IMD3 value is 16.8dB.

  - NOTE: As the PC3 IMD3 may have been missed, this PC2 value may be revised at RAN4#116-Bis with new the requirement for PC3.

- IMD2 /IMD5 due to UL CA_n78A-n79A fall into n28 DL. The IMD2 value is calculated by adding 6dB to the PC3 MSD value, resulting in 22.2dB. PC2 IMD5 value is 5.7dB.

#### The MSD exception is defined as below.

Table 6.x.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78-n79 | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 32.2 | TDD | IMD23,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4440 | 5.7 | TDD | IMD53,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 32.9 | TDD | IMD23 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3420 | 16.8 | TDD | IMD33 |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 800 | 22.2 | FDD | IMD2 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 5.7 | FDD | IMD5 |
|  | n78 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | TDD | N/A |
| NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.X.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

< End of Text Proposal>

3	References

[1] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone". V19.2.0

[2] 3GPP TR 38.746: “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s)”. V0.4.0

[3] R4-2507935, WF on new MSD rules