3GPP TSG-RAN WG4 Meeting #116                                                               R4-2512530

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.21.1

Source:	Samsung

Title:	Way Forward for [116][306] NR_duplex_evo_General

Document for:	Approval

# Topic #1: General system parameters for SBFD operation

### Sub-topic 1-1: Declaration and flexibility for SBFD operation

##### Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - The threshold for Prated,c,sys is defined as:

    - X = 36 dBm.

##### Issue 1-1-2: How to interpret “the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations”

Agreement:

- For each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), out of all declared configurations,

  - The configuration with the narrowest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:

    - For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL

    - For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1

  - The configuration with the widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:

    - For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL

    - For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

Agreement:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 subband configurations allowed for FR1 WA BS

    - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

    - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

  - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

    - The threshold for Prated,c,sys is defined as:

      - X = 36 dBm.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

Agreement:

- For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

  - This restriction shall be applied to FR1 WA BS

  - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

    - The threshold for Prated,c,sys is defined as:

      - X = 36 dBm.

##### Issue 1-2-4: Restriction on DL subband size

Way forward:

- FFS the following aspect in conformance part for testing applicability:

  - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required.

# Topic #2: SBFD-aware UE related issues

### Sub-topic 2-1: Update for HPUE duty cycle evaluation

##### Issue 2-1-1: The necessity to update for HPUE duty cycle evaluation

Agreement:

- For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

- For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

##### Issue 2-1-2: If the update is needed, how to introduce in RAN4 spec

No further agreement, and the WF from RAN4#114bis can be followed in the maintenance of Rel-19 SBFD WI.

# Topic #4: New BS RF requirements for SBFD

### Sub-topic 4-2: In-channel adjacent subband leakage

##### Issue 4-2-1: Adding note for uniform SBFD configuration across sites

Agreement:

- The following note is added as an informative NOTE, with the understanding that such NOTE is not part of RF requirements:

The RF requirements for SBFD-capable BS have been derived based on the simulation assumption that only SBFD-capable BSs are deployed with the identical SBFD configuration under each BS class in the same geographical area.

##### Issue 4-2-2: In-channel adjacent subband leakage for FR1 WA/MR BS, for DU/UD case

Agreement:

- Absolute limit for in-channel adjacent subband leakage for DU/UD case

  - Reuse legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1) for bands:

Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

  - The necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104 shall be studied in maintenance of Rel-19 SBFD WI

### Sub-topic 4-3: In-channel adjacent subband blocking

##### Issue 4-3-1: Interfering signal level for FR1

Agreement:

- On the level of in-channel adjacent subband blocking requirement:

  - for FR1 WA BS:

    - Adopt -28 dBm.

  - For FR1 MR BS:

    - -32 dBm.

  - For FR1 LA BS:

    - -34 dBm.

##### Issue 4-3-2: Interfering signal level for FR2-1

Agreement:

- On the level of in-channel adjacent subband blocking requirement,

  - For FR2-1 WA BS:

    - the way to define the requirement is similar to FR1, i.e., by considering coupling loss between co-site sectors:

      - Absolute value is expected to be solved in this meeting

        - Baseline:  -52dBm.

##### Issue 4-3-3: Assumption for coupling loss

Agreement:

- Adopt the following assumption in the specification, and the detailed wording can be further refined:

  - For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver.

  - For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver.

##### Issue 4-3-5: Minimum/maximum offset

Agreement:

Option 1:

- bandwidth of the interfering signal should be equivalent to the DL subband

  - Introduce a new symbol FC,SBFD,DL for SBFD DL subband, which is similar to FC used in ICS, to represent the reference point for interfering signal.

  - TP below as starting point for discussion:

- Table 12.3.9.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-H

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + [6] dB | Wide Area BS: -28 + x Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS NRB,SBFD,DL |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL2 | CP-OFDM NR signal 30 kHz SCS NRB,SBFD,DL,1 NRB,SBFD,DL,2 |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: x equals to 10*log10(NRB,SBFD,DL /217) for SBFD DU or UD case, while x equals to 10*log10((NRB,SBFD,DL,1 + NRB,SBFD,DL,2)/217) for SBFD DUD case . |  |  |  |  |  |

Option for discussion

- For single carrier case,

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE2） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + [6.2] dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, x RB |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL2 | CP-OFDM NR signal 30 kHz SCS, x RB |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: if DL subband size is >= 51, x =51 (20MHz CBW), otherwise x =24, (10MHz CBW) |  |  |  |  |  |

For CA case,

using the same frequency offset as in-band blocking

| SBFD UL subband (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | SBFD DU/UD or DUD case | Interfering signal centre frequency offset (MHz) (Note 3) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + 6.2 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | 30 (for UD case) or -30 (for DU case) | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |
|  |  |  | DUD | 30 (for lower DL subband) and -30 (for higher DL subband) |  |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2.1. |  |  |  |  |  |

# Topic #5: BS RF conformance requirements for SBFD

### Sub-topic 5-1: General assumptions of conformance testing

##### Issue 5-1-1: System parameters of conformance testing

Agreement:

- For system parameters of conformance testing:

  - The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2.

  - The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2.

##### Issue 5-1-2: BS type for SBFD conformance testing

Agreement:

- For BS type for SBFD conformance testing:

  - The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1.

##### Issue 5-1-3: Declaration of supported SBFD configuration

Agreement:

- Declaration of supported SBFD configuration:

  - At least the following declarations shall be captured in TS 38.141-1/-2:

    - TX power for SBFD;

    - Bands supported for SBFD;

    - Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104;

    - SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

    - Other declarations are not precluded

##### Issue 5-1-4: Testing applicability for supported SBFD configuration

Way forward:

- Further study on the following proposals on test configuration:

  - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. For the restriction on guardband size of SBFD

  - Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations.

##### Issue 5-1-5: RF channels to be tested for SBFD operation

Way forward:

- Further study on the following proposals on RF channels to be tested for SBFD operation:

  - When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

### Sub-topic 5-2: Conformance testing for new RF requirements

##### Issue 5-2-1: Conformance testing related to transient period between SBFD and non-SBFD

Way forward:

- Further study on the following proposals on conformance test for transient period between SBFD and non-SBFD:

  - For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

##### Issue 5-2-2: Conformance testing related to in-channel adjacent subband leakage

Way forward:

- Further study on the following proposals on conformance test for in-channel adjacent subband leakage:

  - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

  - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

  - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

##### Issue 5-2-3: Conformance testing related to in-channel adjacent subband blocking

Way forward:

- Further study on the following proposals on conformance test for in-channel adjacent subband blocking:

  - For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

  - For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

  - For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.