# 3GPP TSG-RAN WG4 Meeting #116	R4-2512569

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on 38.101-4 for UE demodulation performance requirements for 8Rx with inter-cell interference scenario(FDD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel -19, a new demodulation requirements for 8Rx with inter-cell interference cancellation was introduced, new requirements shall be added into specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a demodulation requirements for Rel-19 8Rx interference feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 8Rx interference feature will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2510979 |  |  |  |  |  |  |  |  |

#### < START OF CHANGE>

##### 5.2.4.1.2	Minimum requirements for PDSCH with inter-cell interference

The performance requirements are specified in Table 5.2.4.1.2-3 to Table 5.2.4.1.2-6, with the addition of test parameters in Table 5.2.4.1.2-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.4.1.2-1.

Table 5.2.4.1.2-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance under 8 receive antenna conditions, when the PDSCH transmission from the serving cell is interfered by 1 or 2 interfering cells. | 1-1, 2-1,3-1,4-1 |

Table 5.2.4.1.2-2: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for Test 1-1 and 2-1 Disabled for Test 3-1 and 4-1 |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 2 for Test 1-1 and 2-1, 4 for Test 3-1 and 4-1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-1 and 2-1 N/A for Test 3-1 and 4-1 |
| Time offset to Cell 1 |  | us | N/A | 3 | -1 |
| Frequency shift to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for Test 1-1 and 2-1 7.58 for Test 3-1 and 4-1 | 2.29 for Test 1-1 and 2-1 N/A for Test 3-1 and 4-1 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | 1st SSB in Slot#0 for Test 1-1 and 2-1 2nd SSB in Slot #0 for Test 3-1 and 4-1 | 1st SSB in Slot#0 for Test 1-1 and 2-1 N/A for Test 3-1 and 4-1 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note 1: Cell 1 is the serving cell, Cell 2, 3 are interference cells. Note 2: INR is defined in Annex B.6.1 |  |  |  |  |  |

Table 5.2.4.1.2-3: Minimum performance for rank 2 with Baseline 8Rx receiver

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-25.2 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 2x8, ULA Low | 70 | 11.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Subcarrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

Table 5.2.4.1.2-4: Minimum performance for rank 2 with Simplified 8Rx receiver

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-25.2 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 2x8, ULA Low | 70 | 13.8 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Subcarrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

Table 5.2.4.1.2-5: Minimum performance for rank 4 with Baseline 8Rx receiver

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.1-2.4 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x8, ULA Low | 70 | 11.0 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Subcarrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

Table 5.2.4.1.2-6: Minimum performance for rank 4 with Simplified 8Rx receiver

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.1-2.4 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x8, ULA Low | 70 | 16.3 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Subcarrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

#### < End OF CHANGE>