3GPP TSG-RAN WG4 Meeting #116	R4-2512617

Bengaluru, India, Aug 25th – 29th, 2025

Title: 	Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx

Agenda Item:	7.1.4

Source: 	Huawei, HiSilicon

Document for:	Approval

1 General

Issue 1-1: Test applicability rule for 6Rx tests

- Proposals

- Option 1: Introduce the test applicability rules by following core part agreement.

  - 6 MIMO layers performance requirements will be only applicable for FWA devices

  - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

- Option 1a: Introduce UE declaration for the proper test applicability design if no explicit capability signalling finally confirmed by RAN2

- Option 2: Postpone the discussion until RAN2 concludes the UE capability signalling discussion

Issue 1-2: Test applicability rule for different number Rx antenna ports

- Proposals

- Option 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage as following.

| Supported RX antenna ports | Test type | Test list |  |
| --- | --- | --- | --- |
| UE supports 6Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.X (Note 2) | If UE passes tests in Clause 5.2.X, UE can skip Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
| UE supports both 6Rx and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports 8Rx, 6Rx, 4Rx and 2Rx or UE supports both 8Rx, 6Rx, and 4Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
|  | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 8Rx, 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx Or UE supports both 8Rx and 6Rx | PDSCH | All tests in Clause 5.2.4 (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| UE supports only 6Rx | PDSCH | All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx or 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx or 6 Rx with data source from system simulator, and the other 4 Rx or 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx or 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx or 6 Rx with data source from system simulator, and the other 6 Rx or 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1. |  |  |  |

- Option 2: Postpone the discussion of test applicability rule until the relevant test cases are better defined

Issue 1-3: Specification structure

- Proposals

- Option 1: Add a new clause for 6Rx after the current 8Rx performance requirements

  - Section 5.2.5 6Rx requirement

- Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

  - Section 5.2.3 4Rx requirements

  - Section 5.2.4 6Rx requirements

  - Section 5.2.5 8Rx requirements

2 PDSCH requirements for 6Rx

2.2 Performance requirements for 4 Layers

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE

- Proposals

  - Option 1: One single test case that is applicable for both FWA and Handheld UE

    - Option 1a: α = 0 and β = 0.07179

    - Option 1b: α = 0 and β = 0

  - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

Issue 2-2-2: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

- Agreement:

  - MCS 17 (Table 1)

2.3 Performance requirements for 6 MIMO Layers for FWA

Issue 2-3-1: Test applicability rule for FWA

- Proposals

- Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests

- Option 2: Not introduce additional applicability rule to skip corresponding 4 layers tests if it passes the FWA 6 layers tests.

Issue 2-3-2: DMRS configuration for 6 MIMO layers case

- Proposals

- Option 1: Rel-15 Type 1 DMRS 2+2

  - DMRS ports {1000-1003, 1004, 1006}

- Option 2: Rel-18 eType1 DMRS 1+1

2.4 Summary of test parameters for 6Rx PDSCH tests

Issue 2-4-1: Test parameters for 6Rx PDSCH tests

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: Refer to Issue 2-2-1 and 2-2-2 - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers - ULA Low: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-2 |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  |  | SNR@70% max TP |

3 SDR requirements for 6Rx

Issue 3-1-1: Static channel and precoding for 6Rx

- Agreement

- For 4T6R (Random precoding): confirm the agreement made in RAN4#115

- H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

- Agreement

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

- H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Issue 3-1-2: SDR requirements for 1024QAM with 4 layers

- Proposals

- Option 1: Not Define SDR requirements for 1024QAM with 4 layers

- Option 2: Consider MCS23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers

Issue 3-1-3: UE capability for support 1024QAM with 4 layers for 6Rx (if introduced)

- Proposals

- Option 1: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced

- Option 2: Not define the UE capability pdsch-1024QAM-4MIMO-FR1-r17

Issue 3-1-4: SDR requirements

- Agreement

- Decide the MCS values as per the agreed static channel matrix in Issue 3-1-1 and parameters in Table 5.5A-1/2/3 of TS 38.101-4 in the next meeting

  - QPSK/16QAM/64QAM/256QAM with 6 MIMO layers as below table

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

  - 1024QAM with 2 MIMO layers as below table

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | TBD |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

4 CSI requirements for 6Rx

Issue 4-1-1: CQI test for 6Rx

- Proposals

- Option 1: Not define CQI test

- Option 2: Only define one CQI requirements with rank 4 under static channel

Issue 4-1-2: RI test for only Rank 6

- Proposals

- Option 1: Not define RI tests for 6Rx

- Option 2: Define RI test for 6Rx