3GPP TSG-RAN WG4 #116	R4-2509030

Bengaluru, India, August 25-29, 2025

Agenda item:	7.30.2		UE demodulation for time/frequency interleaving

Source:	Qualcomm Incorporated

Title:						On demodulation test cases for time-frequency interleaving for 5G Broadcast

Document for:	Discussion

# Introduction

Unlike other state-of-the-art broadcast standards, LTE-based 5G broadcast does not support time-frequency interleaving, which severely impairs performance in the presence of multipath and fading channels. The work item in Rel-19 on LTE-based 5G Broadcast Phase 2 addresses this shortcoming [1].

According to the work item planning, RAN4 is supposed to start working on the performance requirements in RAN4 #116. In this paper we provide our view on the demodulation performance requirements for time-frequency interleaving in LTE-based 5G broadcast.

# Discussion

According to the work item description in [1], time-frequency interleaving is specified that maps one transport block to multiple non-consecutive subframes, following the principles of NR TBoMS. The latest status report of the work item is provided in [2]. It has already been mentioned in the work item that time-frequency interleaving is not applicable to SCS = 0.37 kHz subcarrier spacing.

Figure 2-1 shows the structure of the time-frequency interleaving. The original TBS is scaled by a factor of N and the N-scaled transport block is transmitted over N non-contiguous subframes. The parameter M indicates the spreading in time domain. According to [2], RAN1 defined values for M = {4, 8, 16, 32} and N = {1, 2, 3, 8, 16}.

Figure 2-1: Time-Frequency Interleaving Structure

Observation 1: RAN1 has agreed on the following values for M and N: M = {4, 8, 16, 32} and N = {1, 2, 4, 8, 16}

The supported scaled TB size is limited by the soft buffer size of the device. Hence, a (M, N)-tuple with values in the middle of the supported range should be tested. However, it is also important to show maximal performance gain of the time-frequency interleaving. This can be achieved by choosing the maximal values of (M, N).

Proposal 1: Both (M, N) = (8, 4) and (M, N) = (32, 16) should be chosen for defining demod requirements for time-frequency interleaving.

Proposal 2: The corresponding TBS for (M, N) = (8, 4) should be moderate and the corresponding TBS for (M, N) = (32, 16) should be low.

As a starting point for the simulations, we propose the following:

Proposal 3: As a starting point for the simulations, we propose to use 31704 bits scaled TBS for a 7992 bits base TBS for (M, N) = (8, 4) with 16QAM and for (M, N) = (32, 16), we propose 45352 bits scaled TBS for a 2856 bits base TBS with QPSK.

These transport block sizes could be refined if we see in initial simulations that the proposal does not work well.

Since SCS = 0.37 kHz has been excluded from the work item, all other subcarrier spacings {1.25, 2.5, 7.5, 15} kHz are applicable to 5G broadcast phase 2

Observation 2: The subcarrier spacings {1.25, 2.5, 7.5, 15} kHz are applicable to 5G broadcast phase 2.

Since the deployment is an SFN-network, it can be expected that the delay spread of the channel is large. Hence, a large cyclic prefix is needed to avoid inter-symbol interference and a small subcarrier spacing like 1.25 kHz or 2.5 kHz is required. We propose to limit the dmeod tests to 1.25 kHz since this is the most likely deployment.

Observation 3: Since the deployment is an SFN-network, it can be expected that the delay spread of the channel is large. A large cyclic prefix is needed to avoid inter-symbol interference.

Proposal 4: Demod requirements should be defined for SCS = 1.25 kHz.

Another parameter to be decided is the channel bandwidth. Originally, a bandwidth of 10 MHz was introduced for broadcast. In later releases broadcast was enhanced to support also 6/7/8 MHz and new bands have been defined. We propose to define demod requirements for 8 MHz.

Proposal 5: Demod requirements should be introduced for 8 MHz channel bandwidth.

Section B.2.6.2-1 in [3] already defines a channel model for SCS = 1.25 kHz with extended delay spread for a maximal Doppler frequency of 5 Hz. We think that this channel model can be reused for low Doppler scenarios. However, it is important to demonstrate that there are also performance gains if time-frequency interleaving at higher velocities, i.e., at higher Doppler frequencies. Hence, we propose to enhance the existing channel model in section B.2.6.2-1 to also support higher velocities.

Proposal 6: Time-frequency interleaving should be tested for Doppler frequencies of 5 Hz and 50 Hz.

Proposal 7: For SCS = 1.25 kHz and Doppler frequency fD = 5 Hz, the channel model in section B.2.6.2-1 of [3] should be reused.

Proposal 8: For SCS = 1.25 kHz and Doppler frequency fD = 50 Hz, the channel model in section B.2.6.2-1 of [3] should be enhanced accordingly.

So far, two UE features have been introduced for time-frequency interleaving, namely time interleaving and frequency interleaving that can be separately configured. In our view, both time and frequency interleaving should be enabled jointly and, hence, also be tested jointly.

Observation 4: For optimal performance, both time and frequency interleaving should be enabled jointly and, hence, also be tested jointly.

Proposal 9: In the demod requirement test cases, both time and frequency interleaving should be enabled jointly.

In our understanding, RAN1 will introduce one more UE capability for codeblock cycling. However, we do not believe that it is necessary to define separate test cases for this.

Observation 5: In our understanding, RAN1 will introduce one more UE capability for codeblock cycling. However, we do not believe that it is necessary to define separate test cases for this.

# Conclusions

In this paper we provided our views on the demodulation performance requirements for time-frequency interleaving in LTE-based 5G broadcast. Our observations and proposals are summarized as follows:

Observation 1: RAN1 has agreed on the following values for M and N: M = {4, 8, 16, 32} and N = {1, 2, 4, 8, 16}

Proposal 1: Both (M, N) = (8, 4) and (M, N) = (32, 16) should be chosen for defining demod requirements for time-frequency interleaving.

Proposal 2: The corresponding TBS for (M, N) = (8, 4) should be moderate and the corresponding TBS for (M, N) = (32, 16) should be low. Exact values are FFS.

Proposal 3: As a starting point for the simulations, we propose to use 31704 bits scaled TBS for a 7992 bits base TBS for (M, N) = (8, 4) with 16QAM and for (M, N) = (32, 16), we propose 45352 bits scaled TBS for a 2856 bits base TBS with QPSK.

Observation 2: The subcarrier spacings {1.25, 2.5, 7.5, 15} kHz are applicable to 5G broadcast phase 2.

Observation 3: Since the deployment is an SFN-network, it can be expected that the delay spread of the channel is large. A large cyclic prefix is needed to avoid inter-symbol interference.

Proposal 4: Demod requirements should be defined for SCS = 1.25 kHz.

Proposal 5: Demod requirements should be introduced for 8 MHz channel bandwidth.

Proposal 6: Time-frequency interleaving should be tested for Doppler frequencies of 5 Hz and 50 Hz.

Proposal 7: For SCS = 1.25 kHz and Doppler frequency fD = 5 Hz, the channel model in section B.2.6.2-1 of [3] should be reused.

Proposal 8: For SCS = 1.25 kHz and Doppler frequency fD = 50 Hz, the channel model in section B.2.6.2-1 of [3] should be enhanced accordingly.

Observation 4: For optimal performance, both time and frequency interleaving should be enabled jointly and, hence, also be tested jointly.

Proposal 9: In the demod requirement test cases, both time and frequency interleaving should be enabled jointly.

Observation 5: In our understanding, RAN1 will introduce one more UE capability for codeblock cycling. However, we do not believe that it is necessary to define separate test cases for this.

# References

- RP-250794, Revised WID on LTE-based 5G Broadcast Phase 2, RAN #107, March 2025

- RP 251143, Status Report LTE-based 5G Broadcast Phase 2, RAN #108, June 2025

- TS 36.101, User Equipment (UE) radio transmission and reception (Release 19), v.19.2.0, June 2025