3GPP TSG-WG4 Meeting #116	R4-2511688

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3036 | rev | 1 | Current version: | 16.23.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_CADC_R16_2BDL_xBUL) CR to TS38.101-1-gn0: Correction to harmonic mixing MSD – CatF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc., Anritsu, Qualcomm France. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R16_2BDL_xBUL |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no direct-hit for CA_n28-n78 harmonic mixing MSD requirements. This CR brings the corrections discussed in R4-2501987 and ported to subsequent releases based on the agreed WF R4-2502956. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 7.3A.4-4: - CA_n28-n78: introduces a single MSD requirement to reflect the near-miss Rx harmonic mxing interference which affects band n28. Test conditions are specified with new footnote 6 and reflected in uplink configuration table 7.3A.4-4a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CA_n28-n78 harmonic mixing MSD requirements remain erroneous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

<UNCHANGED SECTIONS ARE SKIPPED>

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-4 with uplink configuration specified in Table 7.3A.4-4a. Sensitivity degradation is not required for receiver even order harmonic mixing with aggressor 3rd order and above harmonic interference.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | n713,4 | 26.5 | 23.3 | 20.9 | 15.3 |  |  |  |  |  |  |  |  |  |
| n40 | n284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |  |  |
| n77 | n2 | 6.7 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |  |  |
| n78 | n286 | 6.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| n78 | n402 | 10.4 | 10.4 | 10.4 | 10.4 |  |  | 7.2 | 6.2 | 5.5 |  | 4.5 |  |  |
| n78 | n412 |  | 10.4 | 10.4 | 10.4 |  |  | 8.2 | 7.6 | 7.3 |  | 6.6 | 6.4 | 6.3 |
| n79 | n85 | 25 | 21.8 | 19.4 | 13.8 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 6: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | n28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n77 | n2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n77 | n5 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |  |
| n78 | n28 | 15 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |
| n78 | n40 | 30 | 50 | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 |  |  |
| n78 | n41 | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n79 | n8 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<END OF CHANGES>