3GPP TSG-RAN4 Meeting #116	R4-2512354

Bengaluru, India, 25 August – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6020 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on RRM requirements for LP-WUS/WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Big CR to capture all endorsed draft CRs for TS 38.133 for NR_LPWUS-Core WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR includes contents of the following CRs endorsed in RAN4#116: R4-2512209; Draft CR to TS 38.133 on definitions, symbols and abbreviations; CATT. R4-2512210; Draft CR to TS 38.133 on measurements of inter-RAT E-UTRAN cells for UE with LP-WUR; CATT. R4-2512211; DraftCR on measurements of intra-frequency NR cells for UE with LP-WUR in IDLE and INACTIVE state; OPPO. R4-2512212; Draft CR for Measurements of inter-frequency NR cells for UE with LP-WUR; LG Electronics Inc.. R4-2512213; (NR_LPWUS) draftCR on 5.X.2.3 MR serving cell measurement and evaluation requirements; Xiaomi. R4-2512214; (NR_LPWUS-Core) draftCR on Introduce inter-frequency and inter-RAT cell re-selection requirements for UE with LP-WUR in RRC_Inactive; CMCC. R4-2512215; Draft CR for inactive state LP-WUR serving cell measurement and evaluation requirements; vivo. R4-2512216; draftCR on MR measurement requirements for serving cell in IDLE; Huawei, HiSilicon, vivo. R4-2512217; draftCR on LP-WUS introduction; Ericsson. R4-2512218; (NR_LPWUS-Core) LP-WUR and MR measurement capability in IDLE state; ZTECorporation,Sanechips. R4-2512219; (NR_LPWUS-Core) LP-WUR and MR measurement capability in INACTIVE state; ZTECorporation,Sanechips. R4-2512220; DraftCR to TS 38.133 on LP-WUR requirements; Nokia. |
| Consequences if not approved: |  | RRM requirements for LP-WUR are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3; 3.6.x (new); 4.x (new); 5.x (new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

ATG	Air to Ground

AWGN	Additive White Gaussian Nouse

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CCE	Control Channel Element

CG-SDT	Configured Grant Small Data Transmission

CHO	Conditional Handover

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CN	Core Network

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DAPS	Dual Active Protocol Stack

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DPC	Delta Power Class

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

eDRX	Extended DRX

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EMR	Early measurement reporting

EMW	Effective measurement window

EMWRP	Effective measurement window repetition period

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FH	Frequency Hopping

FR	Frequency Range

GEO	Geostationary Earth Orbit

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

HARQ	Hybrid Automatic Repeat Request

HO	Handover

HST	High Speed Train

GAP	Refers to any of Measurement Gap, activated Pre-MG and NCSG

IMR	Interference Measurement Resource

kHz	Kilo Hertz

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LEO	Low Earth Orbit

LMF	Location Management Function

LO 	LP-WUS Occasion

LPP	LTE Positioning Protocol

LP-SS 	Low Power-Synchronization Signal

LP-WUR 	Low Power-Wake up Receiver

LP-WUS 	Low Power-Wake up Signal

LR 	Low Power-Wake up Receiver

LTM	L1/L2 triggered mobility

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MHz	Mega Hertz

MIB	Master Information Block

ML	Measurement Length

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MR 	Main Receiver

MUSIM	Multi-Universal Subscriber Identity Module

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NGSO	Non-Geosynchronous Orbit

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OOK 	On-Off Keying

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PCI	Physical Cell Identity

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurement Gap

ProSe	Proximity-based Service

PRB	Physical Resource Block

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PPW	PRS Processing Window

PPWL	PRS Processing Window Length

PPWRP	PRS Processing Window Repetition Period

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAN	Radio Access Network

RAT	Radio Access Technology

RF	Radio Frequency

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMC	Reference Measurement Channel

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRH	Remote Radio Head

RRM	Radio Resource Management

RRT	RF Retuning Time

RS	Reference Signal

RSCP	Reference Signal Carrier Phase

RSCPD	Reference Signal Carrier Phase Difference

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTD	Receive Timing Difference

RTOA	Relative Time Of Arrival

RTT	Round Trip Time

S-SSB	Sidelink Synchronization Signal Block

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector or radiated interface boundary.

SA	Standalone operation mode

SAB	Satellite access band

SAN	Satellite Access Node

SCC	Secondary Component Carrier

SCCH	Sidelink Control Channel

SCell	Secondary Cell

SCG	Secondary Cell Group

SCH	Synchronization Channel

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SL AoA	Sidelink AoA

SL PRS-RSRP	Sidelink PRS-based RSRP

SL PRS-RSRPP	Sidelink PRS-based RSRPP

SL RSTD	Sidelink RSTD

SL RTOA	Sidelink RTOA

SL Rx-Tx	Sidelink Receive-Transmit time difference

SL-PRP	SL-PRS Received Power

SL-PRS	Sidelink PRS

SL-RSSI	Sidelink Received Signal Strength Indicator

SLPP	Sidelink Positioning Protocol

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDCP	Time Domain Channel Properties

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TE	Test Equipment

TN	Terrestrial Network

TRP	Transmission-Reception Point

TRS	Tracking Reference Signal

TTI	Transmission Time Interval

U2N	UE-to-Network

U2U	UE-to-UE

UE	User Equipment

UL	Uplink

V2X	Vehicle-to-Everything service

VIL	Visible Interruption Length

VIRP	Visible Interruption Repetition Period

VSAT	Very Small Aperture Terminal

<End of Change 1>

<Start of Change 2>

3.6.x	Applicability of requirements for UE with LP-WUR

The requirements for LP-WUR and LP-WUS apply under the assumption that the LR and the MR are operating on the same carrier frequency.

<End of Change 2>

<Start of Change 3>

4.x	IDLE mode measurement for LP-WUS operation

4.x.1	Introduction

The UE which supports LP-WUS capability in RRC_IDLE shall be capable of:

- performing serving cell measurement based on LP-SS or SSB by LR.

- performing serving cell and neighbour cell measurement relaxation by MR together with LP-SS or SSB based serving cell measurement by LR.

4.X.2 Requirements

4.X.2.1 UE Measurement Capability

4.x.2.1.1 LP-WUR measurement capability

UE LP-WUR shall be capable of monitoring at least the serving cell.

4.X.2.1.2 MR measurement capability with LP-WUR

For the UE measurement capability, the requirements in clause 4.2.2.1 apply.

For the Redcap UE measurement capability, the requirements in clause 4.2B.2.1 apply.

<End of Change 3>

<Start of Change 4>

4.x.2.2 LP-WUR Serving cell measurement and evaluation requirement

4.x.2.2.1 General description

This sub-clause specifies the serving cell measurements and evaluation requirements for a UE with LP-WUR in RRC_IDLE State.

The requirements apply for to a UE supporting FG-62-1 or FG-62-1a and when the corresponding evaluation thresholds are configured by higher layers.

The requirements in this clause 4.x.2 apply when the LP-WUR is in ON state.

Before entering LP-WUS monitoring, RRM offloading or RRM relaxation mode and after exiting LP-WUS monitoring, RRM offloading and RRM relaxation mode, it is up to UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement.

When LP-WUR is in the ON state, the UE may perform serving cell measurements based on LP-SS or PSS/SSS, where the requirements for LP-SS based measurements and evaluations in the clause 4.x.2 apply only if the LP-SS is configured and transmitted on the same carrier frequency as the serving cell.

The requirements in section 4.2 apply when the network has not configured thresholds for RRM measurement offloading or RRM measurement relaxation, or when the corresponding conditions are not fulfilled. If the UE applies RRM measurement offloading or RRM measurement relaxation, it shall perform measurements based on LP-SS or PSS/SSS following the requirements specified in section 4.x.2.2 or 4.x.2.3.

The requirements in this clause apply for Redcap UE supporting FG-62-1 or, FG-62-1a.

LP-WUR evaluation requirements specified in 4.x.2.2 and 4.x.2.3 apply to LP-WUR entry and exit criteria evaluation.

Prior to performing LP-WUR evaluation specified in 4.x.2.2 or 4.x.2.3, if configured, the UE shall meet the corresponding LR entry criteria at least once for:

-	entry condition for LP-WUS monitoring

-	entry condition for RRM relaxation

-	entry condition for RRM offloading

4.x.2.2.2 LP-WUR measurement and evaluation requirements for PSS/SSS

Upon meeting the entry conditions for RRM offloading or RRM relaxation, the UE shall measure the SS-RSRP and SS-RSRQ level once every LO cycle and evaluate whether one or more of the following conditions defined in TS 38.304 [1], if configured, are met within Tevaluate-LP-WUR-PSS/SSS

-    exit condition for LP-WUS monitoring

-    exit condition for RRM offloading

-    exit condition for RRM relaxation

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurement samples. Within the set of measurements used for the filtering, at least two measurement samples shall be spaced by LO-periodicity/2.

Table 4.x.2.2-1: Tevaluate-LP-WUR-PSS/SSS for FR1 and FR2

| LO periodicity [s] Note 1 | Scaling Factor (NLP-WUS) |  | Tevaluate-LP-WUR-PSS/SSS (number of LO Cycles [s]) |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 0.32 | 1 | 8 | 0.32 x 4 x NLP-WUS (1.28s x NLP-WUS) |
| 0.64 |  | 5 | 0.64 x 4 x NLP-WUS (2.56s x NLP-WUS) |
| 1.28 |  | 4 | 1.28 x 4 x NLP-WUS (5.12s x NLP-WUS) |
| 2.56 |  | 3 | 2.56 x 4 x NLP-WUS (10.24s x NLP-WUS) |
| Note 1: The LO periodicity is the same as the configured DRX cycle length |  |  |  |

The UE shall evaluate and consider an entry or exit criteria is fulfilled within Tevaluate-LP-WUR-PSS/SSS, provided that the corresponding criteria is met by a margin of 6 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR1 and by a margin of 7.5 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR2 when SSB Ês/Iot ≥ -3dB

Upon fulfilling a configured entry or exit condition, the UE shall perform corresponding actions as defined in clause 5.2 in TS 38.304 [1].

The requirements in this clause apply for UE which supports FG 62-1a and measures PSS/SSS.

4.x.2.2.3 LP-WUR measurement and evaluation requirements for LP-SS

Upon meeting the entry conditions for RRM offloading or RRM relaxation, the UE shall measure the LP-RSRP and LP-RSRQ level once every LP-SS cycle and evaluate whether one or more of the following conditions defined in TS 38.304 [1] are met within Tevaluate-LP-WUR-LP-SS

-	exit condition for LP-WUS monitoring

-    exit condition for RRM offloading

The UE shall filter the LP-SS measurements of the serving cell using at least 2 measurement samples.

Table 4.x.2.3-1: Tevaluate-LP-WUR-LP-SS

| LP-SS periodicity [s] | Tevaluate-LP-WUR-LP-SS (number of LP-SS Cycles [s]) |
| --- | --- |
| 0.16 | 0.16 x 6 (0.96s) |
| 0.32 | 0.32 x 6 (1.92s) |

The UE shall evaluate and consider an entry or exit criteria is fulfilled within Tevaluate-LP-WUR-LP-SS, provided that the criteria is met by a margin of 6 dB for LP-RSRP and/or 3.5 dB for LP-RSRQ in FR1 when LP-SS Ês/Iot ≥ -3dB

Upon fulfilling a configured entry or exit condition, the UE shall perform corresponding actions as defined in clause 5.2 in TS 38.304 [1].

The requirements in this clause apply for UE which supports FG 62-1, or UE which supports FG 62-1a-LP-SS and measures only LP-SS.

4.X.2.3	Measurement and evaluation of serving cell by MR

The requirements in this claue apply for UE measurement and evaluation of serving cell using MR, when LP-WUS UE is not in serving cell offloading mode as defined in [1].

4.X.2.3.1	Requirements for evaluation of cell selection criterion

When LP-WUS UE is not in relaxed measurement mode as defined in [1], the requirements in clause 4.2.2.2 shall apply.

When LP-WUS UE is in relaxed measurement mode as defined in [1], the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every N1*16 DRX cycle.

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either 8 DRX cycles or DRX cycle/2.

If the LP-WUS UE has evaluated according to table 4.X.2.3.1-1 in 16*Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the LP-WUS UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting LP-WUS UE measurement activities.

If the LP-WUS UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the LP-WUS UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where T= 10s.

Table 4.X.2.3.1-1: Nserv

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv [number of 16*DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 |  |
| 0.32 | 1 | 8 | N1*4 |
| 0.64 |  | 5 | N1*4 |
| 1.28 |  | 4 | N1*2 |
| 2.56 |  | 3 | N1*2 |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. |  |  |  |

4.X.2.3.2	Requirements for evaluation of LP-WUS related conditions

When LP-WUS UE is in relaxed measurement mode as defined in [1], the LP-WUS UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the following LP-WUR related conditions defined in TS 38.304 [1], if configured, for the serving cell at least once every N1*16 DRX cycle.

-	exit condition for relaxed measurement mode

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either 8 DRX cycles or DRX cycle/2.

If the LP-WUS UE has evaluated according to table 4.X.2.3.2-1 in 16*Nserv consecutive DRX cycles that the serving cell fulfils the exit condition for relaxed measurement mode, the LP-WUS UE shall perform corresponding actions as defined in clause 5.2.4.x in [1].

Table 4.X.2.3.2-1: Nserv when UE is in relaxed measurement mode

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv [number of 16*DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 |  |
| 0.32 | 1 | 8 | N1*4 |
| 0.64 |  | 5 | N1*4 |
| 1.28 |  | 4 | N1*2 |
| 2.56 |  | 3 | N1*2 |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. |  |  |  |

4.X.2.3A	Measurement and evaluation of serving cell by RedCap UE

The requirements in this clause apply for RedCap UE measurement and evaluation of serving cell using MR, when the RedCap UE is not in serving cell measurement offloading mode as defined in [1].

Requirements defined in clause 4.2B.1 shall apply.

4.X.2.3A.1	Requirements for evaluation of cell selection criterion for RedCap UE

Requirements in clause 4.x.2.3.1 shall apply, except that clause 4.2.2.2 is replaced with 4.2B.2.2.

4.X.2.3A.2	Requirements for evaluation of LP-WUS related conditions for RedCap UE

Requirements defined in clause 4.x.2.3.2 shall apply.

<End of Change 4>

<Start of Change 5>

4.X.2.4 Measurements of intra-frequency NR cells for UE with LP-WUR

For a UE supporting LP-WUR capability and not configured with eDRX_IDLE cycle, the requirements in clause 4.2.2.3 for FR1 and FR2-1 apply except for the requirements specified in this clause when the relaxed measurement criterion defined in 5.2.4.x.2 in TS 38.304 [1] is fulfilled.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [1] within KLPW x Tdetect,NR_Intra when that Treselection= 0, where KLPW = 16.

The UE shall measure SS-RSRP and SS-RSRQ at least every KLPW x Tmeasure,NR_Intra for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least KLPW x Tmeasure,NR_Intra/2.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS 38.304 [1] within KLPW x Tevaluate,NR_Intra when Treselection = 0, provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3 dB better ranked in FR1.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3 dB better ranked in FR1 if the current serving cell is among them.

Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra are specified in table 4.2.2.3-1 for FR1 and FR2-1.

4.X.2.4A	Measurements of intra-frequency NR cells for RedCap UE with LP-WUR

For a Redcap UE supporting LP-WUR capability and not configured with eDRX_IDLE cycle, the requirements in clause 4.2B.2.3 apply except for the requirements specified in this clause when the relaxed measurement criterion defined in 5.2.4.x.2 in TS 38.304 [1] is fulfilled.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [1] within KLPW x Tdetect,NR_Intra_RedCap when that Treselection= 0, where KLPW = 16.

The UE shall measure SS-RSRP and SS-RSRQ at least every KLPW x Tmeasure,NR_Intra_RedCap for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least KLPW x Tmeasure,NR_Intra_RedCap /2.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS 38.304 [1] within KLPW x Tevaluate,NR_Intra_RedCap when Treselection = 0, provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3 dB better ranked in FR1 or 4.5 dB better ranked in FR2 for 2 Rx RedCap.

-	the cell is at least 4 dB better ranked in FR1 for 1 Rx RedCap.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3 dB better ranked in FR1 or 4.5 dB better ranked in FR2 if the current serving cell is among them for 2 Rx RedCap.

-	the cell is at least 4 dB better ranked in FR1 if the current serving cell is among them for 1 Rx RedCap.

Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are specified in table 4.2B.2.3-1.

<End of Change 5>

<Start of Change 6>

4.X.2.5	Measurements of inter-frequency NR cells for UE with LP-WUR

4.X.2.5.1	Introduction

This clause contains the requirements for measurements on inter-frequency NR cells for UE with LP-WUR when the UE is configured with any of following measurement criteria:

-	Relaxed measurement criterion for UE with LP-WUR defined in TS 38.304 [1],

-	Serving cell measurement offloading criterion for UE with LP-WUR defined in TS 38.304 [1],

4.X.2.5.2	Measurements for UE with LP-WUR fulfilling relaxed measurement criterion

The requirements for measurements on inter-frequency NR cells specified in this clause apply provided that:

-	UE is configured with relaxed measurement [2] criterion and UE has fulfilled the relaxed measurement [2] criterion.

The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

The requirements defined in clause 4.2.2.4 apply for this clause except that:

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then:

-    When T331 is running, for a UE not configured with eDRX_IDLE, Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for NR inter-frequency layers configure and not configured for idle mode CA/DC measurements specified in Table 4.2.2.4-1.

-	When T331 is not running, for a UE not configured with eDRX_IDLE, Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax for NR inter-frequency layers not configured for idle mode CA/DC measurements and NR inter-frequency layers configured for idle mode CA/DC measurements and mobility measurement are as specified in table 4.X.2.5.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

-    When T331 is running, the UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements and inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7.

-	When T331 is not running, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60

Table 4.X.2.5.2-1: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x KLPW x 1.5 (36 x N1 x KLPW x 1.5) | 1.28 x N1 x KLPW 1.5 (4 x N1 x KLPW x 1.5) | 5.12 x N1 x KLPW 1.5 (16 x N1 x KLPW x 1.5) |
| 0.64 |  | 5 | 17.92x N1 x KLPW (28 x N1 x KLPW) | 1.28 x N1 x KLPW (2 x N1 x KLPW) | 5.12 x N1 x KLPW (8 x N1 x KLPW) |
| 1.28 |  | 4 | 32 x N1 x KLPW (25 x N1 x KLPW) | 1.28 x N1 x KLPW (1 x N1 x KLPW) | 6.4 x N1 x KLPW (5 x N1 x KLPW) |
| 2.56 |  | 3 | 58.88 x N1 x KLPW (23 x N1 x KLPW) | 2.56 x N1 x KLPW (1 x N1 x KLPW) | 7.68 x N1 x KLPW (3 x N1 x KLPW) |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: KLPW = 16 is the measurement relaxation factor applicable for UE with LP-WUR fulfilling relaxed measurement criterion. |  |  |  |  |  |

4.X.2.5.3	Measurements for UE with LP-WUR fulfilling serving cell measurement offloading criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	Entry conditions for serving cell measurement offloading for UE with LP-WUR defined in clause 5.2.4.x.4 in 38.304 [1] are fulfilled.

-	UE is configured with Rel-19 LP-WUR and UE has fulfilled serving cell measurement offloading criterion.

The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

The requirements defined in clause 4.2.2.4 apply for this clause except that:

-	When T331 is not running, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60

-	When T331 is running, the UE shall search for inter-frequency layers of higher priority and inter-frequency NR layers configured for idle mode DC measurements at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

4.X.2.5A	Measurements of inter-frequency NR cells for Redcap with LP-WUR

4.X.2.5A.1	Introduction

This clause contains the requirements for measurements on inter-frequency NR cells for Redcap with LP-WUR when the UE is configured with any of following relaxed measurement criteria:

-	Relaxed measurement criterion for UE with LP-WUR defined in clause 5.2.4.x.2 in [1],

-	Serving cell measurement offloading criterion for UE with LP-WUR defined in clause 5.2.4.x.4 in [1],

When the relaxed measurement criterion is not met, or the serving cell measurement offloading criterion is not met or both the relaxed measurement and the serving cell measurement offloading criteria are not met, the requirements in clause 4.2B.2.4 apply for UE.

4.X.2.5A.2	Measurements for UE with LP-WUR fulfilling relaxed measurement criterion

The requirements for measurements on inter-frequency NR cells specified in this clause apply provided that:

-	UE is configured with relaxed measurement [2] criterion and UE has fulfilled the relaxed measurement [2] criterion.

The requirements defined in clause 4.2B.2.4 apply for this clause except that:

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	For a UE not configured with eDRX_IDLE, Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax are as specified in table 4.X.2.5.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	The UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7 and, K2 = 60

4.X.2.5A.3	Measurements for UE with LP-WUR fulfilling serving cell measurement offloading criterion

The requirements for measurements on inter-frequency NR cells specified in this clause apply provided that:

-	UE is configured with serving cell measurement offloading [1] criterion and UE has fulfilled the serving cell measurement offloading [1] criterion.

The requirements defined in clause 4.2B.2.4 apply for this clause except that:

-	The UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7 and, K2 = 60

<End of Change 6>

<Start of Change 7>

4.X.2.6     Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR

4.X.2.6.1	Introduction

This clause specifies the requirements for measurements on inter-RAT E-UTRAN cells performed by MR when the UE with LP-WUR is configured with any of the following relaxed measurement criteria:

-	Relaxed measurement criteria for UE with LP-WUR defined in clause 5.2.4.x.2 in TS 38.304 [1],

-	Serving cell measurement offloading criteria for UE with LP-WUR defined in clause 5.2.4.x.4 in TS 38.304 [1].

4.X.2.6.2	Measurements for UE fulfilling relaxed measurement criteria

The requirements for measurements on inter-RAT E-UTRAN cells specified in this clause apply provided that:

-	Relaxed measurement criteria for UE with LP-WUR defined in clause 5.2.4.x.2 in TS 38.304 [1] are fulfilled.

The UE shall not relax measurements on inter-RAT E-UTRAN carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

The requirements defined in clause 4.2.2.5 apply for this clause except that:

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, then

-	When T331 is running, for a UE not configured with eDRX_IDLE, Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax and Tevaluate,EUTRAN_Relax for inter-RAT E-UTRAN carriers configure and not configured for idle mode CA/DC measurements are specified in Table 4.2.2.5-1.

-	When T331 is not running, for a UE not configured with eDRX_IDLE, Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax and Tevaluate,EUTRAN_Relax for inter-RAT E-UTRAN layers not configured for idle mode CA/DC measurements and inter-RAT E-UTRAN configured for idle mode CA/DC measurements and mobility measurement are as specified in table 4.X.2.6.2-1.

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the requirements are defined as follows:

-	When T331 is running, the UE shall search for inter-RAT E-UTRAN configured for idle mode CA/DC measurements and inter-RAT E-UTRAN frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7.

-	When T331 is not running, the UE shall search for inter-RAT E-UTRAN frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7 and, K2 = 60.

Table 4.X.2.6.2-1: Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax, and Tevaluate,EUTRAN_Relax for UE with LP-WUR

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x KLPW (36 x KLPW) | 1.28 x KLPW (4 x KLPW) | 5.12 x KLPW (16 x KLPW) |
| 0.64 | 17.92 x KLPW (28 x KLPW) | 1.28 x KLPW (2 x KLPW) | 5.12 x KLPW (8 x KLPW) |
| 1.28 | 32 x KLPW (25 x KLPW) | 1.28 x KLPW (1 x KLPW) | 6.4 x KLPW (5 x KLPW) |
| 2.56 | 58.88 x KLPW (23 x KLPW) | 2.56 x KLPW (1 x KLPW) | 7.68 x KLPW (3 x KLPW) |
| NOTE 1: KLPW = 16 is the measurement relaxation factor applicable for UE with LP-WUR fulfilling relaxed measurement criterion. |  |  |  |

4.X.2.6.3	Measurements for UE fulfilling serving cell measurement offloading entry criteria

The requirements for measurements on inter-RAT E-UTRAN cells specified in this clause apply provided that:

-	Entry conditions for serving cell measurement offloading for UE with LP-WUR defined in clause 5.2.4.x.4 in TS 38.304 [1] are fulfilled.

The UE shall not relax measurements on inter-RAT E-UTRAN carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

The requirements defined in clause 4.2.2.5 apply for this clause except that:

-  When T331 is not running, the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7 and, K2 = 60.

-	When T331 is running, the UE shall search for inter-RAT E-UTRAN layers of higher priority and inter-RAT E-UTRAN layers configured for idle mode DC measurements at least every Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7.

Note: It is assumed that Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met when the entry conditions for serving cell measurement offloading are met.

4.X.2.6A   Measurements of inter-RAT E-UTRAN cells for RedCap with LP-WUR

4.X.2.6A.1	Introduction

This clause contains the requirements for measurements on inter-RAT E-UTRAN cells performed by RedCap UE when the UE with LP-WUR is configured with any of the following relaxed measurement criteria:

-	Relaxed measurement criteria for UE with LP-WUR defined in clause 5.2.4.x.2 in TS 38.304 [1],

-	Serving cell measurement offloading criteria for UE with LP-WUR defined in clause 5.2.4.x.4 in TS 38.304 [1].

4.X.2.6A.2	Measurements for UE fulfilling relaxed measurement criteria

The requirements for measurements on inter-RAT E-UTRAN cells specified in this clause apply provided that:

-	Relaxed measurement criteria for UE with LP-WUR defined in clause 5.2.4.x.2 in TS 38.304 [1] are fulfilled:

The requirements defined in clause 4.2B.2.5 apply for this clause except:

-  When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	For a UE not configured with eDRX_IDLE, Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax and Tevaluate,EUTRAN_Relax are specified in Table 4.2B.2.5-1.

-  When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the requirements are defined as follows

-	The UE shall search for inter-RAT E-UTRAN frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7 and, K2 = 60.

4.X.2.6A.3	Measurements for UE fulfilling serving cell measurement offloading entry criteria

The requirements for measurements on inter-RAT E-UTRAN cells specified in this clause apply provided that:

-	Entry conditions for serving cell measurement offloading for UE with LP-WUR defined in clause 5.2.4.x.4 in TS 38.304 [1] are fulfilled.

The requirements defined in clause 4.2B.2.5 apply for this clause except that:

-  	The UE shall search for inter-RAT E-UTRAN layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is specified in clause 4.2.2.7 and, K2 = 60.

Note: It is assumed that Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met when the entry conditions for serving cell measurement offloading are met.

<End of Change 7>

<Start of Change 8>

5.x	INACTIVE mode measurement for LP-WUS operation

5.x.1	Introduction

The UE which supports LP-WUS capability in RRC_IDLE shall be capable of:

- performing serving cell measurement based on LP-SS or SSB by LR.

- performing serving cell and neighbour cell measurement relaxation by MR together with LP-SS or SSB based serving cell measurement by LR.

5.X.2 Requirements

5.X.2.1 UE measurement capability

5.X.2.1.1 LP-WUR measurement capability

The requirements in clause 4.x.2.1.1 could be applied.

5.X.2.1.2 MR measurement capability with LP-WUR

The requirements in clause 4.x.2.1.2 could be applied.

<End of Change 8>

<Start of Change 9>

5.x.2.2	LP-WUR serving cell measurement and evaluation requirements

The requirements in clause 4.x.2.2 shall apply.

<End of Change 9>

<Start of Change 10>

5.X.2.3	Measurement and evaluation of serving cell by MR

The requirements in clause 4.X.2.3 shall apply.

5.X.2.3A		Measurement and evaluation of serving cell by Redcap

The requirements in clause 4.X.2.3A shall apply.

<End of Change 10>

<Start of Change 11>

5.X.2.4	Measurements of intra-frequency NR cells for UE with LP-WUR

The requirements in clause 4.X.2.4 shall apply.

5.X.2.4A	Measurements of intra-frequency NR cells for RedCap UE with LP-WUR

The requirements in clause 4.XA.2.4 shall apply.

<End of Change 11>

<Start of Change 12>

5.X.2.5	Measurements of inter-frequency NR cells for UE with LP-WUR

The requirements in clause 4.X.2.5 shall apply when UE with LP-WUR is not configured with eDRX_IDLE and has fulfilled the relaxed measurement criterion.

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier in clause 4.X.2.5 shall apply regardless of whether the serving cell is subject to CCA or not.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, Kcarrier is replaced with Kcarrier + 1, regardless of whether the serving cell is subject to CCA or not.

5.X.2.5A	Measurements of inter-frequency NR cells for Redcap with LP-WUR

The requirements in clause 4.X.2.5A shall apply when UE with LP-WUR is not configured with eDRX_IDLE and has fulfilled the relaxed measurement criterion.

5.X.2.6	Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR

The requirements in clause 4.X.2.6 shall apply when UE with LP-WUR is not configured with eDRX_IDLE and has fulfilled the relaxed measurement criterion.

5.X.2.6A	Measurements of inter-RAT E-UTRAN cells for Redcap with LP-WUR

The requirements in clause 4.X.2.6A shall apply when UE with LP-WUR is not configured with eDRX_IDLE and has fulfilled the relaxed measurement criterion.

<End of Change 12>