3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513026

Prague, Czech Republic, 13 Oct 2025 - 17 Oct 2025

Source:	Charter Communications

Title:	Improving 6G UL throughput performance

Agenda item:	8.3 [FS_6G_Radio] System Parameters

Document for:	Agreement

Introduction

In this contribution, we propose to study UL throughput performance for specific devices, i.e., Fixed Wireless Access (FWA) in Rel-20 and continue normalization work in Rel-21.

With the new technology of smart receivers (ML) the gNb is expected to receive 1024-QAM throughout most of the cell-site coverage especially for the use case of small-cells, where the propagation loss is minimal

Using AI/ML aided beamforming and CLI mitigation, FWA can expect to operate in good channel conditions and hence can use higher QAM order than 256 in the UL channel.

Discussion

Plenary has set a goal of 3X increase in 6G throughput during their Sep 2025 meeting [1]

RAN1 has started their 6G study of higher modulation. We expect RAN1 to ask RAN4 to drive the RF performance impacts in the study of UL modulation index. See [2], [3].

2.1. Benefits and Motivation:

Enhanced Spectral Efficiency: 1024-QAM encodes 10 bits per symbol, significantly increasing the data rate per Hertz compared to 256-QAM (8 bits/symbol), directly translating to higher throughput and capacity in the uplink.

Meeting 6G Requirements: As mobile communication evolves, there's a continuous need for higher data rates, especially for bandwidth-intensive uplink applications like real-time streaming, high-resolution video uploads, and emerging AR/VR content creation from user devices.

Support for Specific Use Cases: Higher modulation orders are particularly beneficial in scenarios with excellent channel conditions, such as small cells with controlled environments, and potentially in dense urban areas with good line-of-sight conditions and high Signal-to-Noise Ratio (SNR).

Proposal 1: Study the feasibility of higher-order modulation schemes such as 1024 QAM in the UL direction in scenarios when good SNR conditions are available such as in FWA.

2.2. Addressing Technical Challenges:

Increased Sensitivity to Noise and Interference: Higher-order QAM constellations are more susceptible to noise, interference, and channel impairments due to the closer proximity of constellation points.

Error Vector Magnitude (EVM) can limit the use of 1024-QAM, and will require higher linearity and lower noise in both the transmitter and the receiver.

Justification: The feasibility relies on advanced receiver designs at the base station with superior Error Vector Magnitude (EVM) performance and sophisticated interference cancellation techniques.

Higher Power Requirements for User Equipment (UE): Achieving the necessary SNR for 1024-QAM in the uplink requires UEs to transmit with higher power or operate under very favorable channel conditions, which impacts battery life and regulatory limits.

Justification: This can be mitigated through link adaptation, where 1024-QAM is dynamically used only when channel conditions are optimal, and fallback to lower modulation-order otherwise. Beamforming and massive MIMO also help improve the uplink link budget by focusing energy on the UE.  In the case of FWA, UE is a CPE which may be powered via a wall outlet, so impact on battery life may not be applicable.

RF Front-End Linearity Challenges: Implementing 1024-QAM requires highly linear power amplifiers and RF components in the UE to minimize signal distortion and maintain constellation integrity.

Justification: Continued advancements in RF hardware technology, including Silicon Carbide (SiC) and Gallium Nitride (GaN) based power amplifiers, and advanced linearization techniques (e.g., Digital Pre-Distortion – DPD and Non-Uniform Constellation - NUC) can address these challenges.

Complexity in UE Implementation: The signal processing complexity for 1024-QAM, particularly for channel estimation and equalization, places higher demands on UE chipsets.

Justification: Moore's Law and continued innovations in chipset design and signal processing algorithms will enable the necessary computational capabilities in future UEs without excessive power consumption or cost penalties. Also, EVM improvements are expected at the same time, (hence smart receivers)

Observation: 3GPP should study UE UL technology such as AI/ML, SiC and GaN. Specially for RAN4, start a study of UE techniques such as DPD and NUC to enable UL 1024-QAM and study of new chip-set and digital processing technologies to support the complexity of UL 1024 QAM.

2.3. Enabling Technologies and Advancements:

Advanced Channel Estimation and Equalization: Sophisticated algorithms to accurately estimate channel conditions and compensate for EVM impairments are crucial for robust 1024-QAM performance.

Link Adaptation and Scheduling: Intelligent network algorithms that dynamically select the optimal modulation and coding scheme (MCS) based on real-time channel conditions and UE capabilities ensure efficient and reliable operation.

Massive MIMO and Beamforming: These techniques significantly enhance the uplink signal strength and improve the SNR at the base station receiver, making higher-order modulation feasible even for UEs with limited transmit power.

Advanced Channel Coding: Stronger Forward Error Correction (FEC) codes can compensate for the reduced robustness of 1024-QAM in challenging channel conditions, ensuring data integrity.

By leveraging these advancements and implementing 1024-QAM in a controlled and adaptive manner for specific scenarios, its introduction in Release 20 can be justified as a critical step in maximizing spectral efficiency and preparing for the demands of future mobile communication generations.

Proposal 2: Study of gNb UL techniques such as Advanced Channel Estimation (ACE), and optimal modulation and coding scheme. Also study how MIMO and Beamforming with improved FEC can handle the complexity of UL 1024-QAM

Conclusion

The following Observations and Proposals were made:

Proposal 1: Study the feasibility of higher-order modulation schemes such as 1024 QAM in the UL direction in scenarios when good SNR conditions are available such as in FWA.

Observation: 3GPP should study UE UL technology such as AI/ML, SiC and GaN. Specially for RAN4, start a study of UE techniques such as DPD and NUC to enable UL 1024-QAM and study of new chip-set and digital processing technologies to support the complexity of UL 1024 QAM.

Proposal 2: Study of gNb UL techniques such as Advanced Channel Estimation (ACE), and optimal modulation and coding scheme. Also study how MIMO and Beamforming with improved FEC can handle the complexity of UL 1024 QAM

- References

- RP-252878 Sep plenary on technical performance for IMT-2030 radio interface(s)

- RP-251134 FWA

- R1-2506362 1024 QAM on FWA