3GPP TSG-RAN WG4 Meeting #116-bis														R4-2513043

Prague, Czech, 13th – 17th October, 2025

Agenda item:			8.3

Source:	Samsung

Title:	Discussion on system parameters for 6GR

Document for:	Approval

# 1	Introduction

In RAN#108, a SI for a new journey on “6G RAN” was approved, and updated in RAN#109 [1]. Based on the SID, a single technology framework based on a standalone architecture needs to be developed across WGs in order to meet the requirement of next generation cellular communications. RAN4 as one of the important WGs for enabling 6G also discussed the plan and arrangements to clarify the scope of each 6G topic concerned with RAN4 in [2]. This contribution provides a summary of Samsung’s views on the general design principles and directions for the system parameter requirements of 6G RAN.

# 2	Discussion

## 2.1	Waveform and modulation

6G radio (6GR) first needs to consider advanced waveform and modulation techniques to enhance spectral efficiency, energy efficiency, and performance. The waveform has been using CP-OFDM for DL and UL, and DFT-s-OFDM is used supplementary for layer-1 transmission in UL, with frequency-domain spectrum shaping (FDSS) applied to reduce peak-to-average power ratio (PAPR) by 2 - 3 dB, which has the similar amount of impact on MPR/A-MPR requirements in UE RF specifications of 5G NR [3]. This improvement has enabled more efficient power usage and better coverage for UL transmissions. For 6GR, in terms of co-existence with NR via spectrum sharing, the use of OFDM based waveforms which could utilize only the time and frequency resource grid for channel estimation and resource allocation is a must. Therefore, the first agreement of 6GR was achieved to support OFDM-based waveforms such as CP-OFDM (downlink and uplink) and DFT-s-OFDM (uplink) waveform as the basis for 6GR [4].

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Observation 1:	In RAN1#122, the first agreement of 6GR was achieved to support OFDM-based waveforms such as CP-OFDM (downlink and uplink) and DFT-s-OFDM (uplink) waveform as the basis for 6GR.

For modulation, non-uniform constellations designed with AI assistance to overcome the shaping loss inherent in traditional square QAM formats can offer substantial benefits. Those constellations optimize symbol distribution, achieving up to 1.53 dB gain in spectral efficiency compared to uniform designs. This allows higher-order modulations to be more viable for high-data-rate applications, especially when low latency is also a consideration and retransmissions may be detrimental (e.g., XR). For upper limits of modulation order, we expect high-order modulation as similar level of 1K-QAM for DL and 256-QAM for UL with the conventional formats. Together with or without an AI-assisted receiver, study on the level of 1K-QAM for UL or 4K-QAM for DL can be considered in 6GR. As basis for study, RAN1 made following initial agreements on the modulation as below [4].

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Observation 2:	For upper limits of modulation order, the high-order modulation is expected as similar level of 1K-QAM for DL and 256-QAM for UL with the conventional formats as agreed in RAN1 for the study.

Observation 3:	Together with or without an AI-assisted receiver, study on the level of 1K-QAM for UL or 4K-QAM for DL can be considered in 6GR.

From RAN4’s perspective, based on the initial agreements in RAN1, it would be important to carry out the performance assessments by coordinating closely with RAN1 by the interim study results in June 2026. The study outcome should also consider the impact of implementation feasibility, and performance benefits compared to legacy requirements.

Proposal 1:		Based on the initial agreements in RAN1, it would be important for RAN4 to carry out the performance assessments by coordinating closely with RAN1. The study outcome should also consider the impact of implementation feasibility, and performance benefits compared to legacy requirements.

## 2.2	Numerology, FFT size and channel bandwidth

In 5G NR, spectrum was divided into two ranges: FR1 (below 7.125 GHz) and FR2 (24.25–71 GHz). With the evolution toward 6G, broader spectrum utilization is expected, and one possibility is to extend FR1 to 8.4 GHz, which is one of the candidate frequencies ranges of IMT for WRC-27, i.e., 7.125-8.4 GHz. This ensures that emerging 7–8 GHz bands can be treated within the FR1 framework, benefiting from similar propagation and deployment characteristics as the legacy sub-7 GHz spectrum according to the previous studies including a SI for IMT of RAN4, FS_NR_IMT_4400_7125_14800MHz.

Observation 4:	Emerging 7–8 GHz bands can be treated within the FR1 framework, benefiting from similar propagation and deployment characteristics as the legacy sub-7 GHz spectrum according to the previous studies including a SI for IMT of RAN4.

For new spectrum for 6GR including 15 GHz, three options can be considered:

- Option A: Extend FR1 to 8.4 GHz and define a separate FR for the mid-high band (8.4–24.25 GHz) with its own single numerology.

- Option B: Extend FR1 to 15.35 GHz and merge 15.35–24.25 GHz into FR2 family.

- Option C: Define a separate FR for 7.125 – 24.25 GHz.

In either case, spectrum at or above 7.125 GHz should adopt a single SCS, suitable for wider bandwidths and the phase noise environment of higher frequencies.

Proposal 2: 		Among the three options, our recommendation for FR1 is to extend up to 8.4 GHz (option A or option B) at least, considering the channel propagation characteristics and previous studies of IMT SI.

Looking back on the SID [1], it emphasizes the design principle of numerology “Channel Bandwidth (at least minimum and maximum), numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]”. To optimize spectral efficiency and flexibility in 6GR systems while simultaneously pursuing simplification, a tiered approach to subcarrier spacing (SCS) is proposed for the study in both RAN1 and RAN4. The SCS set defined in NR, Δf = 2μ× 15 kHz, where µ ranges from 0 to 3 within a given frequency band (FR1 and FR2-1), provides four distinct SCS options for data – 15 kHz, 30 kHz, 60 kHz, and 120 kHz. However, a long-term design goal should be to strive for the adoption of a single, optimal SCS per band whenever technically feasible. This would further reduce receiver complexity and enable more efficient intra-band inter-cell interference mitigation strategies. For further simplification, the numerology set for a given band is also applicable not only for the data/control SCS but also for the synchronization signal SCS. This allows reducing unnecessary device implementation and testing complexity. For instance, 15 kHz can be selected for lower bands (< 3 GHz), 30 kHz for mid-band and up to 8.4 GHz, and 60 / 120 kHz for above 8.4 GHz. Therefore, to simplify implementation and improve spectral efficiency, we propose a single-SCS per range principle:

- FR1-FDD (≤ 8.4 GHz): 15 kHz

- FR1-TDD (≤ 8.4 GHz): 30 kHz

- 8.4 – 15.35 GHz: 60 kHz

- 15.35 – 24.25 GHz (if defined): 120 kHz

This approach avoids multi-numerology coexistence within a band, thereby reducing guard overhead and complexity. In particular, FR1-TDD = 30 kHz offers clear benefits in spectrum-sharing scenarios such as MRSS, where alignment across operators and networks is essential.

Proposal 3:		To simplify implementation and improve spectral efficiency, we propose a single-SCS per range principle as below.

- FR1-FDD (≤ 8.4 GHz): 15 kHz

- FR1-TDD (≤ 8.4 GHz): 30 kHz

- 8.4 – 15.35 GHz: 60 kHz

- 15.35 – 24.25 GHz (if defined): 120 kHz

In addition, to accommodate the increasingly wide bandwidths anticipated in 6GR deployments, an 8K Fast Fourier Transform (FFT) size is now being considered as a baseline configuration in RAN1. This enables effective processing of channels up to 200 MHz and beyond, allowing for greater spectral efficiency and increased capacity. While flexible FFT sizes may be explored for specific applications which requires more granular bandwidth allocation, the 8K FFT provides a robust foundation for supporting high-throughput communication over a significant portion of the available spectrum. Especially, the new band, e.g., 7 – 15 GHz, should reflect industry demands where at least 200 MHz channel bandwidth needs to be secured. 30 kHz SCS and 8K FFT would be a possible way for realizing the new band requirements. Based on the principle above, it is proposed that a baseline maximum channel bandwidth of 200 MHz across ranges, with a possibility to allow up to 400 MHz in specific bands where contiguous spectrum is available. The corresponding FFT sizes are selected to be powers of two, aligned with sampling rates (up to 491.52 Mbps).

- FR1-FDD (15 kHz): Optimized for wide coverage and legacy continuity; suitable for IoT and control traffic with stringent link budget requirements.

- FR1-TDD (30 kHz): Balances coverage and efficiency, minimizes guard overhead in shared spectrum (MRSS), and enables wideband deployments with practical FFT sizes.

- Above 8.4 GHz (60 or 120 kHz): Appropriate for ≥ 8.4 GHz, providing resilience against phase noise and Doppler while keeping FFT sizes moderate for 200–400 MHz CBW.

Observation 5:	8K FFT size is now being considered as a baseline configuration in RAN1 to enable effective processing of channels up to 200 MHz and beyond, allowing for greater spectral efficiency and increased capacity.

Maintaining consistent numerology, including SCS and FFT size, between NR and 6GR networks is crucial for ensuring seamless mobility and MRSS-based interoperability. A unified numerological framework simplifies handover procedures, minimizes signal degradation during transitions between network generations, and enables a smoother user experience. This approach reduces the complexity of network management and allows devices to operate efficiently across both NR and 6GR infrastructure without requiring extensive reconfiguration or compatibility adjustments. More details can be found in Table 1.

Proposal 4:		Table 1 is proposed as a unified numerological framework that simplifies and reduces the complexity of network management and allows devices to operate efficiently across both NR and 6GR.

Table 1: Potential SCS, FFT size, and CBW per frequency range

| FR | Range | SCS (kHz) | Required FFT size | Max CBW (MHz) | Min CBW (MHz) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 400 MHz – 6.425 GHz | 15 (FDD) 30 (TDD) | 8192 | 100 (FDD) 200 (TDD) | 5 (FDD) 10 (TDD) |
|  | 6.425 – 7.125 GHz | 30 | 8192, [16384] | 200, [400] | 20 |
| FR1-extended | 7.125 – 8.4 GHz | 30 | 8192, [16384] | 200, [400] | 20 |
| [new FR] | 8.4 – 15.35 GHz | 60 | 8192 | 400 | 20 |
| [new FR] | 15.35 – 24.25 GHz | 120 (if defined) | 4096, 8192 | 400, 800 | 50 |
| FR2-1 | 24.25 – 52.6 GHz | 120 | 4096, 8192 | 400, 800 | 50 |

## 2.3	Spectrum utilization and irregular channel bandwidth

Spectrum utilization, as a target requirement for the efficient use of spectrum (%), should have a close relationship between channel bandwidth, the guard band and the maximum transmission bandwidth configuration. The spectrum utilization governs the proportion of channel bandwidth available for data transmission, with standardized rules for resource block (RB) allocation and guard band management. Under this framework, a single set of utilization values was established for both uplink and downlink, ensuring baseline performance across diverse deployments as shown in Figure 1 [3].

Figure 1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration [3]

- Maximum transmission bandwidth: Specified not in MHz but by the maximum number of allocatable RBs for a given channel bandwidth and SCS

- Minimum guard bands: Unused frequency regions at the channel edges designed to mitigate Adjacent Channel Interference (ACI).

As mentioned earlier, given that the fundamental physics of OFDM waveforms remain unchanged for 6G, we therefore recommend that 6G can adopt and evolve 5G NR design philosophy as much as possible, which is to retain the principle of a unified spectral emission mask, defined by a maximum RBs and minimum guardbands. This would provide a stable, predictable foundation for the 6G air interface and leverages a well-understood operational model.

Proposal 5: 6G can adopt and evolve 5G NR design philosophy as much as possible, which is to retain the principle of a unified spectral emission mask, defined by a maximum RBs and minimum guardbands.

Building upon this foundation, as a forward-looking evolution, 6G should explore the concept of irregular or dynamic channel bandwidths. While building upon the 5G framework of irregular channel bandwidth provides an immediate efficiency gain, a truly dynamic approach would enable the network to aggregate fragmented spectrum resources in real-time. It should be also considered that asymmetric bands or channel bandwidths to support flexible utilization of spectrum resources for DL and UL over different carriers/bands, can help asymmetric traffic between DL and UL. By allowing for different UL and DL bandwidths, 6GR can allocate spectrum far more effectively. Meanwhile, the core principles of the spectral mask would still apply independently to each link, ensuring spectral integrity is maintained.

Proposal 6: As a forward-looking evolution, 6GR should explore the concept of irregular channel bandwidths based on 5G framework, and asymmetric band or bandwidths for the extreme spectral efficiency.

## 2.4	Synchronization signal and raster

Synchronization signals for initial access procedures are critical for cellular system and represent a substantial engineering investment with limited opportunities for subsequent modification. Looking back to 5G NR, it first introduced synchronization channel raster (sync raster) to identify the set of possible frequency location of the SS Block, consisting the synchronization channels PSS/SSS and the PBCH. Since the SS block is not in a fixed position within the configured bandwidth of the RF carrier, but can be placed anywhere on the RF carrier, the sync channel raster could be “sparser” than the legacy RF channel raster, which can have an advantage of a reduced search time for the initial access. For 5G sync raster specified in [3], it has been defined by operating band since the SS block has used a predetermined SCS and minimum channel BW per band.

However, this engineering choice led to unintended complexity in 5G. The pursuit of SSB placement flexibility introduced superfluous raster points, increasing UE processing overhead and extending initial cell search duration in wideband scenarios. Furthermore, the default 20ms SSB periodicity constrained network operators’ ability to optimize energy efficiency dynamically.

Observation 6:	The pursuit of SSB placement flexibility of 5G introduced superfluous raster points, increasing UE processing overhead and extending initial cell search duration in wideband scenarios.

Observation 7:	Furthermore, the default 20ms SSB periodicity of 5G constrained network operators’ ability to optimize energy efficiency dynamically.

To overcome these limitations, 6GR should discuss more fundamental shifts in synchronization design. Chief among them is extending the SSB periodicity to 40ms, 80ms, or even 160ms, which could substantially lower network energy consumption. Such changes, however, must not degrade initial cell search performance in terms of the number of sync raster points and the SSB periodicity must remain comparable to 5G levels based on following principle to optimize 6G sync raster design.

- Limited step sizes (e.g., 120kHz for FR1, 480kHz for FR2) to reduce grid density while maintaining CBW coverage

- Adaptive SSB periodicities (e.g., 40ms for coverage areas, 80ms for high-capacity hotspots) to maximizes energy efficiency gains

Although the 6G sync rater discussion still requires a progress of SSB and initial access discussion in RAN1, the proposed principle of 6G sync raster design could be agreed ahead in RAN4 based on the previous experience of 5G NR.

Proposal 7:	It is recommended for RAN4 to have a fundamental principle for 6G sync raster design considering energy efficiency/saving as well as design complexity and flexibility as follows.

- Limited step sizes (e.g., 120kHz for FR1, 480kHz for FR2) to reduce grid density while maintaining CBW coverage

- Adaptive SSB periodicities (e.g., 40ms for coverage areas, 80ms for high-capacity hotspots) to maximizes energy efficiency gains

## 2.5	#Rx and #Tx

Antenna architectures have been a key role as an assumption for UE RF requirements in 5G. For 6GR UE, it will highlight several critical challenges and considerations. First, as RAN4 had discussed in their 6Rx WI in Rel-19, the current form factors of UEs result in significant antenna correlations due to limited space even in the case of foldable devices. For instance, in n41 band where the half-wavelength is approximately 6 cm, placing six antennas leads to combinations with correlations exceeding 0.3 in 25% of antenna pairs, and in some instances, correlations as high as 0.5. This means that adding two additional antennas yields an improvement of less than 1 dB, which is far from ideal considering the additional cost, complexity, and power consumption. Second, even at higher frequencies such as 7 GHz where the wavelength decreases, the half-wavelength remains around 2 cm. In this scenario, the difference in performance between 4Rx and 6Rx configurations is marginal, just 0.6 dB, and indicating diminishing returns with increased antenna counts. Third, UEs often rely on common antennas for multiple frequency bands. This introduces more complication as the device switches between band and band combinations, which makes it nearly impossible to achieve the theoretical 9 dB gain expected from a 6Rx configuration, as the practical implementation is constrained by design limitations and operational complexity.

Observation 8:	Increasing the number of UE antennas does not guarantee the performance improvement according to the previous discussion in RAN4.

While increasing the number of antennas may seem beneficial in theory, real-world constraints such as form factor, antenna spacing, RF impairments, and multi-band operation significantly limit the effectiveness. These factors underscore the need for innovative approaches in 6GR terminal design to overcome the challenges and maximize performance gains. Thus, at least for FR1, there is no strong motivation to enhance over beyond 1T/4R for handheld devices. To better understand the feasibility of RF architecture for the new band beyond FR1, we should consider practical constraints including the number of Tx and the number of Rx.

Proposal 8:		Consider the 1T/4R architecture from NR for FR1 at the UE, and study the feasibility of sNB/UE architectures such are 6Rx only for FR3.

## 2.6	Device types

The evolution of mobile communication systems with each generation has led to a rapid diversification of services and device types. In particular, in LTE, the concept of "UE category" was introduced to simplify the identification of terminal performance. This categorization was based on key performance indicators such as maximum transmission speed, supported antenna count, and receive buffer size, allowing networks to allocate resources and configure scheduling policies efficiently using only the UE category number.

However, with the commercialization of NR from Rel-15, the performance and functionality of terminals became significantly more detailed and diverse compared to LTE. As a result, the simplistic category structure, which summarized performance with a single number, was phased out. Instead, a detailed reporting method based on UE capability Information Elements (IEs) was adopted. The current NR UE capability structure includes numerous detailed items such as supported frequency bands, bandwidth, antenna configurations, power classes, and beam management capabilities. Networks can determine UE capabilities through the UE Capability Enquiry/Information procedure.

Despite these advancements, the existing NR UE capability structure lacks explicit fields to distinguish physical form factors or physical criteria of UEs. For instance, devices like handheld smartphones, Fixed Wireless Access CPEs (Customer Premises Equipment), wearables, drones (UAVs), and industrial IoT devices differ significantly in form and usage environments, yet these distinctions are not clearly reflected in the current capability reporting structure although some performance requirements have been specified by a specific UE type based on the clear target in some WIDs. This limitation leads to several challenges:

- Constraints in network optimization: Even if networks analyse the numerous capability items reported by UEs, it is difficult to determine the physical form or usage environment of the device. This makes it challenging to apply terminal-type-specific optimizations in areas such as beam management, scheduling, and power control.

- Unnecessary functionality: UE vendors may implement features that are not suitable for specific UE types, leading to increased hardware and software complexity and manufacturing costs. For example, a Fixed Wireless Access CPE may still need to perform mobility measurements, resulting in inefficiencies.

- Inefficient testing and certification processes: In RAN4 specifications, requirements for specific UE types are often defined in separate Notes. However, since UEs do not explicitly report their type, testing agencies must manually select the appropriate requirement profile, making the testing and certification process inefficient.

Post-Rel-15, WIs increasingly incorporate requirements tied to form factors.

Looking ahead to the 6G era, it is expected that a wide range of devices with extreme differences in physical form and operational environments will emerge. In such a scenario, it will be essential to not only report UE capability information but also explicitly identify and report UE types from the outset. This will enable the application of type-specific capabilities and requirements, ensuring efficient network operation and resource utilization.

Proposal 9:		Introduce a modular and MAC-layer-assisted UE capability and assistance information reporting framework in 6GR, enabling category-based network optimization while minimizing per-UE management overhead.

In order for that, it would be first important to classify device types according to concrete product groups expected in commercial deployments for 6GR. Based on the discussion and agreements achieved in RAN#109, we would like to propose following device types.

Proposal 10:		It would be important to classify device types according to concrete product groups expected in commercial deployments for 6GR as summarized below.

- Device types

  - Type I: e.g., IoT

  - Type II: e.g., Wearable with limited form factor (e.g., smartwatch)

  - Type III: e.g., Smartphone

  - Type IV: e.g., FWA

- Add-on use cases: NTN, [FFS for others]

- Key parameters for each device types (NTN can be applicable in addition to device types)

| Parameter list | Type IV (FWA) | Type III (Smartphone) | Type II (Wearable) | Type I (IoT) | NTN (if applicable) |
| --- | --- | --- | --- | --- | --- |
| TX antenna | 1-4Tx (depending on FRs) (Note: 2TX can be baseline) | 1Tx baseline (for all FRs), up to 2Tx (optional) | 1Tx | 1Tx | 1Tx to 4Tx (depending on PC). |
| RX antenna | 4, 6 or 8RX (optional) | Below 1 GHz: 2RX, up to 4RX (optional) Around 3.5 GHz: 4RX, Around 7 GHz: 4RX, up to 6 RX (optional) Around 24 GHz: 2RX, up to 4 RX (optional) | 1Rx | 1Rx and up to 2RX (optional) | Depending on device type |
| DL MIMO layer | Up to 4, 6 or 8Layer | Up to 2 or 4layer (depending on FR) | 1 layer | 1 or 2 layers (optional) | 1 layer |
| RF BW (max) | 400 MHz | 200 or [400] MHz | 20 MHz | 20 MHz | Depending on device type |
| Default output power | 26 dBm [or 29 dBm] | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | [Depending on outcome of 5G work] |
| DL max modulation order | 1024QAM or [4096QAM] | 1024QAM or [4096QAM] | 64 QAM | 256 QAM | Depending on device type |
| UL max modulation order | 256QAM or [1024QAM] | 256QAM or [1024QAM] | 64 QAM | 64 QAM | Depending on device type |

# 3	Conclusion

In this contribution, we provide our views on the general design principles and directions for the system parameter requirements of 6G RAN. Following observations and proposals were derived to be considered by this meeting.

Waveform and modulation

Observation 1:	In RAN1#122, the first agreement of 6GR was achieved to support OFDM-based waveforms such as CP-OFDM (downlink and uplink) and DFT-s-OFDM (uplink) waveform as the basis for 6GR.

Observation 2:	For upper limits of modulation order, the high-order modulation is expected as similar level of 1K-QAM for DL and 256-QAM for UL with the conventional formats as agreed in RAN1 for the study.

Observation 3:	Together with or without an AI-assisted receiver, study on the level of 1K-QAM for UL or 4K-QAM for DL can be considered in 6GR.

Proposal 1:		Based on the initial agreements in RAN1, it would be important for RAN4 to carry out the performance assessments by coordinating closely with RAN1. The study outcome should also consider the impact of implementation feasibility, and performance benefits compared to legacy requirements.

Numerology, FFT size and CBW

Observation 4:	Emerging 7–8 GHz bands can be treated within the FR1 framework, benefiting from similar propagation and deployment characteristics as the legacy sub-7 GHz spectrum according to the previous studies including a SI for IMT of RAN4.

Proposal 2: 		Among the three options, our recommendation for FR1 is to extend up to 8.4 GHz (option A or option B) at least, considering the channel propagation characteristics and previous studies of IMT SI.

Proposal 3:		To simplify implementation and improve spectral efficiency, we propose a single-SCS per range principle as below.

- FR1-FDD (≤ 8.4 GHz): 15 kHz

- FR1-TDD (≤ 8.4 GHz): 30 kHz

- 8.4 – 15.35 GHz: 60 kHz

- 15.35 – 24.25 GHz (if defined): 120 kHz

Observation 5:	8K FFT size is now being considered as a baseline configuration in RAN1 to enable effective processing of channels up to 200 MHz and beyond, allowing for greater spectral efficiency and increased capacity.

Proposal 4:		Table X is proposed as a unified numerological framework that simplifies and reduces the complexity of network management and allows devices to operate efficiently across both NR and 6GR.

Table 1: Potential SCS, FFT size, and CBW per frequency range

| FR | Range | SCS (kHz) | Required FFT size | Max CBW (MHz) | Min CBW (MHz) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 400 MHz – 6.425 GHz | 15 (FDD) 30 (TDD) | 8192 | 100 (FDD) 200 (TDD) | 5 (FDD) 10 (TDD) |
|  | 6.425 – 7.125 GHz | 30 | 8192, [16384] | 200, [400] | 20 |
| FR1-extended | 7.125 – 8.4 GHz | 30 | 8192, [16384] | 200, [400] | 20 |
| [new FR] | 8.4 – 15.35 GHz | 60 | 8192 | 400 | 20 |
| [new FR] | 15.35 – 24.25 GHz | 120 (if defined) | 4096, 8192 | 400, 800 | 50 |
| FR2-1 | 24.25 – 52.6 GHz | 120 | 4096, 8192 | 400, 800 | 50 |

Spectrum utilization and irregular CBW

Proposal 5: 		6G can adopt and evolve 5G NR design philosophy as much as possible, which is to retain the principle of a unified spectral emission mask, defined by a maximum RBs and minimum guardbands

Proposal 6: 		As a forward-looking evolution, 6GR should explore the concept of irregular channel bandwidths based on 5G framework, and asymmetric band or bandwidths for the extreme spectral efficiency.

Synchronization signal and raster

Observation 6:	The pursuit of SSB placement flexibility of 5G introduced superfluous raster points, increasing UE processing overhead and extending initial cell search duration in wideband scenarios.

Observation 7:	Furthermore, the default 20ms SSB periodicity of 5G constrained network operators’ ability to optimize energy efficiency dynamically.

Proposal 7:		It is recommended for RAN4 to have a fundamental principle for 6G sync raster design considering energy efficiency/saving as well as design complexity and flexibility as follows.

- Limited step sizes (e.g., 120kHz for FR1, 480kHz for FR2) to reduce grid density while maintaining CBW coverage

- Adaptive SSB periodicities (e.g., 40ms for coverage areas, 80ms for high-capacity hotspots) to maximizes energy efficiency gains

#Rx and #Tx

Observation 8:	Increasing the number of UE antennas does not guarantee the performance improvement according to the previous discussion in RAN4.

Proposal 8:		Consider the 1T/4R architecture from NR for FR1 at the UE, and study the feasibility of sNB/UE architectures such are 6Rx only for FR3.

Device types

Proposal 9:		Introduce a modular and MAC-layer-assisted UE capability and assistance information reporting framework in 6GR, enabling category-based network optimization while minimizing per-UE management overhead.

Proposal 10:		It would be important to classify device types according to concrete product groups expected in commercial deployments for 6GR as summarized below.

- Device types

  - Type I: e.g., IoT

  - Type II: e.g., Wearable with limited form factor (e.g., smartwatch)

  - Type III: e.g., Smartphone

  - Type IV: e.g., FWA

- Add-on use cases: NTN, [FFS for others]

- Key parameters for each device types (NTN can be applicable in addition to device types)

| Parameter list | Type IV (FWA) | Type III (Smartphone) | Type II (Wearable) | Type I (IoT) | NTN (if applicable) |
| --- | --- | --- | --- | --- | --- |
| TX antenna | 1-4Tx (depending on FRs) (Note: 2TX can be baseline) | 1Tx baseline (for all FRs), up to 2Tx (optional) | 1Tx | 1Tx | 1Tx to 4Tx (depending on PC). |
| RX antenna | 4, 6 or 8RX (optional) | Below 1 GHz: 2RX, up to 4RX (optional) Around 3.5 GHz: 4RX, Around 7 GHz: 4RX, up to 6 RX (optional) Around 24 GHz: 2RX, up to 4 RX (optional) | 1Rx | 1Rx and up to 2RX (optional) | Depending on device type |
| DL MIMO layer | Up to 4, 6 or 8Layer | Up to 2 or 4layer (depending on FR) | 1 layer | 1 or 2 layers (optional) | 1 layer |
| RF BW (max) | 400 MHz | 200 or [400] MHz | 20 MHz | 20 MHz | Depending on device type |
| Default output power | 26 dBm [or 29 dBm] | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | [Depending on outcome of 5G work] |
| DL max modulation order | 1024QAM or [4096QAM] | 1024QAM or [4096QAM] | 64 QAM | 256 QAM | Depending on device type |
| UL max modulation order | 256QAM or [1024QAM] | 256QAM or [1024QAM] | 64 QAM | 64 QAM | Depending on device type |

# Reference

[1] RP-252912	Revised SID: Study on 6G Radio		RAN#109

[2]	R4-2511652	Plan on RAN4 6G discussion and meeting arrangement in Rel-20	RAN4#116

[3]	3GPP TS 38.101	NR; User Equipment (UE) radio transmission and reception

[4]	Report of 3GPP TSG RAN WG1 #122