3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513053

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on RAN4 operation efficiency for 6GR

Agenda item:			8.13

Document for:	Approval

# 1. Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [1, RP-252912] in RAN#109. Specifically, the topic of RAN4 operation efficiency in the RAN4 6G topic is planned with the following scopes [2]:

| 6G Topics | Scope | Scope in Q4'25 and Q1'26 |
| --- | --- | --- |
| RAN4 operation efficiency | CR, spec improvement and modernization in general, new tools for band/band combinations, other aspects | CR, spec improvement and modernization in general, new tools for band/band combinations, other aspects |

In this contribution, we would like to provide our viewpoints on RAN4 operation efficiency in the 6G study.

# 2. Discussion

## 2.1 Specification improvement and modernization

Based on Rel-15 experience, the specification editors played the most important role to control the specification quality. RAN4 specification contains very broad areas from RF to baseband, which further require editor-led procedure is important for 6G specification quality control.

Proposal 1: For specification quality improvement, the following procedures led by specification editors can be considered:

- Before CR to be agreed, it is encouraged to use the running CR approach to allow more time for CR editing.

- CR drafted by companies can only be approved after the review of specification editor.

- For each specification, the online drafting session can be organized by the specification editor in the gap period between two RAN4 meetings.

Another issue for specification modernization is how to capture the standardized AI/ML model (e.g., AI/ML model for conformance testing, AI/ML model used for reference purpose, etc.) and standardized dataset (e.g., dataset to derive reference model, etc.) in the 3GPP specification. In Rel-19 discussion on RAN1-led AI/ML work item, RAN4 has practice both dataset and model sharing for the practice of two-sided model interoperability study. The following are agreements achieved for logistical issues:

Agreement For AI/ML model formats: - Use at least ONNX format for sharing. - Companies can also share PyTorch model - The models in different formats from one company shall be the same - Use version v1.16 or later - ONNX version and opset version number to be included in the file Agreement: For dataset format - Use NumPy for dataset sharing - Do not use pickled data(compression mechanism) - Use npy – single array in each file Agreement: Dataset/model input file format: - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers) - In other words, dataset should content following info: - 1st dimension: Number of samples - 2nd dimension: Real and imaginary - 3rd dimension: Number of sub-bands - 4th dimension: Number of antenna ports - 5th dimension: Number of layers – present only if >1 layer - Note: Each element of the dataset will be a float32 real number The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447 Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization Dataset files can be split into multiple files to enable easier upload. - Use the 2 digits for split files starting from 00, increment for each additional file Split files and then archive each file Agreement: For file naming scheme - Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder” - Subfolder created for each meeting File naming scheme( ML model file and dataset file) - a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary) - meeting number - differentiate model and dataset by identifier (2 characters ml and ds) - additional identifier with 2 characters (TBD) - ec for encoder - dc for decoder - ei for encoder input - do for decoder output - lt for latent (decoder input or encoder output) - others to be added as needed - dataset can be split in multiple files – 2 digits - files to be compressed to zips and uploaded Agreement: Files to be shared, to be decided based on option under study - Dataset containing encoder input per subband - Encoder and/or decoder model

Observation 1: With AI/ML-enabled features contained in the 6G specification, how to capture the standardized AI/ML model and the standardized dataset in the 3GPP specification are the issues to be studied.

Based on Rel-19 experience, at least the following aspects needs to be studied for how to capture the standardized AI/ML model and standardized dataset in the 3GPP specification.

Proposal 2: For specification modernization for capturing the standardized AI/ML model and the standardized dataset, the following issues can be studied in RAN4:

- Unified AI/ML model format for sharing

- Dataset format and data description

- AI/ML model and dataset naming rules

- Methods of sharing (including the current FTP-based method) by considering download/upload efficiency, the convenience for model/dataset update, and the availability of the location link for easy citation

## 2.2 New tools for band/band combinations

The current RAN4 auto-tool development is primarily focused on CA configuration tables, i.e., use the updated JSON schema file for further development of the band combination database. As agreed in RAN4#115 [3], RAN4 will focus on inclusion of the CA configuration tables to the database as a first step. Other aspects, such as delta values and MSD requirements, can be considered at a later stage. The goal in 5G is to fully replace RAN4 CA configuration tables with the machine-readable JSON files in Rel-21, while conducting parallel trials in Rel-20.

There were proposals to extend JSON files to other cumbersome requirement tables, such as ΔTIB /ΔRIB and MSD tables. For 6G, the plan should consider the outcomes and progress of these requirements as discussed in 6G SI. For instance, we have proposals in 6G to replace ΔTIB /ΔRIB case-by-case analysis by general rules, except for exceptional cases if identified as significantly deviating from the rules, and to further simplify MSD to explore the band-group basis requirements. With these changes, the tables would only cover exceptional cases which are not expected to occupy extensive pages, thereby eliminating the need for JSON files for these cases. Additionally, unlike CA configurations, these requirements are not stored on either UE or BS side, making machine-readable schema unnecessary.

For CA configuration tables, there is no doubt that 6G should continue the efforts initiated in 5G while also considering the outcome in 6G related to CA configurations. For instance, we have proposals for 6G to discuss the removal of BCSs concept, which will subsequently impact the structure of JSON files.

Proposal 3: The JSON schema continue to be used for the development of band combination database, with careful consideration of the outcomes and advancements in 6G related to band combinations.

- CA configuration tables as first priority

- Other requirement tables are not precluded if justifiable

# 3. Conclusion

In this contribution, we have provided our viewpoints on RAN4 operation efficiency in the 6G study:

Proposal 1: For specification quality improvement, the following procedures led by specification editors can be considered:

- Before CR to be agreed, it is encouraged to use the running CR approach to allow more time for CR editing.

- CR drafted by companies can only be approved after the review of specification editor.

- For each specification, the online drafting session can be organized by the specification editor in the gap period between two RAN4 meetings.

Observation 1: With AI/ML-enabled features contained in the 6G specification, how to capture the standardized AI/ML model and the standardized dataset in the 3GPP specification are the issues to be studied.

Proposal 2: For specification modernization for capturing the standardized AI/ML model and the standardized dataset, the following issues can be studied in RAN4:

- Unified AI/ML model format for sharing

- Dataset format and data description

- AI/ML model and dataset naming rules

- Methods of sharing (including the current FTP-based method) by considering download/upload efficiency, the convenience for model/dataset update, and the availability of the location link for easy citation

Proposal 3: The JSON schema continue to be used for the development of band combination database, with careful consideration of the outcomes and advancements in 6G related to band combinations.

- CA configuration tables as first priority

- Other requirement tables are not precluded if justifiable

# 4. Reference

[1] RP-252912 (revision from RP-251881), “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

[2] R4-2511652, “RAN4 6G Plan and Meeting Arrangement”, RAN4 chair, RAN4#116.

[3] R4-2508014, WF for the ETSI MCC band combination database, RAN#115, Nokia, Ericsson