3GPP TSG-RAN WG4 Meeting #116bis		                                     R4-2513092

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	4.22.3

Source:	OPPO

Title:	Discussion on remaining issues of core requirements maintenance for LP-WUSWUR

Document for:	Approval

- Introduction

In last meeting, the core part of RRM requirements regarding LP-WUS/WUR has been closed and the big CR [1] is agreed. But there are still some remaining issues to be concluded. This contribution will provide our further considerations on the remaining issues captured in [2][3].

- Discussion

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay. Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met Discuss on the measurement periodicity after wake up. Discuss on whether the wake up delay apply in the scenario when exiting from case 3.

In previous meeting, we discussed and agreed the MR wake-up delay that a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In last meeting, we agreed that the same MR wake-up delay is used when UE exiting case 1 due to the exiting conditions of offloading is met. And there are two remaining issues captured in the approved WF.

The first on is whether it should also be applied when UE exiting case 3 which is relaxed measurement mode. Based on the latest RAN2 CR as below, MR threshold is mandatory while LR is optional for the entry condition of case 3. And there is no clear exit conditions defined which means UE can exit case 3 when the entry condition is not met. In such case, we understand we do not need to define wake-up delay for exiting case 3 since MR measurement is needed for evaluation and MR is already wake-up when UE needs to exit case 3 after evaluation.

Proposal 1: Do not apply the wake-up delay in the scenario when exiting case 3.

##### 5.2.4.x.2 Relaxed measurement criterion The relaxed measurement criterion for serving cell and neighbouring cell measurement relaxation on MR is fulfilled when: - Srxlev > SLP_WUS_RelaxThresholdP_MR, and, - Qrxlevmeas_lr > QLP_WUS_RelaxThresholdP_LR, if QLP_WUS_EntryThresholdP_LR is configured, and, - Squal > SLP_WUS_RelaxThresholdQ_MR, if SLP_WUS_RelaxThresholdQ_MR is configured, and - Qqualmeas_lr > QLP_WUS_RelaxThresholdQ_LR, if QLP_WUS_EntryThresholdQ_LR is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB). - Qrxlevmeas_lr = current measured cell RX level value of the serving cell based on LR (RSRP). - Qqualmeas_lr = current measured cell quality value of the serving cell based on LR (RSRQ). - SLP_WUS_RelaxThresholdP_MR: The Srxlev threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on MR. - SLP_WUS_RelaxThresholdQ_MR: The Squal threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on MR. - QLP_WUS_RelaxThresholdP_LR: The cell RX level threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on LR. - QLP_WUS_RelaxThresholdQ_LR: The cell quality threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on LR. These thresholds can be configured separately for LR measurements based on LP-SS and LR measurements based on SSB if a cell supports both measurement types as specified in TS 38.331 [3]. SLP_WUS_RelaxThresholdP_MR is the parameter SSearchThresholdP3 or SSearchThresholdP4 in TS 38.331 [3]. SLP_WUS_RelaxThresholdQ_MR is the parameter SSearchThresholdQ3 or SSearchThresholdQ4 in TS 38.331 [3]. QLP_WUS_RelaxThresholdP_LR is the parameter SRSRPThresholdLR or SRSRPThresholdLR2 in TS 38.331 [3]. QLP_WUS_RelaxThresholdQ_LR is the parameter SRSRQThresholdLR or SRSRQThresholdLR2 in TS 38.331 [3]. If UE supports both measurement types, it is up to UE implementation to choose LR measurments based on LP-SS or based on SSB for the determination of the relaxed measurement criterion for serving cell and neighbouring cell measurement relaxation on MR.

The second issue is the measurement requirements after waking up. There are two cases that UE need to wake up MR. The first one is when UE is at case 1 and need to perform high priority layers measurement. In such case, it is clear that MR only need to perform relaxed neighbor cell measurement. The second one is when UE exits case 1. For such case, RAN2 has defined the entry and exit conditions for serving cell offloading as below.

##### 5.2.4.x.3 Serving cell measurement offloading rules UE supporting LP-WUS may choose to perform serving cell measurement offloading (i.e., serving cell measurement is fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. UE supporting LP-WUS is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. ##### 5.2.4.x.4 Serving cell measurement offloading criterion The entry condition for serving cell measurement offloading is fulfilled when: - Srxlev > SLP_WUS_offloadingEntryThresholdP_MR, and, - Qrxlevmeas_lr > QLP_WUS_offloadingEntryThresholdP_LR, if QLP_WUS_offloadingxEntryThresholdP_LR is configured, and, - Squal > SLP_WUS_offloadingEntryThresholdQ_MR, if SLP_WUS_offloadingEntryThresholdQ_MR is configured, and - Qqualmeas_lr > QLP_WUS_offloadingEntryThresholdQ_LR, if QLP_WUS_offloadingEntryThresholdQ_LR is configured. The exit condition for serving cell measurement offloading is fulfilled when: - Qrxlevmeas_lr < QLP_WUS_offloadingExitThresholdP_LR, or, - Qqualmeas_lr < QLP_WUS_offloadingExitThresholdQ_LR, if QLP_WUS_offloadingExitThresholdQ_LR is configured. Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB). - Qrxlevmeas_lr= current measured cell RX level value of the serving cell based on LR (RSRP). - Qqualmeas _lr = current measured cell quality value of the serving cell based on LR (RSRQ). - SLP_WUS_offloadingEntryThresholdP_MR: The Srxlev threshold for entry condition of serving cell measurement offloading based on MR. - SLP_WUS_offloadingEntryThresholdQ_MR: The Squal threshold for entry condition of serving cell measurement offloading based on MR. - QLP_WUS_offloadingEntryThresholdP_LR: The cell RX level threshold for entry condition of serving cell measurement offloading based on LR. - QLP_WUS_offloadingEntryThresholdQ_LR: The cell quality threshold for entry condition of serving cell measurement offloading based on LR. - QLP_WUS_offloadingExitThresholdP_LR: The cell RX level threshold for exit condition of serving cell measurement offloading based on LR. - QLP_WUS_offloadingExitThresholdQ_LR: The cell quality threshold for exit condition of serving cell measurement offloading based on LR. These thresholds can be configured separately for LR measurments based on LP-SS and LR measurements based on SSB if a cell supports both measurement types as specified in TS 38.331 [3]. SLP_WUS_offloadingEntryThresholdP_MR is the parameter SSearchThresholdP5 or SSearchThresholdP6 in TS 38.331 [3]. SLP_WUS_offloadingEntryThresholdQ_MR is the parameter SSearchThresholdQ5 or SSearchThresholdQ6 in TS 38.331 [3]. QLP_WUS_offloadingEntryThresholdP_LR is the parameter SRSRPThresholdLR3 or SRSRPThresholdLR4 in TS 38.331 [3]. QLP_WUS_offloadingEntryThresholdQ_LR is the parameter SRSRQThresholdLR3 or SRSRQThresholdLR4 in TS 38.331 [3]. QLP_WUS_offloadingExitThresholdP_LR is the parameter SRSRPThresholdLR5 or SRSRPThresholdLR6 in TS 38.331 [3]. QLP_WUS_offloadingExitThresholdQ_LR is the parameter SRSRQThresholdLR5 or SRSRQThresholdLR6 in TS 38.331 [3]. If UE supports both measurement types, it is up to UE implementation to choose LR measurments based on LP-SS or based on SSB for the determination of the entry/exit conditions of serving cell measurement offloading.

Based on the behavior description, LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading is fulfilled. But which requirements to be met after exiting, relaxed measurement or legacy measurement, is not clear. From our understanding, the default behavior should be legacy if no corresponding criterion is evaluated and fulfilled. But we can also see that only one evaluation period will be impacted and after this evaluation, the UE behavior will be clear. So we think it should be also OK to leave it to UE implementation.

Proposal 2: When UE exits the serving cell offloading case, choose one of the following approaches for UE behavior and requirements:

- Option 1: UE follows legacy measurement and meet the corresponding requirements.

- Option 2: Up to UE implementation and no requirements apply.

One additional issue we want to clarify is the following time for cell selection initiation which is 10s for legacy UE without eDRX_IDLE cycle configured. But for LP-WUS UE, the measurement time for intra-frequency, inter-frequency and inter-RAT measurement has already exceeded 10s, the time for cell selection initiation may need to be updated.

##### 4.X.2.3.1 Requirements for evaluation of cell selection criterion When LP-WUS UE is not in relaxed measurement mode as defined in [1], the requirements in clause 4.2.2.2 shall apply. When LP-WUS UE is in relaxed measurement mode as defined in [1], the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every N1*16 DRX cycle. The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either 8 DRX cycles or DRX cycle/2. If the LP-WUS UE has evaluated according to table 4.X.2.3.1-1 in 16*Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the LP-WUS UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting LP-WUS UE measurement activities. If the LP-WUS UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the LP-WUS UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where T= 10s. Table 4.X.2.3.1-1: Nserv DRX cycle length [s]; Scaling Factor (N1); Nserv [number of 16*DRX cycles]. FR1; FR2-1Note1. 0.32; 1; 8; N1*4. 0.64; 5; N1*4. 1.28; 4; N1*2. 2.56; 3; N1*2. NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length..

According to the existing spec, the procedure for cell selection is that: UE performs the measurement on serving cell and evaluate the S criteria, if the UE has evaluated in N consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, UE shall initiate the measurement of all neighbor cell measurement. And then if the UE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].

But for the LP-WUS UE in relaxed measurement mode, the evaluation period for cell selection criterion S and the measurement period for intra-frequency, inter-frequency and inter-RAT measurement are both extended. In such case, the measurement period for neighbor cell already exceeds 10s which means UE may not finish the measurement for neighbor cell in 10s. It is not reasonable to initiate cell selection since it does not mean UE cannot find suitable neighbor cell but because UE has not finished the measurement and evaluation.

Proposal 3: RAN4 to update the time for cell selection initiation for LP-WUS UE in case 3 i.e., relaxed measurement mode.

- Conclusion

This contribution gave the following proposals on the remaining issues for the core requirements of LP-WUR/WUS.

Proposal 1: Do not apply the wake-up delay in the scenario when exiting case 3.

Proposal 2: When UE exits the serving cell offloading case, choose one of the following approaches for UE behavior and requirements:

- Option 1: UE follows legacy measurement and meet the corresponding requirements.

- Option 2: Up to UE implementation and no requirements apply.

Proposal 3: RAN4 to update the time for cell selection initiation for LP-WUS UE in case 3 i.e., relaxed measurement mode.

References

- R4-2512354 Big CR on RRM requirements for LP-WUS/WUR, vivo, RAN4#116

- R4-2509065 Topic summary for [116][222] NR_LPWUS, vivo, RAN4#116

- R4-2512139 WF on RRM requirements for NR_LPWUS, vivo, RAN4#116