3GPP TSG-RAN WG4 Meeting #116bis				                                                              R4-2513098

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	7.12.4

Source: 	OPPO

Title: 	Discussion on RAN4 requirements for measurement event prediction

Document for:	Approval

- Introduction

The WID for Rel-20 AI mobility was agreed in [1] and revised in [2]. The objectives of core part are as following:

The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

According to the objectives, RAN4 need to define core requirements for the supported use cases and LCM related requirements for LCM functionality management for UE sided model. In this contribution, we will discuss the identified requirements for measurement event prediction and give our proposals.

- Discussion

According to the objectives in WID, the indirect measurement event prediction will be supported with the same scenarios as measurement prediction while the feasibility of core requirements for direct event prediction needs to be discussed in Q2 2026. So in this section, the discussion will focus on indirect measurement event prediction.

Observation 1: The feasibility of direct measurement event prediction needs to be studied in Q2 2026.

For indirect measurement event prediction, the inference procedure is illustrated as below (Figure in TR 38.744):

Figure 1: Indirect measurement event prediction

Based on the studies in the SI, for indirect event prediction method, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A, temporal domain case B, frequency domain or spatial domain at first. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

With this definition, we understand the AI/ML model implementation to get predicted L3-RSRP for measurement prediction and indirect measurement event prediction are the same. The model related requirements including measurement delay, prediction delay and LCM related requirements for measurement prediction can be reused to indirect measurement event prediction. And since the occurrence of the measurement event is based on the calculation of measured and predicted results according to the legacy procedure, we think no additional delay is needed between the report of predicted L3-RSRP and report of event occurrence, i.e., the two results are reported together. So the reporting delay requirements can also be reused to indirect measurement event prediction.

Proposal 1: The core requirements defined for measurement prediction e.g., measurement/prediction/reporting delay and LCM related requirements can be reused to indirect measurement event prediction.

In study item, it was agreed that RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP if the report includes the predicted RSRPs corresponding to the predicted event occurrence time. According to RAN2 discussion, the spec impact of measurement prediction can be applicable for measurement event prediction unless otherwise described explicitly. Based on this design, we think the absolute and/or relative accuracy requirements defined for measurement prediction can be also reused for the predicted RSRP in measurement event prediction.

Proposal 2: The absolute and/or relative accuracy requirements defined for measurement prediction can be also reused for the predicted L3-RSRP in indirect measurement event prediction.

The difference between measurement prediction and indirect measurement event prediction is the final output of indirect event prediction, i.e., the occurrence of measurement event. The main issue is the metrics/KPI for this final output which has been discussed in study item.

During study item, RAN2 has used the following metrics for evaluation of indirect event prediction:

### 5.3.1 Evaluation methodology, metrics and assumptions The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa; - Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD]; - Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]. The ETD, i.e. timing difference between ground-truth event and predicted event, is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

But after that, they are also reconsidering the performance metrics for monitoring and some companies think the F1 score may have problem to derive since the ground truth is not available. And they decided to use RSRP difference as the metrics for performance monitoring for indirect event prediction. In RAN4 discussion during study item, there are many proposals to define the performance metrics for the final output of indirect event prediction, we think we need wait for more progress from RAN2 on this case but we are OK to firstly align the understanding on each proposal.

Proposal 3: For indirect measurement event prediction, wait for more RAN2 progress to discuss the performance metrics and accuracy requirements for the final output.

- Conclusions

In this contribution, we discussed the RAN4 requirements for RRM measurement event prediction and got the following observations and proposals:

Observation 1: The feasibility of direct measurement event prediction needs to be studied in Q2 2026.

Proposal 1: The core requirements defined for measurement prediction e.g., measurement/prediction/reporting delay and LCM related requirements can be reused to indirect measurement event prediction.

Proposal 2: The absolute and/or relative accuracy requirements defined for measurement prediction can be also reused for the predicted L3-RSRP in indirect measurement event prediction.

Proposal 3: For indirect measurement event prediction, wait for more RAN2 progress to discuss the performance metrics and accuracy requirements for the final output.

- References

RP-251864 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#108

RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#109

3GPP TR 38.744: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR”