3GPP TSG-RAN WG4 Meeting #116bis		 												R4-2513108

Prague, Czech Republic, October 13th – 17th, 2025

Title: 	On RRM performance requirements of event triggered L1 measurement reporting

Source: 	OPPO

Agenda item:	6.18.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on RRM perf part for event triggered L1 measurement reporting.

Discussion

Event-triggered L1 reporting

- Issue 4-1-1: test case list for event-triggered L1 reporting Candidate options: - Option 1: Nokia - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps) - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps) - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps) - At least 1 test case for event-triggered periodic reporting. - Option 2: CATT - SSB based test cases: - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX - TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading - CSI-RS based test case: - Only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined. - Option 3: Apple, [vivo?] - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1 - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1 - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2 - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2 - Option 4: CTC - LTM Intra-frequency SSB/CSI-RS based event triggered L1-RSRP measurement reporting tests without gap in FR1/FR2. - LTM Inter-frequency SSB based event triggered L1-RSRP measurement reporting tests with/without gap in FR1/FR2. - LTM Inter-frequency CSI-RS based event triggered L1-RSRP measurement reporting tests with gap in FR1/FR2. - Option 5: OPPO - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1 - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1 - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2 - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2 - Option 6: MTK - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1 - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2 - Option 7: CMCC - Both FR1 and FR2-1 are considered. - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered. - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered. - Option 8: ZTE - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps) - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps) - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps) - Option 9: HW - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement) - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement) - FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement) - FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement) - Option 10: E/// Intra-f L1-RSRP measurement for LTM; - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated. Inter-f L1-RSRP measurement with MG; - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells. - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells. Inter-f L1-RSRP measurement without gap for LTM; - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells. - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated. Recommended WF: - Further discuss based on the following directions - For SSB based L1 RSRP measurement reporting: - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple) - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW) - For CSI-RS based L1 RSRP measurement reporting: - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. - For UE supporting event triggered reporting, UE only needs to pass 2) - For UE not supporting event triggered reporting, UE only needs to pass 1) - Issue 4-1-2: test configuration for event-triggered L1 reporting Candidate options: - Proposal 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia) - Use LTM3 as the reporting event in the test cases. - Proposal 2: it is proposed to define test cases for LTM event 2/3/4/5. (CMCC) - Proposal 3: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW) - Proposal 4: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///) Agreement: - On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

For the test cases for event triggered reporting, we agree that the following aspects should be considered, like SSB or CSI-RS based, intra-frequency or inter-frequency, measurement with gap or without gap, FR1 or FR2, non-DRX or DRX, with SSB index reading or without SSB index reading. However, compared to legacy tests for LTM, the new tests should cover the delta parts mainly about event triggered reporting. To reduce the amount of test cases, we prefer to define 2 cases to cover different configurations in different scenarios.

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

Proposal 1: For the test cases for event triggered reporting, we propose 4 tests:

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

Conclusion

In the contribution we provide the following proposals:

Proposal 1: For the test cases for event triggered reporting, we propose 4 tests:

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

Reference

- R4-2512141, WF on NR mobility enhancements Phase 4_part1