3GPP TSG-RAN WG4 Meeting #116bis		 												R4-2513109

Prague, Czech Republic, October 13th – 17th, 2025

Title: 	On RRM performance requirements of CSI-RS based L1 measurement

Source: 	OPPO

Agenda item:	6.18.2.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on RRM perf part for CSI-RS based L1 measurement.

Discussion

- Issue 4-2-1: CSI-RS based L1 measurement accuracy Agreement: - CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell. - New requirements are defined in the following sections Accuracy requirements; Detail. Intra-frequency L1-RSRP accuracy requirements; 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy. 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Relative Accuracy.

RAN4 agreed that for CSI-RS based L1-RSRP measurement on neighbour cell,  reusing legacy accuracy requirements of CSI-RS based L1 measurements for serving cell under the following conditions:

  - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

  - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

And RAN4 will specify absolute and relative intra-frequency L1-RSRP accuracy requirements for FR1 and FR2.

- Issue 4-2-2: test case list for CSI-RS based L1 measurement Candidate options - Option 1: Nokia - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1 - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1 - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping - Option 2: CATT - RAN4 to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2. - Option 3: Apple, OPPO, [vivo?] - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 - Option 4: MTK - CSI-RS based L1 RSRP measurement for neighbour cell in FR1 - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 - Option 5: CMCC - For CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested. - Option 6: HW - As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement. - In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. - FR1 and FR2 LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases for neighbour cell are to be specified. - Option 7: E/// Intra-f L1-RSRP measurement for LTM; - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells,. Inter-f L1-RSRP measurement without gap for LTM; - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells. - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated. Recommended WF: - Discuss the options.

We support to only consider tests for intra-frequency CSI-RS based L1-RSRP measurement.

Proposal 1: Support to define the following test cases:

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW Candidate options: - Option 1: Test configuration is set to have 48 PRBs within the active BWP. (Nokia) Recommended WF: - Agree on option 1. - Issue 4-2-4: others Candidate options: - Proposal 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo) - Proposal 1a: Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 1b: Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 2: Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP. (Nokia) Recommended WF: Discuss the proposals.

RAN4 have no consensus on defining new RAN4 requirements for early CSI acquisition. Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached.

It is RAN4 common understanding that for baseline early CSI acquisition, if fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell. Otherwise, UE shall start CSI acquisition at least after Tprocessing. This means, it is up to UE implementation based on different conditions.

Since no core requirements for early CSI acquisition, there is no need to define any test case(s) to verify early CSI acquisition. It does not impact UE implementation.

Proposal 2: Not define any test case(s) to verify early CSI acquisition.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: Support to define the following test cases:

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

Proposal 2: Not define any Define test case(s) to verify early CSI acquisition if core requirements are specified.

Reference

- R4-2512141, WF on NR mobility enhancements Phase 4_part1