3GPP TSG-RAN WG4 Meeting #116bis		 															R4-2513111

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.19.2

Source:	OPPO

Title:	Discussion on RRM performance requirements for XR

Document for:	Approval

- Introduction

This contribution will provide our considerations on test case design for XR due to the MG cancellation.

- Discussion

### Issue 2-2-3: Per UE and per FR gaps <Way forward> Discuss the following options - Option 1: Test cases are specified only for per-UE MGs - Option 2: Test cases are specified for both per-UE and per-FR MGs

It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs.

As agreed, UE behaviour for the MG skipping is different for per-FR gap and per-UE gap. For per-FR gap, UE can only skip the gap in the same FR and can enable Tx/Rx during the gap occasion in the same FR. However, UE behaviour on the other FR will not be impacted. When the MG skipping is applicable to a per-UE gap, UE can enable Tx/Rx for both FRs. These two cases should be verified in the test cases.

Proposal 1: Consider both per-FR gap and per-UE gap in the test case.

### Issue 2-2-4: Type 1 and Type 2 gaps <Way forward> Discuss the following options - Option 1: Test cases are defined for both Type 1 and Type 2 gaps - Option 2: Test cases are defined only for Type 1 gaps

Besides, the MG skipping scheme applies to Rel-17 concurrent gaps, and the gap occasion surviving the Rel-17 gap collision handling rules will be cancelled in XR. This scenario should also be considered in the test case.

Proposal 2: Consider Rel-17 concurrent gaps in the test case.

### Issue 2-2-7: List of test cases <Way forward> - Define test cases for - Proposal 1: Intra-frequency measurements with gaps - Proposal 2: Inter-frequency measurements with gaps - Proposal 3: Inter-RAT E-UTRAN FDD measurements - Proposal 4: Inter-RAT E-UTRAN TDD measurements - Proposal 5: Inter-frequency L1-RSRP with MG - Proposal 6: L3 and L1 measurements without gaps

When the MG occasion is cancelled by DCI, the delay requirements for the MO which is supposed to be measured in the MG will be extended and RAN4 have identified the impacted clauses as shown above. We think the extended delay requirements for the measurement above should be verified.

Proposal 3: Define test cases for:

- Intra-frequency measurements with gaps

- Inter-frequency measurements with gaps

- Inter-RAT E-UTRAN FDD measurements

- Inter-RAT E-UTRAN TDD measurements

- Inter-frequency L1-RSRP with MG

### Issue 2-2-5: DRX proposals <Way forward> Discuss the following proposals: - Proposal 1: Define the test cases for the cases without DRX and with DRX. - Proposal 2: Test cases without DRX should have higher priority than with DRX. - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG. - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority. - Focus on non-DRX test cases for Type 2 MGs. - Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both.

Although the exact delay extension requirements are not determined yet, it seem different formulas may be used without DRX and with DRX. We propose to consider both cases when defining test cases.

Proposal 4: Define the test cases for the cases without DRX and with DRX.

### Issue 2-2-6: Test target <Way forward> - RRM test cases should verify - Proposal 1: accuracy requirements - Proposal 2: delay extension with gap cancellation - Proposal 3: Reception/transmission without scheduling restrictions during cancelled gap occasions - Proposal 4: The UE processing timeline of receiving the DCI-based MG skipping indication

To reduce the workload on test cases and meanwhile maintain the test coverage, it is better to verify both UE behaviour of Tx/Rx on the skipping MG occasion and the extended delay in the same test cases.

Proposal 5: UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case.

- Conclusion

This contribution gave the following proposals for test cases considering the MG skipping in XR:

Proposal 1: Consider both per-FR gap and per-UE gap in the test case.

Proposal 2: Consider Rel-17 concurrent gaps in the test case.

Proposal 3: Define test cases for:

- Intra-frequency measurements with gaps

- Inter-frequency measurements with gaps

- Inter-RAT E-UTRAN FDD measurements

- Inter-RAT E-UTRAN TDD measurements

- Inter-frequency L1-RSRP with MG

Proposal 4: Define the test cases for the cases without DRX and with DRX.

Proposal 5: UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case.

References

- R4-2512144, WF on RRM requirements for NR_XR_Ph3