3GPP TSG-RAN WG4 Meeting #116bis		 										R4-2513112

Prague, Czech Republic, October 13th – 17th, 2025

Source: 	OPPO

Title: 	On RRM requirements for (e)RedCap UE enhancement in RRM phase 6

Agenda Item:	7.4.3.2

Document for:	Discussion

Introduction

A WI for RRM phase 6 was approved in last RAN-P meeting.

### Objective of SI or Core part WI or Testing part WI - Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

In this document, we provide our views on the RRM requirements of e)RedCap UE enhancement.

Discussion

Existing Rel-17 RLM/BFD relaxation for power saving has been specified only for non-RedCap UEs in FR1 and FR2, and it is natural to extend it to (e)RedCap UEs. Furthermore, it is important to extend less than 5MHz support from normal UE to (e)RedCap UEs to reduce complexity and cost.

For Redcap UE, we think the following aspects of R17 RLM/BFD relaxation for legacy UE can be taken as a baseline:

- the applicability of RLM measurement relaxation

- the relaxed requirements for SSB based radio link monitoring

- the relaxed requirements for CSI-RS based radio link monitoring

- the applicability of RLM measurement relaxation

- the relaxed requirements for SSB based BFD

- the relaxed requirements for CSI-RS based BFD

Proposal 1: The following RRM aspects of R17 RLM/BFD relaxation for legacy UE can be taken as a baseline:

1）	the applicability of RLM measurement relaxation

2）	the relaxed requirements for SSB based radio link monitoring

3）	the relaxed requirements for CSI-RS based radio link monitoring

4）	the applicability of RLM measurement relaxation

5）	the relaxed requirements for SSB based BFD

6）	the relaxed requirements for CSI-RS based BFD

For applicability rules, CA scenario is precluded for Redcap UE and less than 5 MHz BW UE, and RAN4 does not need to consider the case of BFD on SCell. Only RLM and BFD on PCell is considered.

Proposal 2: The relaxed RLM/BFD requirements on PCell is considered for Redcap UE and less than 5 MHz BW.

For relaxed Measurement Criteria, low mobility criterion and good serving cell quality criterion can be reused, and two cases includes

- Case 1: only goodServingCellEvaluationBFD is configured and good serving cell quality criterion is fulfilled and

- Case 2: both goodServingCellEvaluationBFD and lowMobilityEvaluationConnected are configured, and both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] are fulfilled for the serving cell configured with RLM RS or BFD RS.

The similar cases are considered for Redcap UE.

Proposal 3: Two cases are considered as well for Redcap UE or less than 5 MHz BW UE.

- Case 1: only goodServingCellEvaluationBFD is configured and good serving cell quality criterion is fulfilled and

- Case 2: both goodServingCellEvaluationBFD and lowMobilityEvaluationConnected are configured, and both low mobility criterion and good serving cell quality criterion are fulfilled

The relaxed requirements are also not applied to Redcap UE,

- if at least one of the following conditions is met for RLM

  - The UE sends out-of sync indications to the higher layers,

  - The timer T310 is running.

  - No DRX is configured or configured DRX cycle is longer than 80 ms,

- Or, if at least one of the following conditions is met for BFD,

  - The timer beamFailureDetectionTimer is running.

  - No DRX is configured or configured DRX cycle is longer than 80 ms

Proposal 4: The relaxed RLM/BFD requirements are also not applied to Redcap UE or less than 5 MHz BW UE,

- if at least one of the following conditions is met for RLM

  - The UE sends out-of sync indications to the higher layers,

  - The timer T310 is running.

  - No DRX is configured or configured DRX cycle is longer than 80 ms,

- Or, if at least one of the following conditions is met for BFD,

  - The timer beamFailureDetectionTimer is running.

  - No DRX is configured or configured DRX cycle is longer than 80 ms

About UE capabilities, rlm-Relaxation-r17 and bfd-Relaxation-r17 can be extended to Redcap UE or less than 5 MHz BW UE. Whether to support early implementation or release independence, also need to be discussed in RAN4. We prefer it can start from R17.

Proposal 5: UE capabilities for RLM/BFD relaxation can be extended to Redcap UE or less than 5 MHz BW

UE, but early implementation or release independence issues need to be clarified.

About the relaxed requirements, the parameters like 1 Rx and 2Rx, 48RBs and 24 RBs are needed for Redcap UE for RLM and BFD evaluation. Whether the same level of relaxation for normal UE can apply to UE with 1/2Rx or small BW, need to be discussed and evaluated, for example, minimum requirements of RLM are differentiated for UE with 1Rx or 2Rx.

Proposal 6: Whether the same relaxation of RLM/BFD for normal UE can apply to UE with 1/2Rx or smaller BW, need to be discussed and evaluated

The following tables can be referred for the difference of parameter assumption for BFD for normal UE, Redcap UE and less than 5MHz BW UE.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: The following RRM aspects of R17 RLM/BFD relaxation for legacy UE can be taken as a baseline:

1）	the applicability of RLM measurement relaxation

2）	the relaxed requirements for SSB based radio link monitoring

3）	the relaxed requirements for CSI-RS based radio link monitoring

4）	the applicability of RLM measurement relaxation

5）	the relaxed requirements for SSB based BFD

6）	the relaxed requirements for CSI-RS based BFD

Proposal 2: The relaxed RLM/BFD requirements on PCell is considered for Redcap UE and less than 5 MHz BW.

Proposal 3: Two cases are considered as well for Redcap UE or less than 5 MHz BW UE.

- Case 1: only goodServingCellEvaluationBFD is configured and good serving cell quality criterion is fulfilled and

- Case 2: both goodServingCellEvaluationBFD and lowMobilityEvaluationConnected are configured, and both low mobility criterion and good serving cell quality criterion are fulfilled

Proposal 4: The relaxed RLM/BFD requirements are also not applied to Redcap UE or less than 5 MHz BW UE,

- if at least one of the following conditions is met for RLM

  - The UE sends out-of sync indications to the higher layers,

  - The timer T310 is running.

  - No DRX is configured or configured DRX cycle is longer than 80 ms,

- Or, if at least one of the following conditions is met for BFD,

- The timer beamFailureDetectionTimer is running.

- No DRX is configured or configured DRX cycle is longer than 80 ms

Proposal 5: UE capabilities for RLM/BFD relaxation can be extended to Redcap UE or less than 5 MHz BW

UE, but early implementation or release independence issues need to be clarified.

Proposal 6: Whether the same relaxation of RLM/BFD for normal UE can apply to UE with 1/2Rx or smaller BW, need to be discussed and evaluated.

- Reference

[1]. RP-252955.