TSG-RAN Working Group 4 (Radio) meeting #116bis	  R4-2513116

Prague, Czech Republic, 13 – 17 October 2025

Source:	Ericsson

Title:	System parameter considerations for 6G

Agenda item:	8.3

Document for:	Approval

# Background

This contribution contains observations and proposals for the study of 6G system parameters in RAN4: the spectrum utilization, channel arrangement and guard bands, channel and synchronization raster and device types. RAN4 should collaborate with RAN1 and RAN2 (when appropriate) in these areas already from the start of the study.

The detailed objectives of the 6G study are [1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

# Numerology

For NR, up to three SCS configurations/numerologies are supported for data per operating band, one or two numerologies for synchronisation. The specification of e.g. the nominal CA spacing and the guard bands for aggregated carriers therefore considered support of multiple numerologies, but this specification turned out to be complex and is now subject to different interpretations; thus far the use of multiple numerologies in a band is at best very limited if any.

Proposal 2.1: in collaboration with RAN1, study channel arrangements and minimum requirements based on a reduced number of sub-carrier configurations as compared to NR, aiming at a single numerology for data and synchronisation per operating band except for PRACH and possibly SSB while ‘ensuring appropriate set of functionalities’ (objective of the 6G study) and considering MRSS.

Observation 2.1: a single sub-carrier spacing configuration per operating band or frequency range would simplify specification of the channel arrangement, channel/carrier spacing, spectrum utilization and guard bands for 6GR and specification of MRSS.

In view of the use of the band-dependent numerologies in the field thus far, minimum requirements for 6GR could be specified for 15k for FDD bands (assumed to be below 2.7 GHz), 30k for TDD bands (assumed to be above 1 GHz) and 120k for FR2-1 as a starting point, while not excluding additional numerologies in some ranges e.g. 240k for SSB in   FR2-1 if justified from a performance perspective.

# Spectrum utilization, guard bands and channel arrangement

Spectrum utilization

One of the objectives of the 6GR is

- Improved spectrum utilization and operations taking into account diverse spectrum allocations.

which suggests that the spectrum utilization should also account for e.g. multi-carrier operation.

For NR the spectrum utilization was optimised per channel bandwidth considering the maximum possible PRB allocation. This implied that the internal GB of channels did not increase monotonously with the channel bandwidth, which has consequences for NR deployment of smaller UE-specific CHBW within wider BS CHBW and for intra-band CA of carriers of different CHBW. In some cases, this implies sub-optimal configurations or even BS retuning.

For 6GR we therefore propose

Proposal 3.1: the study of the spectrum utilization and the internal guard bands for channel bandwidths should also consider the possibility of locating a smaller bandwidth (carrier or BWP) within a wider channel bandwidth without restrictions due to internal guard bands.

as illustrated in Figure 3.1; the guard bands should be such that it is possible to locate the smaller CHBW at the band edge without violating the internal GB of the smaller (UE-specific) CHBW. This may also facilitate location of carriers for new dynamic spectrum aggregation schemes and addition of new wider bandwidths for a band.

Figure 3.1: the location of a smaller bandwidth within a wider bandwidth.

The spectrum utilization is not only about increasing spectrum utilization over one carrier according to the agreed TP [2] for TR 38.214

- 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

- 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

However, this must also consider flexibility of locating multiple carriers or bandwidth parts within a wider total bandwidth without restrictions. This also applies to BWPs within 6GR carriers.

Channel bandwidth

For 6GR improvement of the NR CA functionality will be studied such as new intra-band multi-carrier schemes including scenarios with fragmented spectrum allocations in a band. An example of the latter is shown in Figure 3.2 with non-contiguous carriers.

Figure 3.2: a multi-carrier scheme with non-contiguous carriers.

The carriers in Figure 3.2 could also be BWPs, a key concept of NR enabling efficient use of spectrum and power; it is expected that there will be a 6G version. The bandwidths of the multi-carrier arrangement and transmissions from CCs or BWPs within should be such that unwanted emissions can be defined outside the necessary/channel bandwidth of these. In view of the Radio Regulations (RR Appendix 4), we propose

Proposal 3.2: specification of bandwidths for new spectrum aggregation or multi-carrier arrangements and transmissions from e.g. CCs or bandwidth parts within these should be such that unwanted emissions can be specified in accordance with applicable ITU-R recommendations, e.g. the ITU-R Rec. SM.1539 and the ITU-R Rec. SM.1541.

This effectively means that channel bandwidths with appropriate guard bands must be specified for CCs or bandwidth parts within the aggregated bandwidth.

Observation 3.1: for NR, UE-specific channel bandwidths containing the active BWP within a wider BS bandwidth were specified to ensure that unwanted emissions limits outside the wider carrier are always met.

Regarding the channel bandwidth to be specified for 6GR, we propose to study wider bandwidth accommodating multi-carrier operations as shown in Figure 3.2 as well as smaller bandwidth to support lower-complexity devices for 6G from the start:

Proposal 3.3: for the study of channel arrangements and bandwidths in collaboration with RAN1, RAN4 considers bandwidths ranging from 3 MHz for low-complexity devices up to 200 MHz already from the start of the 6G study, support of 400 MHz subject to feasibility.

The FFT size will be determined by the L1 design by RAN1. Table 3.1 lists proposed numerology, maximum- and minimum bandwidths proposed for the 6GR study based on expected FFT sizes and target SCS per band and duplex. The table included target single numerologies for the band ranges, not precluding studies of multiple numerologies for e.g. synchronisation if beneficial in some deployment scenarios.

Table 3.1: proposed numerology, FFT size and bandwidth for the 6GR RAN4 study

|  | FDD | TDD below 7125 MHz | TDD ‘around’ 7 GHz | TDD ‘around’ 15 GHz | TDD above 24 GHz (FR2-1) |
| --- | --- | --- | --- | --- | --- |
| Numerology (SCS) for data1 | 15k | 30k | 30k | 30k or 60k | 120k |
| FFT size | 8k | 8k | 8k | 8k | 4k |
| Maximum CHBW | 100 MHz | 200 MHz | 200 MHz or 400 MHz | 200 MHz or 400 MHz | 400 MHz |
| Minimum CHBW | 3 or 5 MHz | 10 MHz2 | 20 MHz | 20 MHz | 50 MHz |
| NOTE 1: Target values for data except for PRACH, multiple numerologies for e.g. synchronisation should also be studied NOTE 2: 3 or 5 MHz minimum bandwidth for low-complexity devices |  |  |  |  |  |

We not that RAN has tasked RAN1 to study the minimum spectrum allocation and the maximum smallest supported RF and BB bandwidths without spectrum aggregation [3], this should also involve RAN4.

Carrier spacing, contiguous and non-contiguous blocks of carriers

For 6GR, intra-band carrier aggregation is likely to be replaced by new concepts such as multi-carrier transmissions. Definitions of carrier spacing, contiguous and non-contiguous blocks are needed, nevertheless.

Observation 3,2: for new concepts/configurations of carrier aggregation within a band, or across overlapping bands if applicable, e.g. for multi-carrier transmissions, ‘nominal carrier spacing’ must still be specified for a definition of contiguous aggregation as needed for UE capability indication. Similarly for non-contiguous carriers e.g. in fragmented spectrum.

Use of a single FFT for multiple carriers or bandwidth parts should be considered, MRSS aspects in addition since NR and 6GR carriers should be sub-carrier aligned.

For specifying RF requirements on receiver selectivity, the spacing to an adjacent carrier (interferer) must also be specified similarly to the carrier spacing for NR.

# Support of spectrum sharing: MRSS

Spectrum sharing

For support of NR-6GR sharing by MRSS we expect that the NR and 6GR carriers will be sub-carrier aligned. Furthermore, carriers that share spectrum must also meet coexistence requirements with adjacent blocks. This affects both the design of GBs for channel bandwidths and the raster for 6GR carriers.

From a UE perspective, the NR and 6GR carriers would be transparent, at least without DC. Carriers must nevertheless be located such that the NW and the UEs can meet unwanted emissions limits within a spectrum block.

NR and 6GR may also be frame/subframe aligned from a timing reference standpoint and for maintaining orthogonality between collocated RATs.

A 7.5kHz shift in the UL is not needed for since sharing with LTE is not expected in general, unless needed for 6G coexistence with NB-IoT and eMTC via semi-static configuration.

# Channel- and synchronisation raster

NR was designed with an ingenious synchronisation raster of a significantly reduced granularity as compared LTE to reduce cell search complexity but that yet allowed location of the smallest bandwidth (5 MHz for Rel-15) at any 100 kHz channel raster entry for migrated LTE bands for compatibility with legacy systems.

Synchronisation raster

Improvements to the synchronisation raster could still be considered for 6GR to improve energy efficiency e.g. an increased SSB periodicity but without sacrificing NW and UE performance. In view of the SI objective

- Energy efficiency and energy saving: both for network and device.

we therefore propose to

Proposal 5.1: in collaboration with RAN1, study a synchronisation raster that allows an increased SSB periodicity for energy efficiency yet without NW and UE performance degradation as compared to that with a 20 ms periodicity. A dual synchronisation raster, one with a sparse and another with a dense SSB granularity, can be considered if required for energy efficiency and energy saving.

Support of the 3 MHz channel bandwidth should also be considered to this end.

The SCS-based raster is less problematic since this already allows a reduced number of GSCN raster entries.

Furthermore, in Rel-19 NTN work item, a default 160ms periodicity SSB signal is introduced due to the limited number of beams at SAN. However, the feature is introduced late and legacy NR NTN devices before Rel-19 may not find the serving cell if network deploy 160ms periodicity SSB and this causes non-backward compatibility (NBC) issue. It would be beneficial for the NTN feature if the default 160ms periodicity SSB could be considered from day 1 in 6G.

Considering that the same 6G UE may support the NTN and TN at the same time, we propose to align the sync raster design for TN and NTN.

Channel raster

The 6GR channel raster should be specified so to allow compatibility of 6GR channels with legacy channels (up to an including NR) located on the 100 kHz channel raster.

Proposal 5.2: given a synchronisation raster, consider a channel raster that allows 6GR compatibility with adjacent legacy channels on the 100 kHz raster and with due account of MRSS.

It is tempting to adopt the enhanced channel raster with its 10 kHz raster straight away, but it is worth observing that

Observation 5.1: the existing NR synchronisation raster does not allow location of an NR carrier at every possible 10 kHz raster entry of the enhanced channel raster.

The enhanced raster was designed primarily for location of a UE-specific channel bandwidth within a wider BS bandwidth without restrictions.

We assume that the NR-ARFCN raster will be reused for 6GR.

Spectrum sharing

Support of MRSS will add constraints on the channel raster: the 6GR carrier must at least be subcarrier aligned with the NR carrier. We propose that

Proposal 5.3: for spectrum sharing (MRSS), the channel raster for 6GR is designed such that a 6GR channel can be subcarrier aligned with an NR channel.

This does not necessarily mean that the 6GR raster must be aligned with the 100 kHz raster.

# Device types and associated capabilities

The device types to be supported by 6GR will be studied by RAN1 and be confirmed by RAN, RAN4 should also be involved. The design of 6GR should contain elements common to all types and yet be scalable for diverse device types.

RAN4 should study minimum requirements for features introduced in later 5G releases, aimed at specific important use cases that can be expected to be part of 6G from the start, e.g. XR/Time-Critical Communication, massive IoT and FWA, more complicated and less efficient to introduce in later releases.

Proposal 6.1: relating to device types, study performance aspects for low-complexity devices e.g. Massive IoT and devices like XR/TCC and FWA from the start.

For NR, RAN4 has specified minimum requirements ‘intended’ for a specific form factor. One example is the specification of REFSENS for UEs equipped with 4Rx in bands below 1 GHz, requirements apply

- when 4 Rx operation is supported by FWA form factor (on group of selected bands)

- when 4Rx operation is supported by handheld UE (a different group)

The number of Rx antennas is not a UE capability -- the number of supported layers in the DL is – the differentiated requirements for 2Rx and 4Rx cannot be used by the network (disregarding that OTA requirements are more relevant for coverage). Then the number of Rx antennas should have been a capability e.g. for indicating features other than the DL-MIMO layer or SRS AS capabilities that depend on the number of Rx antennas but already have their dedicated capabilities. Another example is the MPR for PC1.5 and 2Tx antennas:

“NOTE 1:	This table is targeted to large FWA form factor with 20 dB or above antenna isolation.”

The difference to UEs with lower than 20 dB antenna isolation is less than 1 dB for this conducted requirement.

Minimum requirements for diverse device types should be mapped to values of UE capabilities associated with the device type, i.e. if a capability is supported/indicated, then specific minimum requirements apply.

Proposal 6.2: in collaboration with RAN1, 6GR RAN4 studies requirements for device types characterised by a possibility to signal associated capabilities are specified similarly to e.g. eRedCap or aerials UEs for NR.

For NR, we note that (e)RedCap and aerial UEs are associated with specific UE capabilities in FR1, while handheld, FWA and Vehicular UEs are associated with different power classes indicated to the NW for FR2.

The “form factor” should not be used unless a device type:

Proposal 6.3: RAN4 minimum requirements do not consider “form factors” without associated UE capabilities (then cannot be distinguished by the NW).

The duplex mode, the antenna arrangements, the bandwidth and the maximum output power supported are expected to be important parameters distinguishing different device types with different UE capabilities from an RF standpoint; see Table 6.1.

Table 6.1: duplex mode, antenna arrangements, bandwidth and power capabilities for possible device types with different UE capabilities

|  | Low-complexity devices Massive IoT | Smart Phones, XR | FWA, Vehicular |
| --- | --- | --- | --- |
| Duplex | FDD/HD-FDD/TDD | FDD/TDD | FDD/TDD |
| Number of antennas (baseline) | 1T/1R or 1T/2R below [4] GHz | 1T/2R or 2T/2R below 1 GHz 2T/4T up to “around” 7 GHz “FR2-like” “around” 15 GHz Same as NR for FR2-1 | 2T/4R below 1 GHz 4T/8R up to “around” 7 GHz “FR2-like” “around” 15 GHz Same as NR for FR2-1 |
| Maximum CHBW | Greater than 5 MHz | Table 3.1 | Table 3.1 |
| Minimum CHBW | 3 or 5 MHz | Table 3.1 | Table 3.1 |
| Power capability | 23 dBm (default) N/A for FR2-1 | 23 dBm for FDD, 26 dBm for TDD (default) PC3 the baseline in FR2-1 | Up to at least 32 dBm (subject to MPE limits) PC4 in FR2-1 |

For 7-15 GHz, the lower end of the range is more FR1-like while the upper end is more FR2-like from an antenna arrangement standpoint.

NTN devices may also be associated with specific capabilities, e.g. in case the antenna arrangements are different.

# Proposal

Numerology

Proposal 2.1: in collaboration with RAN1, study channel arrangements and minimum requirements based on a reduced number of sub-carrier configurations as compared to NR, aiming at a single numerology for data and synchronisation per operating band except for PRACH and possibly SSB while ‘ensuring appropriate set of functionalities’ (objectives of the 6G study) and considering MRSS.

Observation 2.1: a single sub-carrier spacing configuration per operating band or frequency range would simplify specification of the channel arrangement, channel/carrier spacing, spectrum utilization and guard bands for 6GR and specification of MRSS.

Spectrum utilization

Proposal 3.1: the study of the spectrum utilization and the internal guard bands for channel bandwidths should also consider the possibility of locating a smaller bandwidth (carrier or BWP) within a wider channel bandwidth without restrictions due to internal guard bands.

Channel bandwidth

Proposal 3.2: specification of bandwidths for new spectrum aggregation or multi-carrier arrangements and transmissions from e.g. CCs or bandwidth parts within these should be such that unwanted emissions can be specified in accordance with applicable ITU-R recommendations, e.g. the ITU-R Rec. SM.1539 and the ITU-R Rec. SM.1541.

This effectively means that channel bandwidths with appropriate guard bands must be specified for CCs or bandwidth parts within the aggregated bandwidth.

Observation 3.1: for NR, UE-specific channel bandwidths containing the active BWP within a wider BS bandwidth were specified to ensure that unwanted emissions limits outside the wider carrier are always met.

Proposal 3.3: for the study of channel arrangements and bandwidths in collaboration with RAN1, RAN4 considers bandwidths ranging from 3 MHz for low-complexity devices up to 200 MHz already from the start of the 6G study, support of 400 MHz subject to feasibility.

Carrier spacing, contiguous and non-contiguous blocks of carriers

For 6GR, intra-band carrier aggregation is likely to be replaced by new concepts such as multi-carrier transmissions. Definitions of carrier spacing, contiguous and non-contiguous blocks are needed, nevertheless.

Observation 3,2: for new concepts/configurations of carrier aggregation within a band, or across overlapping bands if applicable, e.g. for multi-carrier transmissions, ‘nominal carrier spacing’ must still be specified for a definition of contiguous aggregation as needed for UE capability indication. Similarly for non-contiguous carriers e.g. in fragmented spectrum.

Synchronisation raster

Proposal 5.1: in collaboration with RAN1, study a synchronisation raster that allows an increased SSB periodicity for energy efficiency yet without NW and UE performance degradation as compared to that with a 20 ms periodicity. A dual synchronisation raster, one with a sparse and another with a dense SSB granularity, can be considered if required for energy efficiency and energy saving.

Support of the 3 MHz channel bandwidth should also be considered to this end.

Channel raster

Proposal 5.2: given a synchronisation raster, consider a channel raster that allows 6GR compatibility with adjacent legacy channels on the 100 kHz raster and with due account of MRSS.

It is worth observing that

Observation 5.1: the existing NR synchronisation raster does not allow location of an NR carrier at every possible 10 kHz raster entry of the enhanced channel raster.

Spectrum sharing

Proposal 5.3: for spectrum sharing (MRSS), the channel raster for 6GR is designed such that a 6GR channel can be subcarrier aligned with an NR channel.

This does not necessarily mean that the 6GR raster must be aligned with the 100 kHz raster.

Device types

Proposal 6.1: relating to device types, study performance aspects for low-complexity devices e.g. Massive IoT and devices like XR/TCC and FWA from the start.

Proposal 6.2: in collaboration with RAN1, 6GR RAN4 studies requirements for device types characterised by a possibility to signal associated capabilities are specified similarly to e.g. eRedCap or aerials UEs for NR.

The “form factor” should not be used unless a device type:

Proposal 6.3: RAN4 minimum requirements do not consider “form factors” without associated UE capabilities (then cannot be distinguished by the NW).

# References

1.    RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

2.    RP-252951, “Text proposal on spectrum utilization requirements”, Huawei

3.    RAN#109 MoM