3GPP TSG-RAN WG4 Meeting #116bis				                 R4-2513122

Prague, Czech, Oct. 13-17, 2025

Source:	Sony

Title:	Initial views on system parameters of 6GR

Agenda item:	8.3

Document for:	Approval

# Background

In RAN4#116, the following scope on system parameter design of 6G radio (6GR) discussion in RAN4 was proposed [1]: waveform, modulation, CBW, FFT size, SCS numerology, number of antennas, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. In this contribution, we share our initial views on the topics above.

# Device types

The 6GR design targets a scalable and forward-compatible design for diverse device types [2]. Therefore, it is essential to clarify the device types that the 6GR needs to support before we dive into the detailed system parameters. From RAN4 perspective, each device type may represent a set of common assumptions about UE hardware implementation and can be reflected as a set of UE parameters/capabilities/features with different values, e.g., UE CBW, number of antennae, TX power, etc.

Observation 1: Each device type may represent a set of common assumptions about UE hardware implementation and can be specified as a set of UE parameters/capabilities/features e.g., UE CBW, number of antennae, TX power, etc., with different values

The 6GR supports a diverse range of service classes, from immersive multimedia and massive IoT to integrated communications and sensing [2]. To avoid over fragmentation, the device type definitions in 6GR should consider grouping devices that share similar hardware capabilities or performance requirements. For example, both FWA devices and HMD XR devices may need to offer superior throughput to the end user while facing fewer constraints in terms of the device's form factor, which can be categorized into the same device type.

Proposal 1: Devices that share similar hardware capabilities or performance requirements shall be categorized into the same device types.

With the consideration above, it is proposed to consider the following device type as a starting point and further discuss whether more device types would be needed.

- Massive IoT device (e.g., tracker, wearable devices, meters)

  - Aim for device capability above 4G IoT and below 5G NR (e)Redcap,

  - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap)

- Broadband device with limited formfactor (e.g., smartphone, glass type XR)

  - Improved data rate but limited complexity additional to legacy devices (e.g., 5G eMMB)

  - considering the formfactor limitation when determining the device parameters/capabilities

- Ultra Broadband devices (e.g., FWA/CPE, HMD XR)

  - focus on fulfilling the extraordinary performance bounds

  - no major constraints due to device formfactor but with realistic implementation assumption

Proposal 2: Considering 1) Massive IoT device, 2) Broadband device with limited formfactor e.g., smartphone, 3) Ultra Broadband devices (e.g., FWA/CPE, XR) as starting point to define, further discuss if more device types need to be defined in 6GR.

The detailed parameters/capabilities of those devices are listed in Table I below. We further elaborate on the motivation behind our proposed values and parameters in later sections of the paper.

Table I. 6GR device type and associated device capability

|  | Massive IoT device, e.g., tracker, wearable devices, meters | Broadband device with formfactor limitations, e.g., smartphone, glass type XR devices | Ultra broadband device e.g., FWA/CPE, Vehicle, HMD type XR devices. |
| --- | --- | --- | --- |
| Design considerations | - Aim for device capability above 4G IoT and below 5G NR, - with reduced device complexity than legacy devices (e.g., 5G (e)Redcap) | - Improved data rate but limited complexity additional to legacy devices (e.g., 5G eMMB) - consider the formfactor limitation when determining the device parameters/capabilities | - focus on fulfilling the extraordinary performance bounds - no major constraints due to device formfactor but with realistic implementation assumption |
| Frequency range | Focus on frequency bands < 3 or 4 GHz | FR1, FR2-1, “FR3” | FR1, FR2-1, “FR3” |
| Minimum UE CBW | 3MHz | Supporting 3MHz in FR1 | Supporting 3MHz in FR1 |
| Maximum UE CBW (single CC) | UL: 3MHz/ DL: TBD | - 100 MHz in FR1 (same as 5G NR) - 200 MHz in FR3 - 400 MHz in FR2-1(same as 5G NR) | - 200 MHz in FR1 - 400 MHz in FR3 - 400 MHz in FR2-1 |
| Duplex mode | Focus on HD-FDD, FD-FDD/TDD can be supported | FDD/TDD | FDD/TDD |
| Number of antennas | 1T/1R | FR1&FR2-1 antenna assumption to be the same as in 5G NR, 2T/4R in FR3. | 2T/8R in FR1 and FR3 FR2-1 antenna assumption to be the same as in 5G NR. |
| Default power class | 23 dBm | 23 dBm in FR1 26 dBm in FR3 Considering enhancement on PC3 in FR2-1 | 26 dBm in FR1 31 dBm in FR3 PC4 in FR2-1 |
| Mobility | Idle mode mobility as baseline | Connected mode mobility | Connected mode mobility |
| Maximum Modulation order | 64QAM UL/64QAM DL | 256 QAM UL/1024 QAM DL in all frequency ranges | 512 QAM UL/1024 QAM DL in all frequency ranges |

# UE CBW

UE CBW is a critical parameter that not only affects the cost of the baseband but also the RF design. The key design consideration is usually the minimum and maximum UE CBW that a device needs to support.

For the minimum UE CBW, based on the lessons learned from 5G NR, the demand for 3MHz to allow the deployment of 6GR in narrower spectrum allocation would still be present. Meanwhile, we don't expect major impact on supporting the minimum bandwidth from the UE implementation aspect, and thus it should be feasible for all device types to support and operate with 3MHz as the minimum UE CBW. Therefore, for all device types in 6GR, a minimum UE CBW of 3MHz CBW should be supported.

Proposal 3: For all device types in 6GR, they should be able to support a minimum UE CBW of 3MHz in FR1.

On the other hand, the maximum UE CBW affects UE implementations in both RF and BB domains. Therefore, we consider that different UE capabilities shall be defined as part of the scalable design of 6G, where different device types may have different maximum UE CBW.

For massive IoT devices, a typical maximum UE CBW currently considered would be in the range of 3-5 MHz [3]. Within this range, a 3MHz UE CBW, especially in the uplink direction, can enable a true SAW-less design for 6G massive IoT devices, thanks to a narrower spectrum shape than 5MHz [4]. The implementation of the SAW-less device is shown in Fig. 1. SAW-less HD-FDD design is a key consideration for reducing RF domain complexity compared to (e)Redcap in 5G NR [4][5].With a fully SAW-less HD-FDD design:

- A true single SKU design for global operation is possible, allowing for true economies of scale.

- Additionally, a fully SAW-less HD-FDD design can achieve a reduced chip footprint and a smaller IoT module.

- Last but not least, the insertion loss on Tx and Rx chain can be reduced, and the power efficiency of the system can be enhanced.

A similar design principle has been widely adopted in 4G IoT, e.g., cat-M1 and NB-IoT, in commercial production, thanks to 4G-IoT's even smaller CBW. However, it is not the case in 5G NR as larger BW needs to be supported, even for (e)Redcap devices, which is one of the factors that creates a significant implementation feasibility difference between 4G IoT and 5G (e)Redcap devices. The UL CBW is particularly critical here, as SAW filters are usually required to suppress high-order intermodulation distortion (IMD) products to meet the spurious emission limits defined either in 3GPP or by regulators. Therefore, a slightly reduced bandwidth compared to 5G (e.g., 3 MHz or allowable output power backoff in a challenging scenario) would be required in the 6G design to enable a fully SAW-less HD-FDD implementation.

Fig. 1. Comparison of massive IoT device with SAW filter/duplexer design vs a fully SAW-less design.

From a DL perspective, though, it mainly affects digital baseband design from the UE implementation perspective. A balance between device complexity and potential impact on other device types needs to be considered. Different UL and DL UE CBW should also be considered as part of the 6GR design.

Proposal 4: For massive IoT devices, consider 3 MHz as the maximum UE CBW in UL. Further discuss the DL CBW, considering device complexity and its potential impact on other device types.

Proposal 5: The 6GR design considers the case where the UL UE CBW is different to the DL UE CBW.

For broadband and ultra-broadband devices, a larger single CC UE CBW can be considered for 6G, supporting higher data rates in the upper part of FR1 and FR3, compared to 5G. Having said that, it is proposed that the maximum UE CBW with single CC for broadband devices is 100 MHz on FR1, 200 MHz in FR3 and 400 MHz in FR2-1. For ultra-broadband devices, it can be 200 MHz in FR1, 400 MHz in FR3, and 400 MHz in FR2-1 in 6GR.

Proposal 6: It is proposed that the maximum UE CBW with a single CC for broadband devices be 100 MHz in FR1, 200 MHz in FR3, and 400 MHz in FR2-1. For ultra-broadband devices, it can be 200 MHz in FR1, 400 MHz in FR3, while keeping 400 MHz in FR2-1.

# Waveform

In RAN1#122, it has been agreed that CP-OFDM and DFT-s-OFDM, which have been used in 5G NR, will also serve as the basis for 6G. Adopting the same waveform as 5G NR in 6GR would help to reduce the R&D complexity for 6G devices, accelerate the time to market and facilitate 5G-6G coexistence. Therefore, for RAN4 aspects, CP-OFDM and DFT-s-OFDM should also be considered as basis for 6G.

Proposal 7: Adopt the same waveform as 5G NR for 6GR design as the basis.

To further improve the network coverage, especially DL coverage, DFT-s-OFDM can also be considered in the DL direction. We expect this feature can be useful at least in scenarios like NTN or massive IoT with deep indoor deployment. In addition, a new waveform that can further improve the DL/UL link budget, e.g., lower PAPR, can also be further studied and considered if it does not require major hardware changes.

Proposal 8: Further study whether DFT-s-OFDM can be supported in DL as well to improve the network coverage.

From a sensing aspect, it is proposed to study the sensing performance over existing waveforms and potential new waveforms and further discuss whether a new waveform would be needed to support the sensing feature in 6GR.

Proposal 9: Further study whether a new waveform would be needed for sensing in 6GR.

# Modulation

Modulation is also one of the key parameters that needs to be considered under a scalable design for diverse device types.

- For massive IoT devices, considering the device complexity and the demand spectral efficiency of IoT deployment, the modulation order needs to be carefully selected to meet the target data rate without adding additional complexity to device implementations. Assuming a target data rate for massive IoT in 6GR would be in the range between 5-10 Mbps,with a 3MHz UE CBW in the UL direction, it can support a peak data rate near 10 Mbps with 64QAM (assuming 5G NR MCS). To support a higher modulation order, it may require improved linearity in the device transmitter and a more advanced receiver in the DL direction, which may add additional complexity for IoT devices. Thus, it is proposed that 6G massive IoT supports up to 64 QAM.

- For broadband devices, it is proposed to adopt a similar modulation order as in 5G NR, with 256 QAM UL/1024 QAM DL in all frequency ranges. A higher modulation order can be considered if a clear demand for a higher data rate and hardware feasibility can be identified.

- For ultra-broadband devices, e.g., FWA/CPE devices, due to the demand for higher data rates and the possibility of supporting higher-performing transceiver designs with less constrained power consumption and form factors, it is proposed to consider 512 QAM UL/1024 QAM DL in all frequency ranges.

Proposal 10: It is proposed that 6G massive IoT can support up to 64 QAM. For broadband and ultra broadband devices, 256 QAM UL/1024 QAM DL and 512 QAM UL/1024 QAM can be considered in 6GR, respectively.

# FFT size

For massive IoT devices, it is considered that the bandwidth required to support them would be relatively small compared to broadband devices. This allows the same FFT assumption as legacy IoT devices, which would help reduce the R&D complexity of new 6G massive IoT devices and enable some hardware reuse. Therefore, 2K or 4K FFT size can be assumed for massive IoT devices.

For broadband and ultra-broadband devices, to support the enlarged UE CBW mentioned above, it may be necessary to consider a larger FFT size than 5G NR. Therefore, it is proposed to consider an 8K FFT size as a baseline for further study.

Proposal 11: It is proposed to assume a 2k or 4k FFT for massive IoT devices, while an 8k or even larger FFT size is considered for higher-capability device types, pending the feasibility study.

# SCS Numerology

In 5G NR, multiple numerologies were defined for each frequency band to offer sufficient flexibility for different usage scenarios. However, in practice, only a limited set of numerologies was deployed, and typically a single fixed numerology per band has been deployed. Supporting multiple numerologies in 5G NR places additional complexity in both device and network implementation, but without improving the actual field performance. Therefore, in 6GR, it is proposed to consider only a single numerology per band, e.g., a single SCS for each frequency band.

Proposal 12: It is proposed that 6GR adopts single numerology per frequency band.

Typically, the SCS is selected based on the frequency, with a larger SCS being used for higher frequencies to mitigate the increased phase noise. As a starting point, it is proposed that 6GR shall adopt 15 kHz SCS for frequency bands below 3 GHz and 30 kHz for frequency bands above 3 GHz in FR1, and further study the appropriate numerology on other frequency ranges.

Proposal 13: It is proposed that 6GR should adopt 15 kHz SCS for frequency bands below 3 GHz and 30 kHz for frequency bands above 3 GHz in FR1, and further study the appropriate numerology for other frequency ranges.

# Number of Antennas

For Massive IoT devices, considering the device complexity and possible form factor limitations for some use cases, such as wearable devices and smaller trackers, it is proposed to adopt 1T/1R as the baseline.

Proposal 14: For massive IoT devices, it is proposed to adopt 1T/1R as a baseline.

For broadband devices with form factor limitations, such as smartphones, their form factor is unlikely to change in the 6G era. Therefore, it is proposed to retain the same number of antennas as in 5G NR as the starting point in FR1. For FR3, it is proposed to adopt 2T/4R as the starting point and allow companies to investigate whether a larger number of antennas can be fitted or needed. In FR2-1, a device with two antenna panels, each with 4*1 antenna elements, can be used as a baseline to define the spherical coverage without compromising the minimum requirements of single-panel implementations.

Proposal 15: For broadband devices with form factor limitations, like smartphones, it is proposed to retain the same number of antennas as in 5G NR as the starting point for FR1. For FR3, it is proposed to adopt 2T/4R as the starting point. In FR2-1, a device with two antenna panels, each with 4*1 antenna elements, can be used as a baseline to define the spherical coverage without compromising the minimum requirements of single-panel implementations.

For ultra broadband devices, due to higher demand on the device performance and fewer constraints on the device's form factor, it is proposed to adopt 2T/8R in FR1 and FR3, while for FR2-1, three antenna panels with at least 4*1 antenna elements can be used as a baseline assumption to define the minimum requirements of the device type.

Proposal 16: For ultra broadband devices, it is proposed to adopt 2T/8R in FR1 and FR3, while for FR2-1, three antenna panels with at least 4*1 antenna elements can be used as a baseline assumption to define the minimum requirements of the device type.

# Channel Raster

An enhanced channel raster with 10 kHz was introduced in the later stages of 5G NR to offer more efficient spectrum usage. The channel raster should not significantly impact the UE hardware implementation. On the other hand, it needs to be considered whether a 10 kHz channel raster would affect coexistence with 5G NR or LTE systems, as many legacy systems still operate with a 100 kHz channel raster. Therefore, we propose that 6G consider the 10 kHz channel raster from the beginning to ensure that the spectrum usage of 6G can be more efficient. Meanwhile, further study is needed to determine if different channel rasters between 5G NR/LTE and 6GR would cause any issues in terms of MRSS and/or in-band coexistence.

Proposal 17: It is proposed 6G consider the 10 kHz channel raster from the beginning to ensure the spectrum usage of 6G can be more efficient than 5G. Meanwhile, further study if a different channel raster between 5G NR/LTE and 6GR would cause any issue in terms of MRSS and/or in-band coexistence

# Coexistence between 6GR and legacy RATs

For legacy RATs, especially legacy IoT services, considering their longevity, it is necessary to ensure coexistence between legacy RATs, such as 4G IoT and 6GR, as it can be foreseen that these RATs will coexist in the field for at least another decade. It has been agreed in RAN#109 that 6GR should support semi-static coexistence between 6GR and 4G IoTs, including NB-IoT and cat-M1. Therefore, from the RAN4 side, it is proposed to study how to ensure the coexistence between 6GR and 4G IoTs.

Proposal 18: It is proposed that RAN4 should study how to ensure the coexistence between 6GR and 4G IoTs.

# Conclusion

In this contribution, we share our initial view on the system design of 6GR from the RAN4 perspective, where the following observations and proposals are given:

Observation 1: Each device type may represent a set of common assumptions about UE hardware implementation and can be specified as a set of UE parameters/capabilities/features e.g., UE CBW, number of antennae, TX power, etc., with different values

Proposal 1: Devices that share similar hardware capabilities or performance requirements shall be categorized into the same device types.

Proposal 2: Considering 1) Massive IoT device, 2) Broadband device with limited formfactor e.g., smartphone, 3) Ultra Broadband devices (e.g., FWA/CPE, XR) as starting point to define, further discuss if more device types need to be defined in 6GR.

Proposal 3: For all device types in 6GR, they should be able to support a minimum UE CBW of 3MHz in FR1.

Proposal 4: For massive IoT devices, consider 3 MHz as the maximum UE CBW in UL. Further discuss the DL CBW, considering device complexity and its potential impact on other device types.

Proposal 5: Different UL and DL UE CBW can also be considered in the 6GR design.

Proposal 6: It is proposed that the maximum UE CBW with a single CC for broadband devices be 100 MHz in FR1, 200 MHz in FR3, and 400 MHz in FR2-1. For ultra-broadband devices, it can be 200 MHz in FR1, 400 MHz in FR3, while keeping 400 MHz in FR2-1.

Proposal 7: Adopting the same waveform as 5G NR for 6GR design as the basis.

Proposal 8: Further study whether DFT-s-OFDM can be supported in DL as well to improve the network coverage.

Proposal 9: Further study whether a new waveform would be needed for sensing in 6GR.

Proposal 10: It is proposed that 6G massive IoT can support up to 64 QAM. For broadband and ultra broadband devices, 256 QAM UL/1024 QAM DL and 512 QAM UL/1024 QAM can be considered in 6GR, respectively.

Proposal 11: It is proposed to assume a 2k or 4k FFT for massive IoT devices, while an 8k or even larger FFT size is considered for higher-capability device types, pending the feasibility study.

Proposal 12: It is proposed that 6GR adopt single numerology per frequency band.

Proposal 13: It is proposed that 6GR should adopt 15 kHz SCS for frequency bands below 3 GHz and 30 kHz for frequency bands above 3 GHz in FR1, and further study the proper numerology for other frequency ranges.

Proposal 14: For massive IoT devices, it is proposed to adopt 1T/1R as a baseline.

Proposal 15: For broadband devices with form factor limitations, like smartphones, it is proposed to retain the same number of antennas as in 5G NR as the starting point for FR1. For FR3, it is proposed to adopt 2T/4R as the starting point. In FR2-1, a device with two antenna panels, each with 4*1 antenna elements, can be used as a baseline to define the spherical coverage without compromising the minimum requirements of single-panel implementations.

Proposal 16: For ultra broadband devices, it is proposed to adopt 2T/8R in FR1 and FR3, while FR2-1, three antenna panels with at least 4*1 antenna elements can be used as a baseline assumption to define the minimum requirements of the device type.

Proposal 17: It is proposed 6G consider the 10 kHz channel raster from the beginning to ensure the spectrum usage of 6G can be more efficient. Meanwhile, further study if different channel raster between 5G NR/LTE and 6GR would cause any issue in terms of MRSS and/or in-band coexistence

Proposal 18: It is proposed that RAN4 should study how to ensure the coexistence between 6GR and 4G IoTs.

# References

- R4-2511652		Plan on RAN4 6G discussion and meeting arrangement in Rel-20	RAN4 Chair (Apple)

- RP-252912		Revised SID: Study on 6G Radio		NTT DOCOMO, CMCC, AT&T, Vodafone.

- RP-251951		Minimum requirements for Massive Communication for IoT	Ericsson, AT&T, BT, CHTTL, Deutsche Telekom, FirstNet, Fujitsu, Intel Corporation, KDDI, KT Corp., NTT DOCOMO, OPPO, Semtech, Sequans, SK Telecom, Sony, Spark NZ, Telstra, Thales, T-Mobile USA, Verizon, Vodafone

- RP-252119		6G Massive IoT Requirements	Sony Europe Limited, Nordic Semiconductor ASA

- RP-252327		On 3GPP Support of Massive IoT in 6G	AT&T, Ericsson, FirstNet, KT Corp., MediaTek Inc., Nokia, Nordic Semi, NTT DOCOMO, Qualcomm, Rakuten, Sequans, Spark NZ, SONY, T-Mobile USA, Verizon