3GPP TSG-RAN WG4 Meeting #116bis				                   R4-2513124

Prague, Czech, Oct. 13-17, 2025

Source:	Sony

Title:	RRM consideration on 6G Massive IoT

Agenda item:	8.7

Document for:	Approval

# Background

In RAN4#116, the following scope for the RRM of 6G was proposed [1]: RAN4-driven non-AI RRM topics, MIMO operation, mobility, initial access, spectrum aggregation, multi-TRP, RRM-related coverage and energy efficiency, and other PHY signal/channel/procedure-related topics.

We shared our discussion of device types in our companion paper [2], where we highlight the need for a 6G massive IoT device type. In this contribution, we share some initial thoughts on the RRM design related to 6G massive IoT.

# Discussion

For 6GR design, it targets a scalable and forward-compatible design for diverse device types [3]. In particular, the massive IoT device is among the devices that will be supported in 6GR. It is likely that 6G massive IoT will become the device type with the lowest capability among all devices, which can further determine a common but minimum set of features for 6GR.

The 6G massive IoT aims to achieve reduced complexity and improved energy efficiency compared to legacy IoT devices, thereby enhancing the economy of scale and sustainability of the 6G system. To achieve this goal, RRM is a critical part that needs to be considered in the 6G era for massive IoT devices. Based on lessons learned from legacy IoT devices, a significant portion of the memory footprint of a cellular IoT chipset is occupied by storing measurement results from RRM, and frequent searching/measurement/reporting consumes a significant amount of power for an IoT device.

Observation 1: For legacy IoT devices, a significant part of the memory footprint is occupied by storing measurement results, and considerable power consumption is incurred during searching/measurement/reporting.

In our view, some further simplifications in 6GR RRM design can be considered to reduce the complexity and power consumption of massive IoT devices. Taking mobility as an example, many massive IoT devices operate in stationary cases, such as surveillance cameras and smart meters. Therefore, considering measurements (e.g., cell searching) and reporting reductions for such types of devices could be studied.

In addition, even in a mobile scenario, some use cases may not even require seamless handover. In those cases, connected mode mobility may not even be required, and Idle mode mobility can be taken as a baseline. Similar design considerations were taken into account for the early releases of NB-IoT.

Observation 2: For stationary use cases or mobile use cases that do not require seamless handover, measurements and reporting relaxations need to be studied, and Idle mode mobility can be considered as a baseline.

Proposal 1: RAN4 should study the RRM relaxation and simplification for 6G massive IoT, comparing it with legacy IoT devices, to reduce device complexity and improve network/device energy efficiency.

From an energy-saving perspective, we see the benefit of supporting legacy power-saving mechanisms in 6GR as well, for example, PSM/MICO, eDRX, LP-WUS, and others. Some other possible directions here could include, for example, allowing more RRM measurements to be offloaded to a low-power receiver (LR), to enable more energy savings.

Proposal 2: RAN4 shall study the power-saving mechanism from the RRM perspective, including legacy power-saving techniques, and can further investigate whether more RRM measurements can be offloaded to LR.

Other potential techniques to achieve complexity reduction from an RAN4 RRM perspective include reducing the number of measurement gap patterns. Currently, over 20 measurement gap patterns are defined in the RAN4 spec, but only a limited set of these patterns are commonly used in the field, as we understand it. Therefore, for massive IoT devices or all device types, a reduced number of measurement gap patterns could provide further benefit in reducing device complexity. Another direction to consider could be to have a more unified measurement framework to reduce redundant measurement results.

Proposal 3: RAN4 can also study methods to reduce the number of measurement gap patterns and eliminate possible redundant measurement results if identified.

# Conclusion

In this contribution, we share some intial thoughts and directions for RRM design in 6GR, particularly concerned to 6G massive IoT device type. The following observations and proposals are given:

Observation 1: For legacy IoT devices, a significant part of the memory footprint is occupied by storing measurement results, and considerable power consumption is incurred during searching/measurement/reporting.

Observation 2: For stationary use cases or mobile use cases that do not require seamless handover, measurements and reporting reductions need to be studied, and Idle mode mobility can be considered as a baseline.

Proposal 1: RAN4 should study the RRM relaxation and simplification for 6G massive IoT, comparing it with legacy IoT devices, to reduce device complexity and improve network/device energy efficiency.

Proposal 2: RAN4 shall study the power-saving mechanism from the RRM perspective, including legacy power-saving techniques, and can further investigate whether more RRM measurements can be offloaded to LR.

Proposal 3: RAN4 can also study methods to reduce the number of measurement gap patterns and eliminate possible redundant measurement results if identified.

# References

- R4-2511652		Plan on RAN4 6G discussion and meeting arrangement in Rel-20	RAN4 Chair (Apple)

- R4-25xxxxx		Initial views on 6GR system parameters	Sony

- RP-252912		Revised SID: Study on 6G Radio		NTT DOCOMO, CMCC, AT&T, Vodafone.