# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513125

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	8.9

Source: 	CMCC

Title: 			 Discussion on AI/ML for 6GR study

Document for:	Discussion

- Introduction

In RAN #108 meeting, the SID on 6G radio was approved [1]. The objectives on AI/ML are duplicated as following.

- (8)AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In last RAN4 meeting, there was discussion about RAN4 6G topic planning based on 6G SID, and RAN4 6G plan and meeting arrangement was approved [2]. This meeting is RAN4 first meeting to discuss 6GR. This contribution provides discussion on RAN4 driven use cases for AI/ML.

- Discussion

## 2.1 Review on AI/ML in 5G-A

There are two WIs on AI/ML in 5G-A, one is AI/ML for NR air interface, the other one is AI/ML for mobility.

AI/ML for NR air interface includes beam management, positioning, CSI prediction, CSI compression. For beam management, BM-Case1 and BM-Case2 are considered. BM-Case1 is spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams. BM-Case2 is temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams. For positioning accuracy enhancements, both direct AI/ML positioning and AI/ML assisted positioning are considered. Direct AI/ML positioning include case 1: UE-based positioning with UE-side model and case 3b: NG-RAN node assisted positioning with LMF-side model. AI/ML assisted positioning include case 3a: NG-RAN node assisted positioning with gNB-side model. CSI prediction is time domain CSI prediction using UE-side model. Rel-18 study phase [3] mainly focus on evaluating performance benefits of AI/ML based algorithms for above use cases, accessing potential specification impact including PHY layer aspects, protocol aspects, and interoperability and testability aspects. The main work of Rel-19 WI [4] is specification support for beam management, positioning, CSI prediction, including signaling and protocol aspects of Life Cycle Management (LCM), enabling method(s) to ensure consistency between training and inference, RAN4 core requirements definition. The study on testability and interoperability for CSI compression is continued in Rel-19 [5]. Rel-20 WI [6] targets to provide specification support for CSI compression, including necessary signaling/mechanism(s) specification, inter-vendor training collaboration, core requirements definition, etc.

Besides, use case about CSI compression with two sided model is studied, mainly about testability and interoperability in Rel-19. According to Rel-20 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, the normative support for the CSI feedback enhancement (two-sided model) use case as well as standards-based UE data collection are considered.

Observation 1: the use case of AI/ML for NR air interface in 5G-A includes beam management, positioning, CSI prediction, CSI compression.

As discussed above, AI/ML based beam management includes BM-Case1 and BM-Case2. BM-Case1 is spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams. BM-Case2 is temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams. It can be seen that, for both BM-Case1 and BM-Case2, the target is DL Tx beam prediction.

Observation 2: Beam prediction in 5G-A focus on DL Tx beam prediction.

The output of AI/ML model for beam prediction is L1-RSRP and/or beam ID. In Rel-19, there is discussion on whether RX beam knowledge, i.e. whether RX beam corresponding to the predicted TX beam, is known. The agreements are as following:

- If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known.

- If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

Since this is for L1 level beam prediction, the main spec impact is TCI state known condition. The TCI state known condition need to be updated to capture the AI/ML case.

Observation 3: The output of AI/ML model for beam prediction is L1-RSRP and/or beam ID.

Observation 4: the discussion on RX beam knowledge in 5G-A is about whether RX beam corresponding to the predicted TX beam is known, and the discussion is about L1 level beam, the main spec impact is TCI state known condition.

For AI/ML for mobility, Rel-19 study [7] focus on RRM measurement prediction and measurement event prediction. Even though there are 6 scenarios for RRM measurement prediction, scenario 2, 3, 4 are prioritized.

Table 1 Evaluation scenarios for RRM measurement prediction

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Scenario 4 is FR2 intra-frequency temporal domain case A (i.e., prediction of measurements in a prediction window by using historical actual measurements in an observation window). Scenario 2 is FR1 intra-frequency temporal domain case B (similar to case A, but predicted measurement result(s) in previous prediction window are skipped). And scenario 3 is FR1 inter-frequency prediction. The study on temporal domain case A is to verify performance improvement, i.e. how much handover performance can be improved based on predicted measurement results/event in future. The motivation of study on temporal domain case B is about measurement reduction, i.e. whether the degradation of handover performance is acceptable while partial measurement is skipped. The target of Rel-20 WI is specification support of AI/ML assisted mobility in RRC_CONNECTED mode, both RRM measurement prediction and measurement event prediction are considered.

Observation 5: the use case of AI/ML for mobility in 5G-A includes RRM measurement prediction and measurement event prediction.

As shown in Table 1, spatial domain prediction (scenario #6) is in middle priority for Rel-19 study. And, from RAN4 side, there is little study on this scenario. According to Rel-20 WID [8], for UE sided model, spatial domain prediction and L3 beam level prediction are not supported. Rel-20 WI focus on temporal domain prediction case A and case B, and frequency domain prediction for co-located cases. And L3 Cell level predication is supported.

Observation 6: AI/ML for mobility in 5G-A focus on temporal domain prediction case A and case B, and frequency domain prediction for co-located cases.

Observation 7: for AI/ML for mobility in 5G-A, for UE sided model, spatial domain prediction is not supported.

## 2.2 Consideration on RAN4 driven use case for AI/ML in 6GR

### 2.2.1 Use case on AI/ML based UE RX beam prediction

One potential RAN4 driven use case for AI/ML in 6GR is AI/ML based UE RX beam prediction. The motivation is the FR2 RRM requirements allow excessively long delay with RX beam sweeping factor in NR. Taking cell re-selection as an example, the related delay requirements are duplicated as following. For DRX cycle length of 0.32s, the Tdetect for FR2-1 with 20ms SMTC periodicity is about 138 seconds. Even with UE velocity of 30km/h， it moves about 1150 meters, which is not good for mobility, especially for FR2 deployment with limited cell coverage. The main contributor for long delay is the scaling factor (N1), which could be up to 8 for FR2-1, and even higher, e.g. 12, for FR2-2.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 8 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 6 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 5 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. NOTE 3: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. |  |  |  |  |  |  |

5G spectrum includes FR1(410MHz - 6.425 GHz) and FR2 (24.25-52.6GHz). Candidate 6G new spectrum is about 7 to 24 GHz. It can be expected that beam sweeping will play an important role for high frequency range.

UE RX beam prediction is not considered in 5G-A. As discussed in section 2.1, for AI/ML for air interface, beam prediction in 5G-A focus on DL Tx beam prediction. The discussion on RX beam knowledge in 5G-A is about whether RX beam corresponding to the predicted TX beam is known, and the discussion is about L1 level beam, the main spec impact is TCI state known condition. For AI/ML for mobility, the focus is temporal domain prediction case A and case B, and frequency domain prediction for co-located cases. Spatial domain prediction is not supported.

However, we believe DL RX beam prediction is also feasible. For example, spatial-domain DL Rx beam prediction for Set A of beams can be performed based on measurement results of Set B of beams, which means UE no need to measure all the candidate RS, the measurement to determine the best RX beam can be reduced. In this case, the RX beam sweeping factor can be reduced.

With AI/ML based UE RX beam prediction, it is expected that RX beam sweeping factor could be reduced. With reduced RX beam sweeping factor, cell identification delay, including PSS/SSS detection, measurement period, SSB index detection, SCell activation delay, handover delay, PSCell addition delay, CGI identification delay, RRC Re-establishment/RRC Connection Release with Redirection delay can be reduced, which is beneficial for system.

Proposal 1: it is proposed to consider the use case of AI/ML based UE RX beam prediction, to reduce or eliminate RX beam sweeping factor.

### 2.2.2 Use case on AI/ML based frequency domain measurement prediction for non co-located scenario

Another consideration is AI/ML based frequency domain measurement prediction for non co-located scenario. The motivation is to reduce MG configuration in practice.

As discussed in section 2.1, frequency domain prediction is considered in 5G-A AI/ML for mobility, but only co-located scenario is considered. In this case, the impact on MG configuration is limited. Network has to configure measurement gap even if there is only one MO which needs MG. As duplicated as following, there are many conditions under which the measurement is considered as inter-frequency SSB based measurements without measurement gaps. For inter-frequency measurement, the common case is that measurement gap is needed. Only consider co-located scenario, the actual help in reducing measurement gap configuration is limited. It is necessary to study the AI/ML based frequency domain measurement prediction for non co-located scenario.

TS 38.133 A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15]. Besides the conditions listed above, - for UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for inter-frequency measurement, - An inter-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE - For inter-frequency SSB based measurements without MG and NCSG, UE may cause scheduling restriction as specified in clause 9.3.9.4. - An inter-frequency SSB measurement is defined as measurement with NCSG if - the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE - For inter-frequency SSB based measurements with NCSG, UE may cause scheduling restriction as specified in clause 9.3.10.3. - An inter-frequency SSB measurement is defined as measurement with gap if - the UE indicates ‘gap’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE - for UE supporting nr-NeedForInterruptionReport-r18 and indicating NeedForInterruptionInfoNR for inter-frequency measurement, - An inter-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘no-gap’ via NeedForGapsInfoNR and the UE indicates ‘no-gap-no-interruption’ or no-gap-with-interruption via NeedForInterruptionInfoNR-r18 for the inter-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE The interruption requirement during inter-frequency measurement without gap is defined in clause 8.2.2.2.19 when UE indicate no-gap-with-interruption. No interruption is allowed due to inter-frequency measurement without gap when following conditions are met: - the SSB is completely contained in the active BWP of the UE, or - UE indicates “no-gap-no-interruption”, or - UE indicates “no-gap-with-interruption” but inter-frequency SMTC is partially or fully overlapping with measurement gaps, During inter-frequency SSB based measurements without gap, UE may cause scheduling restriction as specified in clause 9.3.9.4. - An inter-frequency SSB measurement is defined as measurement with gap if - the UE indicates ‘gap’ via NeedForGap-InfoNR for the inter-frequency measurement.

Proposal 2: it is proposed to consider the use case of AI/ML based frequency domain measurement prediction for non-co-located scenario, to reduce the the usage of measurement gap.

### 2.2.3 Use case on AI/ML based non-linearity compensation

According to the Rel-20 RAN4 WID on UE RF enhancements for NR FR1 [9], one of the objectives is power enhancement with relaxed UE Tx EVM and advanced BS receiver.

Rel-20 WID on UE RF enhancements for NR FR1 (RP-252942) Power enhancement with relaxed UE Tx EVM and advanced BS receiver - Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

Relaxed UE Tx EVM and advanced BS receiver is beneficial for system performance. With the relaxation of Tx EVM requirements for 64QAM and 256QAM and advanced BS receiver, the enhancement of UE output power can be achieved and the uplink coverage for high modulation order can be improved accordingly. It is proposed to consider Relaxed UE Tx EVM and advanced BS receiver in 6GR.

As shown in the note of the WID, for 5G-Adv, solution should not be AI based. However, AI/ML is another approach to enable relaxed UE Tx EVM, and can be considered in 6GR.

In last RAN1 discussion, there are proposals on study on AI/ML for PA non-linearity handling. However, this use case can also be considered in RAN4 from the perspective of RF requirements considering that PA non-linearity has impact on EVM, MPR, etc. With AI/ML based PA non-linearity handling at UE side, higher TX power and higher modulation can be expected.

Proposal 3: it is proposed to consider the use case of AI/ML based PA non-linearity handling.

- Conclusion

This contribution provides discussion on RAN4 driven use cases for AI/ML. The observations and proposals are:

Proposal 1: it is proposed to consider the use case of AI/ML based UE RX beam prediction, to reduce or eliminate RX beam sweeping factor.

Proposal 2: it is proposed to consider the use case of AI/ML based frequency domain measurement prediction for non-co-located scenario, to reduce the the usage of measurement gap.

Proposal 3: it is proposed to consider the use case of AI/ML based PA non-linearity handling.

Observation 1: the use case of AI/ML for NR air interface in 5G-A includes beam management, positioning, CSI prediction, CSI compression.

Observation 2: Beam prediction in 5G-A focus on DL Tx beam prediction.

Observation 3: The output of AI/ML model for beam prediction is L1-RSRP and/or beam ID.

Observation 4: the discussion on RX beam knowledge in 5G-A is about whether RX beam corresponding to the predicted TX beam is known, and the discussion is about L1 level beam, the main spec impact is TCI state known condition.

Observation 5: the use case of AI/ML for mobility in 5G-A includes RRM measurement prediction and measurement event prediction.

Observation 6: AI/ML for mobility in 5G-A focus on temporal domain prediction case A and case B, and frequency domain prediction for co-located cases.

Observation 7: for AI/ML for mobility in 5G-A, for UE sided model, spatial domain prediction is not supported.

- Reference

- RP-251881, New SID: Study on 6G Radio

- R4-2511652, RAN4 6G Plan and Meeting Arrangement

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- RP-242393, Revised SID on AIML for mobility in NR

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5