# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513126

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	8.10

Source: 	CMCC

Title: 			 Discussion on spectrum sharing for 6GR study

Document for:	Discussion

- Introduction

In RAN #108 meeting, the SID on 6G radio was approved [1]. The objectives on spectrum sharing are duplicated as following.

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

In last RAN4 meeting, there was discussion about RAN4 6G topic planning based on 6G SID, RAN4 6G plan and meeting arrangement was approved [2]. This meeting is RAN4 first meeting to discuss 6GR. This contribution provides discussion on spectrum sharing.

- Discussion

## 2.1 Multi-RAT Spectrum Sharing

According to the SID [1], at least 5G-6G Multi-RAT Spectrum Sharing and mobility between 5G NR and 6GR are considered. Whether additional migration mechanism is necessary will be studied in RAN plenary.

In RAN#109 meeting, the pCR on requirements for architecture and migration for 6G was approved [2]. It can be seen that Multi-RAT Spectrum Sharing between 6GR and NR and inter-RAT mobility between the 6GR and NR are considered for migration from 5G NR to 6GR.

Agreements on architecture and migration for 6G in RAN#109 The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

It is noted there is discussion on whether to consider other migration mechanism, e.g. inter-RAT dual connectivity, in RANP. We do not think this issue need to be discussed in RAN4. The discussion can be left to RANP. RAN4 study can follow RAN plenary decision and focus on Multi-RAT Spectrum Sharing, and inter-RAT mobility .

Another issue is whether to consider the spectrum sharing between 6GR and LTE, which are under discussion in RANP/RAN1. To avoid duplication, the discussion can be left to RANP/RAN1.

Based on above consideration, it is propose to consider Multi-RAT Spectrum Sharing between 6GR and NR, and inter-RAT mobility between the 6GR and NR for RAN4 study.

Proposal 1: For Multi-RAT Spectrum Sharing, it is proposed that RAN4 focus on MRSS between 6GR and NR.

Proposal 2: for inter-RAT mobility, it is proposed that RAN4 focus on inter-RAT mobility between 6GR and NR .

According to the approved pCR on requirements of new and existing services for 6G [4], as highlighted in red below, 6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration. RAN4 need to study the coexistence between 6GR and NB-IoT (all deployment modes), and coexistence between 6GR and eMTC.

Agreements in RAN#109 The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

Proposal 3: it is proposed to consider the spectrum sharing between 6GR and NB-IoT, spectrum sharing between 6GR and eMTC.

MRSS provides a smooth and efficient evolution from NR to 6G by enabling NR and 6G systems to dynamically share the same spectral resources. For legacy frequency range, e.g. FR1 and FR2-1, the common case for MRSS is FR1, considering the better coverage of low band and limited bandwidth. For FR2-1, firstly, the coverage is limited. Secondly, large channel bandwidth, e.g. 100MHz, is assumed. Based on these consideration, we are not sure whether MRSS like DSS is needed for FR2-1. For MRSS between 6GR and NR, it is proposed to focus on bands where there is commercial NR deployment in FR1.

Proposal 4: For MRSS between 6GR and NR, it is proposed to focus on legacy bands where there is commercial NR deployment in FR1. FFS whether to consider MRSS for FR2-1.

6425 - 7125 MHz range has been identified as IMT spectrum regional at WRC-23, and defined as band n104 in 3GPP. It is expected that there is no NR deployment at U6G, no need to consider MRSS.

Following list the candidate IMT spectrum under WRC-27 agenda item 1.7. For these candidate new IMT spectrum, there is no NR deployment, no need to consider MRSS.

- 4400 - 4800MHz: the major proponents is Russia. US tends to abandon 7125-8400MHz and shift its position to support these frequency range

- 7125 - 8400MHz: this frequency range was opposed by many countries and regional groups, including CEPT, due to strategic usages. These strategic usages are, and will remain, critical in all of this frequency band. Besides, this frequency range is intensively used by a large number of government satellite systems for down-links (i.e. 7 250-7 750 MHz) and up-links (i.e. 7 900-8400 MHz). The upper part of the band (i.e. 8 025-8 400 MHz) is also used for the downlink of Earth observation data by a large majority of Earth Observation satellites. On the other side, Japan and Korea tends to support such frequency range.

- 14.8 - 15.35GHz: now no countries/groups strongly support such bands

Proposal 5: for the legacy bands where there is no commercial NR deployment e.g. U6G, and new 6G spectrum, no need to consider MRSS.

For 4G-5G DSS (dynamic spectrum sharing), it has been mainly focused on FDD deployment. Given the different spectrum situation per operator/region, it is not a good way to consider MRSS only in FDD just like DSS case, and the support of MRSS in TDD as well as FDD also needs to be taken into account.

Proposal 6: it is proposed to support MRSS in both TDD and FDD.

To facilitate MRSS, many aspects need to be considered. According to the RAN4 6G Plan and Meeting Arrangement [2], the scope of system parameters include waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. At least waveform, CBW, numerology, channel raster, etc may have impact on the support of MRSS. Besides, MRSS may also have impact on RF requirements design. Since this is the first meeting, it is better to identify the aspects which impact MRSS facilitation.

Proposal 7: it is proposed to firstly identify the aspects that RAN4 need to study for MRSS.

Proposal 8: to facilitate MRSS, it is proposed to taking following aspects as starting point for RAN4 study

- Waveform

- CBW

- Numerology

- Channel/Synchronization raster

- RF requirements

To support MRSS, the 6G waveform need to be orthogonal with 5G waveform. Whether 5G and 6G transmissions can co-exist within shared spectrum is impacted by time-frequency orthogonality.

Agreements in RAN1 #122 meeting CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

In last RAN1 meeting, it was agreed that the waveforms defined in 5G NR are supported as the basis for 6GR. FFS other waveforms. To enable simultaneous operation, eliminate guard band, simplify receiver complexity for MRSS, it is expected to consider the impact on MRSS for the discussion on new waveform.

Proposal 9: it is proposed to consider the impact on MRSS when discuss the waveform for 6GR

- RAN1 agreed that the waveforms defined in 5G NR are supported as the basis for 6GR

- For other waveform for 6GR, impact on MRSS need to be considered

RAN1 is discussing smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective. Normally, the minimum CBW is defined in RAN4 taking operators’ spectrum allocation, SSB minimum bandwidth, etc, into account. For NR, the CBW multiples of 5MHz is the basic. However, irregular channel bandwidth is considered in later release. There are some candidate solutions, e.g. overlapping channels from network perspective, are considered in the study phase. In the work phase, RAN4 adopt the solution to define native irregular channel BWs, in detail, 7MHz is defined in Release 19 and 6MHz is defined in Release 20. For 6GR, One issue is how to support irregular CBW in 6GR. Defining native irregular channel BWs, e.g. 6MHz, 7MHz, may not cover all the possible irregular channel bandwidth from operators, and will introduce RAN4 workload. If the solution like overlapping channels from network perspective are considered, it may have impact on MRSS. It is proposed to discuss whether MRSS is considered for irregular channel bandwidth pending on the solution to support irregular channel bandwidth.

Proposal 10: it is proposed to consider the impact on MRSS when discussing irregular channel bandwidth. The solution to support irregular channel bandwidth may have impact on MRSS support.

Different sub-carrier spacing for MRSS between 5G and 6G will increase the spectral leakage and introduce ICI. The scheduling coordination would be very complicated and the efficiency for spectrum sharing will be decreased. To facilitate MRSS between 5G NR and 6GR within legacy bands, it is expected that 6GR support the SCS configurations currently predominant in 5G NR deployments.

In last meeting, RAN1 has agreed that 15kHz SCS for FDD and 30kHz SCS for TDD are supported for sub 6GHz. 120kHz is supported for the frequency range between 24.25GHz - 52.6GHz. FFS other SCS options. It is proposed to follow RAN1 agreements.

Agreement in RAN#109 - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

Proposal 11:  the numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD and 30KHz SCS for TDD.

To facilitate MRSS, channel raster also need to be considered. If the channel raster are not aligned for MRSS between 5G and 6G, it will complex the scheduling and reduce spectrum efficiency. In NR, 100KHz channel raster are considered for band below 3GHz except band n41 to align with LTE channel raster. For MRSS between NR and 6G, we are not sure whether 100KHz channel raster are still needed for low band, and can be further discussed.

Proposal 12: For MRSS between NR and 6GR, it is proposed to discuss whether 100KHz channel raster for low band are still needed.

7.5KHz uplink shift is considered for Dynamic Spectrum Sharing (DSS) between LTE and NR. Since LTE uplink and NR uplink adopt different waveform, 7.5KHz Uplink shift is used to guarantee the orthogonality between carriers and avoid inter-carrier inference. As discussed above, RAN1 has agreed that the waveforms defined in 5G NR are supported as the basis for 6GR, i.e. CP-OFDM and DFT-s-OFDM waveforms are supported for 6GR for uplink. No need to consider 7.5KHz Uplink shift for MRSS between NR and 6GR.

Proposal 13: No need to consider 7.5KHz uplink shift for MRSS between NR and 6GR.

Inter-RAT mobility between NR and 6G, including handover and cell re-selection, is an important and simple way to ensure UE’s service continuity when it moves between a 6G cell and a 5G cell. However, the corresponding interruption during inter-RAT handover cannot be avoided. For 6G, we may aim for a solution to minimize the interruption time. Normally, handover interruption comprises the time required to search the target cell, time for fine time tracking, time for UE UE processing. One way to reduce interruption delay is to discuss whether the components of interruption and the time for each components can be reduced.

Proposal 14: it is proposed to minimize handover interruption time for inter-RAT handover between NR and 6GR.

Another consideration is to minimize the data loss when performing inter-RAT measurement. In Rel-18, inter-RAT measurements without gaps, including inter-RAT NR measurement and inter-RAT LTE measurement, are introduced in RAN4 and helpful in reducing the impact on throughput due to measurement gap. It is proposed to support inter-RAT measurements without gaps from 6G day-1.

Proposal 15: it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1.

- Conclusion

This contribution provides discussion on spectrum sharing. The proposals are:

Proposal 1: For Multi-RAT Spectrum Sharing, it is proposed that RAN4 focus on MRSS between 6GR and NR.

Proposal 2: for inter-RAT mobility, it is proposed that RAN4 focus on inter-RAT mobility between 6GR and NR .

Proposal 3: it is proposed to consider the spectrum sharing between 6GR and NB-IoT, spectrum sharing between 6GR and eMTC.

Proposal 4: For MRSS between 6GR and NR, it is proposed to focus on legacy bands where there is commercial NR deployment in FR1. FFS whether to consider MRSS for FR2-1.

Proposal 5: for the legacy bands where there is no commercial NR deployment e.g. U6G, and new 6G spectrum, no need to consider MRSS.

Proposal 6: it is proposed to support MRSS in both TDD and FDD.

Proposal 7: it is proposed to firstly identify the aspects that RAN4 need to study for MRSS.

Proposal 8: to facilitate MRSS, it is proposed to taking following aspects as starting point for RAN4 study

- Waveform

- CBW

- Numerology

- Channel/Synchronization raster

- RF requirements

Proposal 9: it is proposed to consider the impact on MRSS when discuss the waveform for 6GR

- RAN1 agreed that the waveforms defined in 5G NR are supported as the basis for 6GR

- For other waveform for 6GR, impact on MRSS need to be considered

Proposal 10: it is proposed to consider the impact on MRSS when discussing irregular channel bandwidth. The solution to support irregular channel bandwidth may have impact on MRSS support.

Proposal 11:  the numerology discussion for 6GR in legacy band has impact on MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD and 30KHz SCS for TDD.

Proposal 12: For MRSS between NR and 6GR, it is proposed to discuss whether 100KHz channel raster for low band are still needed.

Proposal 13: No need to consider 7.5KHz uplink shift for MRSS between NR and 6GR.

Proposal 14: it is proposed to minimize handover interruption time for inter-RAT handover between NR and 6GR.

Proposal 15: it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1.

- Reference

- RP-251881, New SID: Study on 6G Radio

- R4-2511652, RAN4 6G Plan and Meeting Arrangement

- RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G

- RP-252960, pCR for 38.914 on Requirements of new and existing services for 6G