3GPP TSG RAN WG4 Meeting #116bis		R4-2513174

Prague, Czech, October 13th – 17th , 2025

Title: 	Discussion on R20 HPUE

Agenda Item:	7.1.4

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN#109 meeting, a new WID [1] for UE RF enhancement phase 5 was approved as one of R20 RAN4 5G-A package. The objectives are listed below.

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

As the last two objectives are to start in 2026Q4, in this paper, we’d like to provide our views on first two objectives, i.e. 4Tx for CPE/FWA UE with 4x26dBm PA configurations and PC1.5 with 2Tx for FDD bands for handheld and FWA UE.

2 Discussion on TDD 4Tx with 4x26dBm PA

Based on the WID, 4x26dBm PA is assumed for 4Tx TDD bands. No matter whether RAN4 request a new capability for Power Class 1bis or reuse the existing ‘PC1’ indication for TDD bands, the nominal output for UE should be 32dBm as the RF architecture support. Since the exemplary bands n41, n77 and n78 can supported PC1 31dBm in the latest specification, it may cause some NBC issue or ambiguity if RAN4 just simply reuse the existing ‘PC1’ indication.

Proposal 1: the nominal output power for 4Tx TDD bands with 4x26dBm PA should be 32dBm. In addition, since the exemplary bands n41, n77 and n78 can supported PC1 31dBm in the latest specification, it may cause some NBC issue or ambiguity if RAN4 just simply reuse the existing ‘PC1’ indication.

The upper tolerance of output power was also discussed during the discussion in the RAN#109 meeting. Some companies proposed to consider 32dBm+1dB tolerance for this case, as the biggest value of P-max specified in TS 38.331 is 33dBm. As the baseline assumption for 4Tx bands is to reuse the 26dBm PA component which is used in smartphone to reduce the whole costs, it’s questionable whether these 26dBm PA components can is capable of handling 1dBm upper tolerance. In addition, the insertion loss of front-end RF for FWA/CPE can be smaller than the smartphone. From technical perspective, we didn’t see any points to support +1dB upper tolerance for TDD 4 Tx with 4x26dBm PA.

Proposal 2: 2dB upper tolerance is proposed for 32dBm nominal output power.

Referring to the TS 38.331, the IE P-Max is specified between -30 ~ 33. As 32dBm+2dB tolerance with 4x26dBm PA can achieve 34dBm output power, RAN4 can request RAN2 to extend the range of P-Max without any NBC issue.

Proposal 3: As 32dBm+2dB tolerance with 4x26dBm PA can achieve 34dBm output power, RAN4 can request RAN2 to extend the range of P-Max without any NBC issue.

3 Discussion on PC1.5 with 2Tx for FDD bands for handheld and FWA UE

As RAN4 has completed all the general Tx RF requirements for PC1.5 with 2Tx for TDD bands, RAN4 can discuss whether the general Tx RF requirements for TDD bands with 2Tx PC1.5 can be reused for FDD bands, at least including MOP+tolerance, ACLR, MPR requirements.

Proposal 4: RAN4 can discuss whether the general Tx RF requirements for TDD bands with 2Tx PC1.5 can be reused for FDD bands, at least including MOP+tolerance, ACLR, MPR requirements.

For the band-specific requirements, RAN4 need to discuss the AMPR and RSD requirements for band n1 and n25.

For AMPR, the following NS label should be considered for band n1 and n25.

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.7 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 |  | Clause 6.2.3.4 (NOTE 7) |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.26-1, Table 6.2.3.26-3 | Table 6.2.3.26-2, Table 6.2.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.27-1, Table 6.2.3.27-3 | Table 6.2.3.27-2, Table 6.2.3.27-4 (NOTE 7) |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz,10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. PC1 operation is not allowed. BWChannel less than 30 MHz are addressed in Table 6.5.3.2-1. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 or txDiversity2Tx-r18 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. NOTE 9: Void NOTE 10: This NS value is applicable for cells below 3980 MHz that are partly or fully within the range 3650-3980 MHz for operations in Canada. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 11: Void. NOTE 12: Void. NOTE 13: Void. NOTE 14: Void. NOTE 15: Support of the additional requirement indicated by this network signalling value is indicated by modified MPR behaviour for this band (Annex L.1). |  |  |  |  |  |

For RSD, RAN4 need to discuss the Reference Sensitivity Degradation from PC3 to PC1.5 for FDD bands n1 and n25 for UE supporting Tx Diversity.

Proposal 5: RAN4 can need to discuss the AMPR and RSD requirements for band n1 and n25.

For AMPR, the following NS label should be considered for band n1 and n25. NS_03/NS_03U/NS_05/NS_05U/NS_48/NS_49/NS_100.

For RSD, RAN4 need to discuss the Reference Sensitivity Degradation from PC3 to PC1.5 for FDD bands n1 and n25 for UE supporting Tx Diversity

4 Summary

Proposal 1: the nominal output power for 4Tx TDD bands with 4x26dBm PA should be 32dBm. In addition, since the exemplary bands n41, n77 and n78 can supported PC1 31dBm in the latest specification, it may cause some NBC issue or ambiguity if RAN4 just simply reuse the existing ‘PC1’ indication.

Proposal 2: 2dB upper tolerance is proposed for 32dBm nominal output power.

Proposal 3: As 32dBm+2dB tolerance with 4x26dBm PA can achieve 34dBm output power, RAN4 can request RAN2 to extend the range of P-Max without any NBC issue.

Proposal 4: RAN4 can discuss whether the general Tx RF requirements for TDD bands with 2Tx PC1.5 can be reused for FDD bands, at least including MOP+tolerance, ACLR, MPR requirements.

Proposal 5: RAN4 can need to discuss the AMPR and RSD requirements for band n1 and n25.

For AMPR, the following NS label should be considered for band n1 and n25. NS_03/NS_03U/NS_05/NS_05U/NS_48/NS_49/NS_100.

For RSD, RAN4 need to discuss the Reference Sensitivity Degradation from PC3 to PC1.5 for FDD bands n1 and n25 for UE supporting Tx Diversity

Reference

[1] RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator (Vivo)