3GPP TSG RAN WG4 Meeting #116bis		R4-2513175

Prague, Czech, October 13th – 17th , 2025

Title: 	Discussion on 3MHz (e)RedCap UE

Agenda Item:	7.4.3.1

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN#109 meeting, a new WID [1] for 3MHz (e)RedCap UE was approved as one of R20 RAN4 5G-A package. The objectives are listed below.

The core part of WI: - Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

In this paper, we’d like to discuss the potential RF impacts on the 3MHz (e)RedCap UE.

2 Discussion

As the 3MHz UE RF requirements for the exemplary bands n100, n106, n26, n28 and n85 have been specified, the general RF requirements for 3MHz (e)RedCap UEs at least include the following aspects.

Firstly, the restriction in clause 5.3I can be removed.

5.3I Channel bandwidth for (e)RedCap The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

Secondly, RAN4 need to discuss the value of ΔR1R for PC3 3MHz FDD bands.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R Operating band; Channel bandwidth (MHz); ΔR1R (dB). TDD band; 5, 10, 15, 20; 2.5. FDD band; 3; FFS. FDD band; 5; 2.5. FDD band; 10, 15, 20; 3.0.

Thirdly, as the exemplary bands n26, n28 and n85 can support power class 2. RAN4 may need to discuss whether 3MHz PC2 (e)RedCap UE is in the scope or not.

Finally, the other UE RF requirements including one Tx RF requirements and two Rx RF requirements for 3MHz non-Redcap UE can be applicable to the 3MHz (e)RedCap UE.

Proposal 1: RAN4 to discuss the following aspects about UE RF impacts.

Firstly, the restriction in clause 5.3I can be removed.

5.3I Channel bandwidth for (e)RedCap The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

Secondly, RAN4 need to discuss the value of ΔR1R for PC3 3MHz FDD bands.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R Operating band; Channel bandwidth (MHz); ΔR1R (dB). TDD band; 5, 10, 15, 20; 2.5. FDD band; 3; FFS. FDD band; 5; 2.5. FDD band; 10, 15, 20; 3.0.

Thirdly, as the exemplary bands n26, n28 and n85 can support power class 2. RAN4 may need to discuss whether 3MHz PC2 (e)RedCap UE is in the scope or not.

Proposal 2: the other UE RF requirements including one Tx RF requirements and two Rx RF requirements for 3MHz non-Redcap UE can be applicable to the 3MHz (e)RedCap UE.

3 Summary

Proposal 1: RAN4 to discuss the following aspects about UE RF impacts.

Firstly, the restriction in clause 5.3I can be removed.

5.3I Channel bandwidth for (e)RedCap The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

Secondly, RAN4 need to discuss the value of ΔR1R for PC3 3MHz FDD bands.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R Operating band; Channel bandwidth (MHz); ΔR1R (dB). TDD band; 5, 10, 15, 20; 2.5. FDD band; 3; FFS. FDD band; 5; 2.5. FDD band; 10, 15, 20; 3.0.

Thirdly, as the exemplary bands n26, n28 and n85 can support power class 2. RAN4 may need to discuss whether 3MHz PC2 (e)RedCap UE is in the scope or not.

Proposal 2: the other UE RF requirements including one Tx RF requirements and two Rx RF requirements for 3MHz non-Redcap UE can be applicable to the 3MHz (e)RedCap UE.

Reference

[1] RP-252955, “WID: NR Radio Resource Management (RRM) Phase 6”, Moderator (Apple)