3GPP TSG RAN WG4 Meeting #116bis		R4-2513176

Prague, Czech, October 13th – 17th , 2025

Title: 	Discussion on R20 low bands CA via switching

Agenda Item:	7.8.3

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN#109 meeting, a new WID [1] was approved as one of R20 RAN4 5G-A package. The objectives are listed below.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this paper, we’d like to discuss the potential RF architecture which enable the switching between UL/DL FDD band and Dual-DL FDD+SDL bands and potential requirements.

2 Discussion

2.1 RF architecture and switching time mask

For CA_n12A-n29A, the reference RF architecture is illustrated below. The band n29 DL filter will not work together with the UL chain of UL band n12. The on/off switch of DL band n29 can be operated simultaneously together with the on/off switch of band n12 UL chain. It’s noted that one reference RF architecture can be found without any interruption on DL band n12.

Figure 1 The illustration of reference RF architecture for CA_n12-n29

Proposal 1: The reference RF architecture for CA_n12-n29 shown in the figure 1 can be considered for the discussion.

Figure 2 The whole picture on switching time mask

The whole picture on the switching time mask between UL/DL FDD band and Dual-DL FDD+SDL bands is shown in the figure 2.

It’s clear that the switching gap in SDL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band. However, RAN4 need to discuss whether UE can maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band.

Observation 1: the switching gap in SDL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band.

It’s clear that the switching gap in UL FDD band is needed when the status is switching from UL/DL FDD band to Dual-DL FDD+SDL bands. However, RAN4 need to discuss whether UE can maintain the DL reception in DL FDD band during the switching gap from UL/DL FDD band to Dual-DL FDD+SDL bands.

Observation 2: the switching gap in UL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band.

Proposal 2: RAN4 need to discuss whether UE can maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band or from UL/DL FDD band to Dual-DL FDD+SDL bands.

2.2 REFSENS requirements and DL RMC

As a new switching pattern between the PCell and the SCell will be indicated by network IE LBCA-SwitchingPattern referring to clause 24 of TS 38.213, current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching. That means current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for this case.

The clause 7.3A.2.3 of TS 38.101-1: 7.3A.2.3 Reference sensitivity power level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6. For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2. The reference sensitivity exceptions due to harmonic, harmonic mixing, cross band isolation and power class 2 or power class 3 CA intermodulation interferences are applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO or Tx diversity operation.

Observation 3: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for R20 Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching.

Apart from REFSENS requirements, it’s very important to verify the DL receiving performance for UE before and/or after the Switching gap. Alternatively, RAN4 can design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Observation 4: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

In order to verify the DL REFSENS requirements and DL receiving performance for UE before and/or after the Switching gap, it’s proposed to schedule the DL switch-from slots and switch-to slots for the DL RMC.

Considering the worst case and simplifying the specification, it’s proposed to configure two symbols PDSCH mapping Type B for DL RMC referring to the table 5.1.2.1-1 of TS 38.214. It’s better to configure the same length of symbols in all the DL switch-from slots and switch-to slots.

Table 5.1.2.1-1: Valid S and L combinations PDSCH mapping type; Normal cyclic prefix; Extended cyclic prefix. S; L; S+L; S; L; S+L. Type A; {0,1,2,3} (Note 1); {3,…,14}; {3,…,14}; {0,1,2,3} (Note 1); {3,…,12}; {3,…,12}. Type B; {0,…,12}; {2,4,7}; {2,…,14}; {0,…,10}; {2,4,6}; {2,…,12}. Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3.

For the LBCA-SwitchingPattern, it’s proposed to design the DL RMC as ‘00011 00110 01100 11001 11100 11001 10011 00110’ in order to increase DL samples (18 DL Slots for PCell and 18 DL Slots for SCell). Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots. In addition, one symbol PDCCH is scheduled at the first symbol in each DL slot except for SSB slots. The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown below.

Slot Index; Pattern; UL FDD band; DL FDD band; SDL band; Frame Index. 0; 0; PUCCH/PUSCH; SSB; NA; i. 1; 0; PUCCH/PUSCH; SSB; NA. 2; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 3; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 4; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 5; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 6; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 7; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 8; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 9; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 10; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA; i+1. 11; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 12; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 13; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 14; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 15; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 16; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 17; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 18; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 19; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 20; 1; NA; SSB; SSB; i+2. 21; 1; NA; SSB; SSB. 22; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 23; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 24; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 25; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 26; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 27; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 28; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 29; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 30; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH; i+3. 31; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 32; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 33; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 34; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 35; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA. 36; 0; PUCCH/PUSCH; SF-PDCCH and PDSCH; NA. 37; 1; NA; ST-PDCCH and PDSCH; ST-PDCCH and PDSCH. 38; 1; NA; SF-PDCCH and PDSCH; SF-PDCCH and PDSCH. 39; 0; PUCCH/PUSCH; ST-PDCCH and PDSCH; NA.

Figure 3 The DL RMC switching pattern for R20 LB CA via switching

Proposal 3: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

3-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

3-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

3-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

3-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 3.

3 Summary

Proposal 1: The reference RF architecture for CA_n12-n29 shown in the figure 1 can be considered for the discussion.

Observation 1: the switching gap in SDL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band.

Observation 2: the switching gap in UL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band.

Proposal 2: RAN4 need to discuss whether UE can maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band or from UL/DL FDD band to Dual-DL FDD+SDL bands.

Observation 3: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for R20 Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching.

Observation 4: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Proposal 3: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

3-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

3-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

3-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

3-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 3.

Reference

[1] RP-252953, “New WID on enhancement of low NR band carrier aggregation via switching”, Moderator (Huawei)