3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513202

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on RRM requirements for HD-FDD for Ku band with FR1 numerology

Source: 	CATT

Agenda item:	7.6.4.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for enhanced requirements for NR NTN and IoT NTN Phase 2 in Rel-20 was approved in RAN#109.The current objectives of the work item which are related to HD-FDD for NTN is copied as follows [1]:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

This document will discuss HD-FDD impact for NTN RRM requirements for VSATs in both Ka and Ku bands and give our proposals.

- Discussion

HD-FDD technology has been widely used, demonstrating the fact that cost reduction is a key driving factor for successful markets in other non-3GPP satellite communication systems. In addition, automotive-mounted VSAT (circa. 20 x 20 cm) are considered a key segment for Ku/Ka band NR-NTN market penetration due to large volumes. Considering that HD-FDD antennas are simpler to implement than FD-FDD antennas and better match the consumer expected price point, introducing HD-FDD Mobile VSAT with small antennas for Ku/Ka band NR-NTN is very meaningful [1].

  - RRM impact

Regarding NR NTN RRM requirements for Ku band introduced in Rel-19, the following requirements were defined [2]:

Table 1 NTN RRM requirements for Ku band introduced in Rel-19

| Handover for SAN | NR SAN FR2 – NR SAN FR2 Satellite switching with re-synchronization |
| --- | --- |
| Timing requirements | UE transmit timing for Satellite Access |
| Measurement requirements | Intra-frequency measurements without measurement gaps for NTN band above 10 GHz Intra-frequency measurements with measurement gaps for NTN band above 10 GHz |
|  | Inter-frequency measurement with measurement gaps for NTN band above 10 GHz Inter-frequency measurements for NTN band above 10 GHz Inter-frequency measurements without measurement gaps for NTN band above 10 GHz |

Since RAN4 to firstly define the requirements for Ku band with FR1 numerology in Phase 1, so RAN4 need to consider the HD-FDD impact for VSATs in Ku band with FR1 numerology, and the requirements defined for NTN in Ku band with FR1 numerology can be a baseline.

Proposal 1: RAN4 to consider the HD-FDD impact for VSATs in Ku band with FR1 numerology, and the requirements defined for NTN in Ku band with FR1 numerology can be a baseline.

In legacy, RAN4 only consider HD-FDD duplex mode for RedCap UE, but the HD-FDD related requirements only defined for FR1. According to the RedCap WI, the following  RRM requirements are affected by HD-FDD for (e)RedCap UE in TN network [3].

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

For HD-FDD Mobile VSAT, RAN4 need to discuss how to define HD-FDD related requirements for FR2, and the HD-FDD impact defined for RedCap UE can be a reference when defining requirements for HD-FDD VSATs type.

Observation 1: In legacy, RAN4 only consider HD-FDD duplex mode for RedCap UE, but the HD-FDD related requirements only defined for FR1.

Proposal 2: RAN4 need to discuss how to define HD-FDD related requirements for FR2, and the HD-FDD impact defined for RedCap UE can be a reference when defining requirements for HD-FDD VSATs type.

  - Specification structure

Since HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) is a new VSAT type, and it is operating in both Ka and Ku bands with FR1 and FR2 numerologies, in Phase 1, we propose that HD-FDD related requirements should be defined under the clauses defined for NTN RRM requirements for Ku band in Rel-19.

Proposal 3: In Phase 1, HD-FDD related requirements should be defined under the clauses defined for NTN RRM requirements for Ku band in Rel-19.

- Conclusion

This document discussed RRM requirements for HD-FDD VSATs in Ku bands and presented following observation and proposals:

Proposal 1: RAN4 to consider the HD-FDD impact for VSATs in Ku band with FR1 numerology, and the requirements defined for NTN in Ku band with FR1 numerology can be a baseline.

Observation 1: In legacy, RAN4 only consider HD-FDD duplex mode for RedCap UE, but the HD-FDD related requirements only defined for FR1.

Proposal 2: RAN4 need to discuss how to define HD-FDD related requirements for FR2, and the HD-FDD impact defined for RedCap UE can be a reference when defining requirements for HD-FDD VSATs type.

Proposal 3: In Phase 1, HD-FDD related requirements should be defined under the clauses defined for NTN RRM requirements for Ku band in Rel-19.

- Reference

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

- R4-2512351, Big CR on RRM requirements of Ku band for NR NTN, Apple

- 3GPP TS 38.133, NR Requirements for support of radio resource management