3GPP RAN WG4 Meeting #116bis	R4-2513221

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on RRM performance requirements of SBFD operation

Source: 	CATT

Agenda item:	6.14.3.2

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 approved the work plan for defining the SBFD RRM performance requirements. In this contribution, we will discuss the details of performance requirements for SBFD-based RRM measurements based on the RAN4#116 WF [1] and open issue summary [2].

Discussion

CSI-RS based LI measurement accuracy requirement

In RAN4#116 meeting, RAN4 agreed the measurement period requirement for CSI-RS based L1 measurement while the measurement accuracy requirements need further discussion:

RAN4#116: Agreement: - For UE supporting simultaneous DL reception in 2 DL subbands, - For measurement period: - For UE indicating CSI processing time scaled by 2, - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - For UE not indicating CSI processing time scaled by 2, - Lower bound is not needed and existing measurement period applies - Further discuss the measurement accuracy requirements in Perf part of the WI - For UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply. - Option 2: accuracy requirements based on 24 RB for Case 3 apply. - For UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB for Case 3 apply. - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

In RAN4#115 meeting, RAN4 agreed to define relaxed accuracy requirements based on 24 PRBs for Case 3:

RAN4#115: - Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

For UE supporting simultaneous DL reception in 2 DL subbands, UE still could indicate whether the CSI processing time needs to be scaled by 2. For UE not indicating CSI processing time scaled by 2, considering that 2 DL subbands are not contiguous and there might be performance degradation when UE receives both DL subbands simultaneously. Therefore for this case, it is reasonable to apply the accuracy requirements based on 24 PRBs for Case 3. For UE indicating CSI processing time scaled by 2, in our understanding, UE can meet the accuracy requirements based on 48 PRBs since more time is used for CSI processing.

Proposal 1: For UE supporting simultaneous DL reception in 2 DL subbands for CSI-RS L1 measurement:

- For UE indicating CSI processing time scaled by 2, existing accuracy requirements based on 48 RB for Case 3 apply.

- For UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB for Case 3 apply.

Conclusions

This contribution discussed the RRM performance requirements for SBFD-based RRM measurements and some proposals are provided:

Proposal 1: For UE supporting simultaneous DL reception in 2 DL subbands for CSI-RS L1 measurement:

- For UE indicating CSI processing time scaled by 2, existing accuracy requirements based on 48 RB for Case 3 apply.

- For UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB for Case 3 apply.

References

[1] R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#116.

[2] R4-2509061, Topic summary for [116][218] NR_duplex_evo, Moderator (Huawei, HiSilicon), RAN4#116.