3GPP RAN WG4 Meeting #116bis	R4-2513224

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on inter-vendor training collaboration for two-sided AIML models

Source: 	CATT

Agenda item:	7.9.3

Document for:	Discussion

Introduction

In RAN#109 meeting, RAN approved the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1] with the following objectives for RAN4:

… Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we will discuss the issues on inter-vendor training collaboration and present our views.

Discussion

According to the objective in the WID, RAN4 need to study the feasibility of fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account and provide feedbacks to RAN plenary, i.e., RAN#110, for checking whether the model with scalability can serve the purpose of inter-vendor training collaboration for “Case 0”, i.e., CSI spatial/frequency compression without temporal aspects (two-sided model).

Observation 1: According to WID, RAN4 needs to study the feasibility of “Direction C” with RAN1 scalable model considered and provides RANP with feedbacks.

In RAN1, the agreements related to (“Direction C”) are reproduced below for reference:

RAN1#118: Agreement Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing? - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it? - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it? - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs? - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it? - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it? - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them? - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it? - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing? - Note: - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder. - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder. - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder. - Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data. - Note: Some issues identified in one direction may/may not be applicable for other Direction. - Note: potential down selection among the 3 directions is not precluded - Study of data distribution mismatch to consider the use of synthetic data and/or field data. RAN1#118bis: Agreement - For Direction C: - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data - Including performance evaluation of training/finetuning the models by modelling data distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration. - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3) - FFS whether the reference model is the same as RAN4 specified model - FFS whether a single or multiple reference (and how many) models need to be specified RAN1#119: Agreement - For Direction A Option 3a-1 and Direction C, study the feasibility of scalable model structure specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots. - Feasibility of Direction A Option 3a-1 is contingent on the feasibility of the scalable model structure specification. - Feasibility of Direction C is contingent on the feasibility of the scalable model structure specification and model parameters specification. - Note: Angular/Delay/Doppler domain conversion and quantization are considered to be part of feasibility study RAN1#120: Conclusion - Direction C with standardized model ensures a minimum performance requirement. - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. - Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance. RAN1#120bis: Agreement In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned or how the ID is specified - Note: whether the purpose for pair will be specified will be discussed separately. Agreement For inter-vendor collaboration Direction C, - Use standardized quantization codebook For inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), - Standardize configuration(s) of quantization codebook, e.g., scalar or vector quantization, segment size of VQ, codebook size. - FFS: applicability of the above for Case 2 - Exchange quantization codebook of (the selected) standardized configuration(s) from NW-side to UE-side along with each exchanged dataset or model parameters. - FFS: whether quantization codebook may be different across different payload size configurations Conclusion - For Direction A Option 3a-1, based on evaluation results, RAN1 concludes that scalable model structure specification is feasible. - For Direction C, based on evaluation results, RAN1 concludes that scalable model structure specification and model parameters specification is feasible. - Note: RAN4 feasibility is a separate study to be confirmed by RAN4. Conclusion - RAN1 concludes that both Direction A and Direction C are feasible. - RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible. - This includes both 3a-1 with and without target CSI sharing. RAN1#121: Agreement For addressing inter-vendor collaboration complexity, RAN1 identifies the following specification impacts for supporting sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C - Reference model / structure specification (for sub-option 3a-1 and Direction C) - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. - Note: The model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 can be same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder. - To reduce the workload of the potential normative work scope, the model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 is(are) same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder. - Note: RAN1 has studied and agreed on a scalable model structure for feasibility study during Rel-19 study, and considers that can serve as a starting point for the model structure specification for the potential normative work. - Format and contents of parameter/dataset/information exchange: dataset (for 4-1 and 3a-1 with target CSI)/ encoder parameter (for 3a-1) / additional information (for 3a-1 and 4-1). - The workload can be reduced by identifying the common part among different aspects.

For Case 0 with spatial-frequency domain input, RAN1 agreed that Transformer is used as the backbone structure:

RAN1#120: Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment. Decoder description: The decoder has a mirroring design as the encoder. Details are to be discussed. Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

In SI phase, RAN4 also carried out feasibility study on inter-vendor collaboration from testing perspective mainly based on CNN backbone. Multiple rounds of simulations were implemented and the simulation results were summarized in [2], based on which RAN4 concluded that the Option 3 (fully specified standardized test decoder) can provide a good performance alignment and thereby is selected for WI:

RAN4#116: Further observation on option 4: Agreement. - Option 4 can work as long as the decoder structure is fully specified and the same dataset is used by all TE vendors and 3GPP. - Some validation of the TE decoder may be needed Note that option 3 is selected for the follow-on WI.

In addition, RAN4 also evaluated Option 3 with transformer backbone, i.e., Encoder 2 in [2], and its structure and the corresponding simulation results are copied below [3], which essentially is the same as the scalable transformer model studied in RAN1:

Figure 1 – Structure and results of transformer encoder for CSI compression evaluation

Observation 2: RAN4 had simulated the CSI compression performance of transformer encoder-CNN decoder pair and good performance alignment was achieved.

Observation 3: The structure of transformer encoder used in RAN4 simulations is essentially the same as the RAN1 scalable transformer model.

Therefore RAN4 can conclude that Direction C is also feasible from the performance/testing perspective. Regarding on how to define/specify the scalable transformer model in spec, more discussions are needed.

Proposal 1: RAN4 to conclude that Direction C is also feasible from the performance/testing perspective.

Proposal 2: RAN4 to discuss how to define/specify the scalable transformer model in spec.

Conclusions

This contribution discussed the issues on inter-vendor training collaboration for AI/ML enabled CSI compression, and some observations and proposals are provided:

Observation 1: According to WID, RAN4 needs to study the feasibility of “Direction C” with RAN1 scalable model considered and provides RANP with feedbacks.

Observation 2: RAN4 had simulated the CSI compression performance of transformer encoder-CNN decoder pair and good performance alignment was achieved.

Observation 3: The structure of transformer encoder used in RAN4 simulations is essentially the same as the RAN1 scalable transformer model.

Proposal 1: RAN4 to conclude that Direction C is also feasible from the performance/testing perspective.

Proposal 2: RAN4 to discuss how to define/specify the scalable transformer model in spec.

References

[1] RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm, RAN#109.

[2] R4-2510880, Collection of simulation results on CSI compression, Huawei, HiSilicon, RAN4#116.

[3] R4-2508084, Simulation assumptions on AI/ML CSI compression, Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, RAN4#115.