3GPP TSG-RAN WG4 Meeting #116-bis	                                                                                       R4-2513232

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on general aspect for NR NTN and IoT NTN Phase 2

Agenda item:	7.6.2

Document for:	Approval

- Introduction

In RAN #109 meeting, a new WID [1] for NTN and IoT NTN phase 2 has been approved. The work item will focus on the further enhancement of NTN HPUE and HD-FDD VSAT. This paper is initially shares some views on NTN and IoT NTN phase 2.

- Discussion

In RAN #109 meeting, a new WID for NTN and IoT NTN phase 2 has been approved, as shown below:

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes. HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable. DL intra-band intra-SAN CA for NTN Core Part - Specify RF and RRM requirements for NR NTN CA for UE/VSAT and SAN core requirements with 2DL component carrier and 1UL - Contiguous carriers in Ka band (Band n512) - 400 MHz + 400 MHz channel bandwidths - Non-contiguous carriers in L band (Band n255) - 5 MHz + 5 MHz channel bandwidths The DL intra-band intra-SAN CA objectives will commence upon completion of HPUE for NTN Core Part (PC1.5 for IoT-NTN) and RAN1/RAN2 availability.

The WID indicates that for NTN HD-FDD, the work procedure will be divided into phase 1 and phase 2. It is worth noting that the work in phase 1 and phase 2 is interrelated, so it is necessary to ensure that both phase 1 and phase 2 are completed on time. Before the specific work begins, a reasonable plan could help to achieve this goal. One possible approach is for RAN4 to study and specify band agnostic requirements first, which could leverage those completed work for all requirements at all phases.

Proposal 1:RAN4 should first specify the band agnostic requirements for NTN HD-FDD VSAT in phase 1.

Meanwhile, the workload should be evaluated dynamically. Although the work on NTN HPUE and HD-FDD VSAT for Ku band FR1 will be launched simultaneously, due to the significant difference in workload between two items (the MPR/A-MPR evaluation for HPUE is usually complex and time-consuming), it can be predicted that HD-FDD for Ku band will be completed first. However, in order to maximize the likelihood that all tasks in this WI can be completed on time, we recommend starting work on the HD-FDD VSAT phase 2 immediately once any task is completed.

Proposal 2: Once any task in this WID is completed, RAN4 should immediately start the work of HD-FDD VSAT phase 2.

In addition, a timeline for HD-FDD VSAT phase 1 can be considered, which will help with management and the progress of future work. Considering that this meeting will discuss the work plan for HD-FDD VSAT, it is recommended to establish a progress checkpoint (such as March 2026) in RAN4 to avoid jeopardizing the subsequent phase 2 works.

Proposal 3: It is recommended to establish a progress checkpoint (such as March 2026) in RAN4 to avoid jeopardizing the subsequent phase 2 works.

- Conclusion

This contribution provides our initial views on NTN and IoT NTN phase 2. The following proposals are concluded as follows:

Proposal 1:RAN4 should first specify the band agnostic requirements for NTN HD-FDD VSAT in phase 1.

Proposal 2: Once any task in this WID is completed, RAN4 should immediately start the work of HD-FDD VSAT phase 2.

Proposal 3: It is recommended to establish a progress checkpoint (such as March 2026) in RAN4 to avoid jeopardizing the subsequent phase 2 works.

- References

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN #109