3GPP TSG RAN WG4 Meeting #116bis		R4-2513244

Prague, Czech Republic, Oct 13th – 17th , 2025

Title: 	Discussion on Demodulation performance requirements for LP-WUS/WUR

Agenda Item:	6.16.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

With the 100% completion of core part of the work item LP-WUS/WUR [1], the focus of this WID becomes performance part, though 25% completion for this part is declared, and CRs to TS 38.141-1/38.141-2 are also approved [2][3].

In this contribution, we present our views on aspects related to demodulation performance requirements.

# 2 Discussion

False Alarm Rate (FAR)

In the early stage of this work item, RAN4 has reached an agreement on performance metrics for LP-WUR [4]:

Issue 1-1-11: Metric for LP_WUR requirements Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values.

A false alarm forces the main radio to wake up unnecessarily, leading to power waste. Since the primary purpose of a low-power receiver is to save energy, the false alarm rate should be strictly limited. A value of 1% is consistent with existing requirements and provides a reasonable balance between detection reliability and implementation complexity. This level prevents excessive wake-ups from false detections, thereby preserving UE power efficiency and aligning with the low-power design objectives of LP-WUS. Accordingly, the detection of LP-WUS shall meet a false alarm rate (FAR) of 1%.

Proposal 1: RAN4 to set a false alarm rate at 1% for LP-WUS demodulation requirements.

Separate demodulation requirements aligned with UE capabilities

Given that UE capabilities already distinguish between OFDM-based and OOK-based LP-WUS receivers, the demodulation requirements should be defined separately for each receiver type. This ensures that requirements are applied only to the UEs declaring the corresponding capability, however, the common test methodology can be retained, while performance targets are tailored to the receiver principle.

Proposal 2: RAN4 to introduce separate demodulation requirements for OOK-based and OFDM-based LP-WUS receivers.

Coverage of non-repetition and full-repetition cases for OFDM-based receivers

For UEs declaring OFDM-based LP-WUS capability, demodulation requirements should explicitly address both one-shot, i.e., non-repetition, and full-repetition transmission modes. Since repetition has significant influence on detection reliability, energy efficiency, and latency, the inclusion of both cases ensures not only consistent and fair performance expectations but also flexibility across different network configurations.

Proposal 3: RAN4 to specify demodulation requirements for both one-shot and full-repetition cases for OFDM-based receivers.

# 3 Conclusion

In this contribution, we have the following proposals for demodulation performance requirements for LP-WUS/WUR:

Proposal 1: RAN4 to set a false alarm rate at 1% for LP-WUS demodulation requirements.

Proposal 2: RAN4 to introduce separate demodulation requirements for OOK-based and OFDM-based LP-WUS receivers.

Proposal 3: RAN4 to specify demodulation requirements for both one-shot and full-repetition cases for OFDM-based receivers.

# Reference

[1] RP-252854, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR); rapporteurs: vivo, NTT DOCOMO”, vivo, NTT DOCOMO

[2] R4-2512596, “BigCR to TS 38.141-1 on support for LP-WUS”, CATT, ZTE

[3] R4-2512597, “BigCR to TS 38.141-2 on support for LP-WUS”, CATT, ZTE

[4] R4-2410569, “WF for [111][136] NR_LPWUS_UERF”, vivo