3GPP TSG-RAN WG4 Meeting 116-bis	R4-2513283

# Prague, Czech Republic, October 13th – 17th, 2025

Source: 	Xiaomi

Title: 	Overview for 6GR RAN4 centric AI use cases

Agenda Item:	8.9

Document for:	Discussion

# 1 Introduction

Radio Resource Management (RRM) measurements are the cornerstone of network performance. However, the 5G NR framework, as specified in TS 38.133, established a sophisticated but functionally siloed measurement system. We have observed a growing consensus that this fragmented design presents fundamental challenges that hinder performance and increase UE complexity.

While evolutionary enhancements can provide incremental benefits, we believe 6G requires a paradigm shift. This proposal argues that by leveraging Artificial Intelligence (AI), we can create a truly unified and predictive measurement framework, and that RAN4 is the ideal group to lead this crucial study for 6G.

# 2 Discussion

2.1 RRM centric use case

2.1.1 Current RRM Framework and Inefficiencies

Extended measurement delay

In NR today, L3-RSRP measurements for mobility are performed using wide Rx beams, while L1-RSRP measurements for beam management use narrow Rx beams. As a result, even for the same serving cell, the UE cannot complete L1 and L3 measurements simultaneously.

In the 5G specifications, L1-RSRP and L3-RSRP measurements are scheduled in a time-division multiplexing (TDM) manner: L3 reporting and L1 measurement are separated in time. Specifically:

- The UE uses SMTC resources to perform L3-RSRP measurement.

- The UE uses resources outside SMTC occasions for L1-RSRP measurement.

Fig.1 TDM between L1 and L3 measurement

Furthermore, the standard prioritizes L3 measurement over L1, ensuring mobility robustness. To capture this interaction, 38.133 defines a sharing factor (P), which extends the effective measurement time of L1 whenever resources overlap with L3 measurement occasions.

For example:

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Besides, RX beam sweeping factor, N, also works in TDM manner.

This time-sharing model directly increases L1 measurement latency and reduces scheduling flexibility. This leads to two core problems:

- Cross-Layer Conflict: By treating L1 and L3 measurements as separate, conflicting tasks, the current framework creates an artificial inefficiency. It prevents the system from simultaneously exploiting the strong underlying correlation between them, leading to longer measurement times and reduced flexibility.

- Brute-Force Procedures: The framework relies on exhaustive Rx beam sweeping (factor N), where a UE must sweep across all configured receive beams. This approach scales poorly in FR2 and will become unsustainable in 6G. This exhaustive sweeping is a primary driver of measurement latency and UE power consumption.

Observation 1: Current specifications impose a time-sharing model (factor P) and RX beam sweeping factor N, which increases measurement latency for L1 and reduces scheduling flexibility.

Severe Scheduling Restrictions:

Beyond latency, the framework imposes significant restrictions on data transmission. As specified in TS 38.133, on any symbol configured for L1 RRM (via SSB, CSI-RS) or L3 RRM (within SMTC) measurements, the UE is not expected to transmit or receive data (e.g., PUSCH/PDSCH) in FR2.

Therefore, every symbol dedicated to these exhaustive measurements is a symbol that cannot be used for the data transmission. This directly carves out radio resources, creating a hard limit on cell throughput and spectral efficiency.

Observation 2: In FR2, the current framework define scheduling restriction for data transmission. This imposes a direct spectral efficiency cost, as every symbol allocated for measurement is a symbol unavailable for user data.

2.1.2 The Unexploited Opportunity of Cross-Domain Correlation

The inefficiencies described above are avoidable because the measurements are not independent. There exists a strong, unexploited optimization potential rooted in:

- Spatial Correlation: A wide Rx beam inherently aggregates several narrow Rx beams; a measurement on one provides a strong statistical hint about the others.

- Temporal Correlation: L3-RSRP is derived by filtering L1-RSRP results over time. This means short-term L1 behavior and long-term L3 trends are inherently linked.

- Statistical Predictability: Due to these correlations and the often-gradual evolution of the radio channel, a small subset of observations can be used to predict the rest with high reliability.

Observation 3: The strong space-time correlations between L1 and L3 measurements are not leveraged in the current framework, leaving significant optimization potential unused.

2.1.3 RAN4's Unique Role in Unification

The following table contrasts RAN4's necessary focus with the ongoing AI work in other WGs:

| Aspect | RAN1 AI Focus (L1) | RAN2 AI Focus (L3) | Proposed RAN4 Focus (Unified Framework) |
| --- | --- | --- | --- |
| Domain | Intra-Layer, Narrow-Beam | Intra-Layer, Wide-Beam | Cross-Layer (L1↔L3) & Intra-Layer Enhancement |
| Goal | Optimize beam selection | Improve mobility prediction | Fuse measurements and simplify procedures system-wide |
| Output | Beam-specific metrics | Handover performance | End-to-end UE performance requirements (latency, power, efficiency) |

While RAN1 and RAN2 focus on applying AI to optimize specific, feature based procedures (e.g., beam selection, handover prediction), their scope ends at the boundary of their respective layers. RAN4 holds the unique responsibility and vantage point to define the performance of the UE as an integrated measurement system. Our mandate is therefore to specify how AI orchestrates measurements across these layers to achieve globally optimal outcomes in latency, power, and spectral efficiency—a system-level optimization that is absent in the current architecture and falls squarely within RAN4's domain.

Observation 4: RAN4's role is complementary and systemic. While RAN1 and RAN2 use AI to optimize within their domains, RAN4 focuses on the system-level performance resulting from unification, which is essential for overall efficiency but falls outside the scope of other WGs.

2.1.4 Benefits of a Unified AI-based Framework

Exploiting these correlations through a unified, AI-based framework can deliver substantial benefits:

- Reduced Measurement Latency & Power Consumption: Intelligently reducing separate and exhaustive measurements directly cuts UE measurement time and energy drain.

- Increased Data Throughput by Lifting Scheduling Restrictions: By reducing or eliminating the need for physical measurements on certain symbols, the associated scheduling restrictions are lifted. These symbols can be reclaimed for PDSCH/PUSCH transmission, directly converting measurement overhead into higher cell throughput and improved spectral efficiency.

- A Simplified and Scalable Framework for 6G: An AI-native framework is essential to manage the increasing complexity of 6G (e.g., higher frequency bands, more beams) in a sustainable way.

Observation 5: A unified framework can break the fundamental trade-off between measurement overhead and performance, benefiting both UE efficiency and network capacity.

2.1.5 Defining RAN4’s Unique Role

RAN4 is uniquely positioned to lead this study for two key reasons:

- Cross-Layer Fusion: While other groups focus on single-layer AI optimizations, RAN4’s mandate is to break the silos between L1 and L3, creating a holistic framework that improves overall RRM performance. e.g. apply L1 measurement to predict L3 measurement or vice versa.

- UE-centric Optimization: The optimization of the UE’s internal receiver procedures, such as reduced Rx beam sweeping, is the quintessential role of RAN4. It directly impacts core KPIs central to our work: UE complexity, power consumption, and measurement latency.

Observation 6: RAN4 is the natural forum to lead this study, given its role in capabilities, requirements, and testing.

2.1.6. Summary

The current fragmented RRM framework imposes avoidable inefficiencies and direct data throughput limitations that will become critical bottlenecks in 6G. By initiating a study on a unified, AI-enabled framework, RAN4 can take the lead in defining next-generation UE capabilities and requirements that break these limitations. We welcome discussion and seek consensus to begin this essential work to prepare the RRM framework for the demands of 6G. To address the identified problems and realize the potential benefits, we put forward the following proposal:

Proposal 1: RAN4 to initiate a Study Item (SI) on an “AI-enabled Unified RRM Measurement Framework for 6G.”

The scope of the study should include, but not be limited to:

- Investigating and characterizing potential gains in measurement latency, UE power consumption, and network spectral efficiency by targeting the reduction of TDM-based measurement conflicts and exhaustive beam sweeps.

- Defining the framework for new UE capabilities that would enable predictive RRM measurements.

- Establishing the principles for new or modified RRM performance requirements, including accuracy, reliability, and testability aspects for a UE implementing such capabilities.

- The study will explicitly not standardize specific AI/ML algorithms, models, or training methodologies, preserving implementation freedom and focusing solely on the RAN4-centric aspects of performance and interoperability.

2.2 AI/ML-based transceiver optimization

The transceiver, as a fundamental component of radio access networks, directly impacts network efficiency, spectral utilization, energy consumption, and overall system performance. Traditional transceiver design methodologies are increasingly inadequate for 6G requirements, and AI/ML techniques provide the necessary flexibility to handle these challenges through continuous learning and adaptation. The integration of AI/ML into transceiver design also aligns with the 6G vision of intelligent networks where devices can self-optimize and adapt to changing conditions, reducing the need for manual intervention and improving overall network efficiency.

Observation 7: For 6G system, it is beneficial to study an AI/ML-driven paradigm for transceiver performance optimization and adaptation.

AI/ML-assistant PA Performance Optimization is one of use case to optimize the transceiver performance. Digital pre-distortion is a critical technique for compensating PA nonlinear distortions. AI/ML can significantly enhances DPD systems through:

- Model Precision: AI/ML establish more accurate PA nonlinear models, improving pre-distortion compensation effectiveness

- Real-time Adaptation: AI/ML enable DPD systems to continuously learn and optimize pre-distortion parameters during operation, adapting to dynamic working conditions

- Online Calibration: AI/ML maintain optimal performance through continuous monitoring of output signals and real-time adjustment of pre-distortion parameters

Observation 8: AI/ML-enhanced DPD elevates PA performance by delivering more accurate nonlinear modeling, real-time adaptation, and continuous online calibration.

# 3 Conclusion

RRM centric use case

Observation 1: Current specifications impose a time-sharing model (factor P) and RX beam sweeping factor N, which increases measurement latency for L1 and reduces scheduling flexibility.

Observation 2: In FR2, the current framework define scheduling restriction for data transmission. This imposes a direct spectral efficiency cost, as every symbol allocated for measurement is a symbol unavailable for user data.

Observation 3: The strong space-time correlations between L1 and L3 measurements are not leveraged in the current framework, leaving significant optimization potential unused.

Observation 4: RAN4's role is complementary and systemic. While RAN1 and RAN2 use AI to optimize within their domains, RAN4 focuses on the system-level performance resulting from unification, which is essential for overall efficiency but falls outside the scope of other WGs.

Observation 5: A unified framework can break the fundamental trade-off between measurement overhead and performance, benefiting both UE efficiency and network capacity.

Observation 6: RAN4 is the natural forum to lead this study, given its role in capabilities, requirements, and testing.

Observation 7: For 6G system, it is beneficial to study an AI/ML-driven paradigm for transceiver performance optimization and adaptation.

Observation 8: AI/ML-enhanced DPD elevates PA performance by delivering more accurate nonlinear modeling, real-time adaptation, and continuous online calibration.

Proposal 1: RAN4 to initiate a Study Item (SI) on an “AI-enabled Unified RRM Measurement Framework for 6G.”

AI/ML-based transceiver optimization

Observation 7: For 6G system, it is beneficial to study an AI/ML-driven paradigm for transceiver performance optimization and adaptation.

AI/ML-assistant PA Performance Optimization is one of use case to optimize the transceiver performance. Digital pre-distortion is a critical technique for compensating PA nonlinear distortions. AI/ML can significantly enhance DPD systems through:

- Model Precision: AI/ML establish more accurate PA nonlinear models, improving pre-distortion compensation effectiveness

- Real-time Adaptation: AI/ML enable DPD systems to continuously learn and optimize pre-distortion parameters during operation, adapting to dynamic working conditions

- Online Calibration: AI/ML maintain optimal performance through continuous monitoring of output signals and real-time adjustment of pre-distortion parameters

Observation 8: AI/ML-enhanced DPD elevates PA performance by delivering more accurate nonlinear modeling, real-time adaptation, and continuous online calibration.