3GPP TSG-RAN WG4 Meeting #116bis	R4-2513293

Prague, CZ, 13th – 17th October 2025

Agenda item:	8.9

Source: 		NTT DOCOMO, INC.

Title: 	6G RAN4 Study on AI

Document for:	Discussion

- Introduction

In RAN4#116, the plan and meeting arrangement for 6G RAN4 study were introduced through contribution R4-2511652[1]. Starting from this meeting, discussions on 6G begin within each RAN4 topic. In this contribution, we present our view on 6G AI in the context of RAN4.

- Discussion

According to R4-2511652[1], the scope of AI as 6G topics was shared. We contribute the 6G concept from each RAN4 perspective.

In 3GPP NR RAN4, discussions on measurement relaxation for RRM using AI/ML have been actively conducted since Release 18. We summarize the key topics considered in RAN4, including those currently under discussion as below.

Rel-18 (SI: FS_NR_AIML_Air): RAN4 clarified the need for a framework for testability and interoperability of AI/ML for NR air interface. The main outcome was the establishment of evaluation principles and the consolidation of results in TR 38.843. Model specification is out of scope; all work is within the existing RAN architecture, and the need for AI processing capability definition was considered.

Rel-19 (WI: NR_AIML_Air): The normative work moved to a one-sided model framework, including LCM (Life Cycle Management). RAN4 was responsible for defining core requirements and test frameworks for use cases such as beam management and positioning, with impacts on 38.133 and other TSs.

Rel-19 (SI: FS_NR_AIML_Mob): The study on AI/ML for mobility began. RAN4 evaluated RRM requirements, KPIs, testability, and interoperability for mobility use cases, with results to be consolidated in TR 38.744. Two-sided models are out of scope for this SI.

Rel-20 (WI: NR_AIML_air_Ph2): The scope expanded to two-sided models (e.g., CSI feedback enhancement/compression). RAN4 is tasked with defining standardized test decoders, datasets, and reference encoders to ensure interoperability, as well as demodulation requirements for CSI compression (Case 0) and RRM test cases for LCM procedures. Offline training remains the assumption.

Rel-20 (WI: NR_AIML_Mob): RAN4 will specify core requirements and performance tests for RRM measurement prediction and event prediction.

As described above, while some AI/ML-related topics are still under consideration for 5GA, it is important that useful items are actively standardized from day 1 of 6G. In doing so, rather than simply reusing 5GA parameters, it will be necessary to coordinate closely with RAN1 and RAN2, reconsider assumptions as needed for 6G and develop appropriate test specifications. Furthermore, for measurement relaxation techniques using AI/ML that have not been considered in 5GA, it is also important to introduce any useful approaches from day 1 of 6G.

Observation 1: There have been active discussions on AI and ML in 5GA, but it is preferable to introduce useful measurement relaxation techniques, including those not yet considered, from the first day of 6G.

Proposal 1: When introducing these features in 6G, it is necessary for RAN4 to develop test specifications in close collaboration with RAN1 and RAN2.

- Conclusion

In this contribution, we offer our 6GR view as follows.

Observation 1: There have been active discussions on AI and ML in 5GA, but it is preferable to introduce useful measurement relaxation techniques, including those not yet considered, from the first day of 6G.

Proposal 1:  When introducing these features in 6G, it is necessary for RAN4 to develop test specifications in close collaboration with RAN1 and RAN2.

References

[1] 3GPP, R4-25116522, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple).