3GPP TSG RAN WG4 Meeting #116-bis	R4-2513302

Prague, Czech, 13 – 17 October, 2025

Agenda item:			8.13

Source:	vivo

Title:	On RAN4 operation efficiency

Document for:	Discussion

# Introduction

In the RAN#109 meeting, the revised SID [1] for 6G radio study was approved. One of the objectives is to study 6G core and performance requirements. The specification improvement, including spec structure and drafting principles are to be studied as highlighted below.

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

According to RAN4 agenda for 6G, there is guidance on discussions on CR operation improvement and other RN4 operation related aspects.

- RAN4 operation efficiency [FS_6G_Radio] # NOTE: The scope includes spec improvement and modernization in general, new tools for band/band combinations, CR operation improvement and other RAN4 operation related aspects

In this contribution, we provide our views on specification improvement and RAN4 operation related aspects.

# Discussion

## RRM spec improvement

### Spec structure

For RRM specification TS 38.133, the spec structure is in general inherited from LTE spec TS 36.133. It is further revised based on newly introduced features, e.g., non-standalone operations. In 6G, standalone operation is of high priority and we can focus on this part firstly.

The top-level structure of the RRM specification should be fine. Section 4, 5, and 6 are for mobility requirements under different RRC states. Section 7 is for timing requirements only. Section 9 is for measurement reporting requirements and section 10 is for measurement performance (accuracy) requirements. All of these sections can be reused for 6G RRM specification.

However, section 8 for signaling characteristics needs further discussion. In our view, there are three different types of requirements are captured in this section. Radio link monitoring and link recovery procedure is more of L1 measurement related requirements. Interruption requirements are for interruptions due to procedures in section 8 and measurements in section 9. Other parts are mainly for activation/deactivation delay requirements, e.g., SCell activation/deactivation delay, and switch delay requirements, e.g., BWP switch delay, TCI state switch delay etc. It is not so clear to capture all these requirements into one section. Further discussion is needed.

L1 measurements such as radio link monitoring and link recovery procedure has relation to L3 measurements where measurement occasions may need to be shared by L3 and L1 measurement. Furthermore, since there is expectation that unified L1/L3 measurements may be supported in 6G. It would be better to group L3/L1 measurement requirements together.

The scheduling availability/restriction requirements are captured in the sub-sections for measurements which causing scheduling requirements. However, these requirements have little difference for the different measurements. It is better to capture all the scheduling availability/restriction requirements in one section to avoid duplicated requirements. Interruption requirements may have impact to scheduling depending on NW implementation. At least, there is similarity to scheduling availability/restriction requirements. It can be captured in the same section.

With the changes on how to capture L1 measurement requirements and interruption requirements, section 8 can be used to capture activation/deactivation delay and switch delay related requirements.

Proposal 1: For the 6G RRM spec, top level of sections 4, 5, 6, 7, 9, 10 in TS 38.1133 can be reused.

Proposal 2: Section 8 is used to capture procedure delay related requirements.

Proposal 3: Scheduling restriction related requirements and interruption requirements are capture in one high-level section.

Proposal 4: L1 measurement requirements including radio link monitoring and link recovery and L3 measurements requirements are captured in one high-level section.

Sidelink relevant requirements are captured in a separated section. Since it has distinct requirements compared to other requirements, it makes sense to follow the same approach in 6G. In addition, there will be new features to be introduced in 6G, e.g., ISAC etc. It needs study depending on progress of the feature how the requirements can be captured.

Proposal 5: Some distinct features can be captured in a separated section, e.g., sidelink requirements. RAN4 to study how new features to be introduced in 6G are captured in RRM requirements specification.

when features are introduced in later releases, there are basically two approaches to capture RRM requirements for new features. If the feature only impacts few sections, then the requirements are incorporated in the existing sections. Most of the features are handled in this way. If it is a relatively large feature which have impacts on multiple top-level sections, e.g., NR-U, RedCap, NTN, etc., it is captured in separated sub-sections under corresponding requirements. We think this approach can also be used in 6G. It is worthy of noting that if suffix is used for the new sub-sections, same suffix should be used on all the sub-sections for the feature.

Proposal 6: New features introduced in later releases are captured either by incorporating in existing section(s) or by creating new sub-sections. Same suffix should be used for the same feature in different sub-sections.

One another aspect that would be considered for RRM specification improvement is to split RRM spec into two parts, one for core part and another for performance part. By further considering good drafting rules, new tools for tabulated requirements etc., the spec size and quality can be improved. It would provide good experience for the reader.

Proposal 7: RAN4 to consider split RRM spec into two files for core part and performance part, respectively.

### Drafting rules

The 5G RRM specification is very large when release-19 requirements are completed. There are more than 20 files in the TS 38.133 zip file. In our view, it is mainly due to two factors. One is there are many duplicated requirements, e.g., sharing factor for L1 and L3 measurements in many sections, and the other factor is the test cases.

For example, in L1 measurement requirements (RLM/BFD/CBD/L1-RSRP/L1-SINR), there are the same requirements regarding the scaling factor for all the SSB based L1 measurements, which is copied below.

For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, and these measurement gaps are overlapping with some but not all occasions of the SSB; and - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. For FR2, - P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod). - P is Psharing factor, when the RLM-RS resource is not overlapped with measurement gap and RLM-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod). - P=11-TSSBMGRP - TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and - TSMTCperiod ≠ MGRP or - TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod - P=Psharing factor1-TSSBMGRP, when the RLM-RS is partially overlapped with measurement gap and the RLM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5 × TSMTCperiod - P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap - P=Psharing factor1-TSSBMRGP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP) - Psharing factor = 1, if the RLM-RS resource outside measurement gap is - not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and, - not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured. - Psharing factor = 3, otherwise. where, If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Even for CSI-RS based measurements, the similar scaling factor is specified with the difference of changing SSB period to CSI-RS period.

Thus, there is obvious benefit to capture all of these just in one place and refer this part in different requirements. It would make the requirements simple and easy to understand. Since it is not a completed requirement, it can be considered as baseline block for requirements, similar to CSSF.

Another example is for L3 measurements, the following part is captured in multiple places.

Mpss/sss_sync_with_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync with_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync with_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync with_gaps = 36. Mmeas_period_ with_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 4, Mmeas_period with_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_ with_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_ with_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_ with_gaps = 36. - MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48. For a UE supporting FR2 power class 3, MSSB_index_intra = 48.

It can also be considered as basic block for requirements that it is specified in one place and referred in the different requirements. Furthermore, it is better that this part is captured in tabulated format.

One more example is for scheduling restriction/availability requirements for L1 measurements. The requirements are quite similar for different L1 measurements. It can also be defined with block-based approach.

Proposal 8: Uses block-based approach to define core requirements as much as possible.

For the test cases, similar approaches should be used. There are now about 15 files being related to test cases, which is not good indeed. The import thing is that there are many duplicated parts in different test cases.

If we look at the test configurations/parameters, the common test setup, such as PDSCH/PDCCH/BWP/RACH/Antenna configuration etc. are the same in multiple test cases, at least for the same type of tests, e.g., L1 measurements, L3 measurements. Multiple basic blocks could be built and then referred in test setup in different test cases. This could significantly reduce the size of the test cases. Furthermore, it is beneficial for tests setup alignment.

Proposal 9: Uses block-based approach to define test cases, especially for test setup.

Moreover, the WORD is not very friendly for tabulated requirements, especially for test cases where there is bunch of tables. Since 3GPP is studying spec modernization and new tools. If possible, it is strongly proposed to use new tool to capture test setup.

Proposal 10: A new tool, if possible, is used to capture tabulated test setup in test cases.

## RF spec improvement

### Spec structure

For 6G UE RF spec, one question mark here is how to treat the requirement for new spectrum FR3. In NR, the spec is split by frequency range, e.g., FR1 or FR2. Such approach can also be considered for FR3 which mean allocate a dedicated spec for the new spectrum, but in our understanding, the rationale behind is not purely due the frequency range but is related to the difference between conductive requirement and radiated requirement. So, it is also valid to still split the spec as radiated requirement and conductive requirement, and the FR3 requirement can be capture after the requirement applicability is decided.

Observation 1: The NR UE RF spec is split based on frequency range (FR1/FR2) but the rationale behind should be the difference between conductive requirement and radiate requirement.

Proposal 11: RAN4 to discuss how to capture the UE RF requirement for new spectrum (e.g., FR3):

Option 1: Allocate a dedicated spec for the new 6G spectrum

Option 2: Two spec is created for conductive requirement and radiated requirement respectively, and capture the RF requirement of FR3 based on the decision of requirement applicability.

From the perspective of a RF engineer who will design the real product, current NR is too complicated to be read since different feature are mixed into a single spec. it would be better to decouple the requirement for mandatory feature and optional feature. The spec with mandatory feature is easier to be maintained by us and read by the engineer outside of 3GPP, and the spec for optional feature can be used for engineer when they deicide implement a dedicated feature.

Observation 2: NR spec mixed the requirement for both mandatory and optional feature into a single spec, which make the readability is not good enough for the RF engineer who only want to implement mandatory feature, and make the maintenance is quite difficult.

Proposal 12: It is suggested to discuss whether it is feasible to decouple the mandatory feature and optional feature into different spec.

## 2.3 CR operation improvement

In 5G, draft big CR approach is used for an ongoing WI. Work split is necessary to avoid duplicated draft CRs. However, draft big CR in mainly to capture the contents in separated endorsed draft CRs. The process is done after meeting. In general, few companies may check the draft big CR. Furthermore, for the endorsed draft CRs, there is still very short time for company to check during the meeting which would impact the spec quality and leave a lot of extra efforts during maintenance phase.

Therefore, it is expected the CR handling procedure can be improved. Running CR should be good approach. The main point of running CR is that it is updated per meeting cycles. Company only needs to check the updates during the meeting. This can ensure the quality of CR. Moreover, it could also relieve the delegates burden at the meeting, especially for company with small delegation.

For running CR, the structure to capture the potential requirements should be analyzed and decided at the initial phase of a WI. With this, work split for the WI can be done as early as possible. According to impacted requirements, several companies can take of the running CRs as responsible editor. Then, responsible editors can try to capture the agreements in the previous meeting in a running CR. It is not necessary to capture the agreements during the meeting which may rush into unstable or low-quality requirements. The responsible editors may also share the draft of running CR before the meeting to collect comments.

Proposal 13: Running CR approach is used in RAN4 for 6G CR handling, which is used to capture requirements for the agreements in the previous meeting unless it is the last meeting and is updated per meeting cycle.

Proposal 14: Work split should be done as early as possible to assign responsible editors for the running CRs.

## 2.4 Other RAN4 operation improvement

RAN4 is a very cooperative working group. The delegates are hardworking which ensure RAN4 can successfully deliver specifications for feature commercialization. On the other hand, there is continuing comments on RAN4 workload. It would be better to consider some operation improvement procedures.

One thing we can think of is on decision making on Friday. Lots of decision making were delayed to Friday to give companies more time to work offline. However, it increases burden on Friday. So, it would be better we should try to avoid delay decision making to Friday unless it is necessary. Another part is CR handling on Friday. Since there are quite lots of CRs and some CRs may need to be changed in several rounds, which makes Friday is extremely busy in RAN4 compared to other working groups. RAN4 should study how to improve this. With running CR approach being used, it could relax this a bit.

Proposal 15: RAN4 to study procedures how to relieve the workload on Friday, e.g., making decision early during the meeting etc.

# Summary

In this contribution, we further provided views on provide our views on specification improvement and RAN4 operation related aspects. Following proposals are provided.

Proposal 1: For the 6G RRM spec, top level of sections 4, 5, 6, 7, 9, 10 in TS 38.1133 can be reused.

Proposal 2: Section 8 is used to capture procedure delay related requirements.

Proposal 3: Scheduling restriction related requirements and interruption requirements are capture in one high-level section.

Proposal 4: L1 measurement requirements including radio link monitoring and link recovery and L3 measurements requirements are captured in one high-level section.

Proposal 5: Some distinct features can be captured in a separated section, e.g., sidelink requirements. RAN4 to study how new features to be introduced in 6G are captured in RRM requirements specification.

Proposal 6: New features introduced in later releases are captured either by incorporating in existing section(s) or by creating new sub-sections. Same suffix should be used for the same feature in different sub-sections.

Proposal 7: RAN4 to consider split RRM spec into two files for core part and performance part, respectively.

Proposal 8: Uses block-based approach to define core requirements as much as possible.

Proposal 9: Uses block-based approach to define test cases, especially for test setup.

Proposal 10: A new tool, if possible, is used to capture tabulated test setup in test cases.

Proposal 11: RAN4 to discuss how to capture the UE RF requirement for new spectrum (e.g., FR3):

Option 1: Allocate a dedicated spec for the new 6G spectrum

Option 2: Two spec is created for conductive requirement and radiated requirement respectively, and capture the RF requirement of FR3 based on the decision of requirement applicability.

Proposal 12: It is suggested to discuss whether it is feasible to decouple the mandatory feature and optional feature into different spec.

Proposal 13: Running CR approach is used in RAN4 for 6G CR handling, which is used to capture requirements for the agreements in the previous meeting unless it is the last meeting and is updated per meeting cycle.

Proposal 14: Work split should be done as early as possible to assign responsible editors for the running CRs.

Proposal 15: RAN4 to study procedures how to relieve the workload on Friday, e.g., making decision early during the meeting etc.

Observation 1: The NR UE RF spec is split based on frequency range (FR1/FR2) but the rationale behind should be the difference between conductive requirement and radiate requirement.

Observation 2: NR spec mixed the requirement for both mandatory and optional feature into a single spec, which make the readability is not good enough for the RF engineer who only want to implement mandatory feature, and make the maintenance is quite difficult.

# References

- RP‑252476	Revised SID: Study on 6G Radio,	NTT DOCOMO, CMCC, AT&T, Vodafone