3GPP TSG-RAN WG4 Meeting #116bis                                        R4-2513304

Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Huawei, HiSilicon

Title: 	Consideration on system parameters for 6G

Agenda Item:	8.3

Document for:	Approval

Introduction

SI on 6G study was formally approved in RAN#108 meeting [1], and this is the first meeting in RAN4 to start the 6G study.

This contribution provides our consideration on system parameters for 6G.

Discussion

Channel bandwidth

Min/Max channel bandwidth per band

Channel bandwidth is a topic closely tied to progress in both RAN1 and the RAN plenary. It significantly impacts both BS and UE RF/Baseband implementation, and is intrinsically linked to other key RAN1 parameters such as numerology, FFT size, and initial access design, etc.

Recent progress on subcarrier spacing (SCS), as reflected in [2], is still under discussion and requires further study. On the topic of minimum channel bandwidth, although RAN4 previously provided spectrum-oriented analysis during the NR era, RAN1 ultimately emphasized the restrictions imposed by SS/PBCH Block design, tying the minimum bandwidth to specific SCS and frequency range constraints.

Regarding maximum channel bandwidth, previous NR discussions in RAN4 centered on implementation feasibility—maximum output power, spurious emission, impairment related to channel bandwidth or system bandwidth, EVM and SNR requirement for high order modulation, particularly FFT size limitations, taking into account complexity, cost, and power consumption aspects. It is essential that RAN4 conducts its evaluations, particularly from an implementation and spectrum efficiency standpoint, while continuously integrating RAN1 and RAN-level assumptions and agreements.

Observation 1-1: Channel bandwidth and related parameters are being discussed across RAN4, RAN1 and the RAN plenary.

Proposal 1-1: RAN4 conduct studies on channel bandwidth and related parameters (e.g., numerology) from an implementation perspective, tracking progress within RAN1 and RAN. All decisions should be closely coordinated with relevant working groups and the RAN plenary to ensure consistency and feasibility.

Improved procedure to introduce new CBW in future releases

Based on the RAN4 agreements regarding NR UE channel bandwidth, the following key principles were established for NR:

- UE maximum channel bandwidth is a UE capability.

- UE shall support any Rel-15 channel bandwidth that is smaller than its UE supported maximum channel bandwidth.

- The maximum channel bandwidth supported by UE can be different for UL & DL and SCS specific.

- Different maximum channel bandwidths can be defined for different SCS

- Maximum channel bandwidths for certain SCS can be smaller than the ones listed above but not larger

- UE minimum bandwidth in the context of RMSI and COREST containing PDCCH scheduling RMSI should be no less than 20MHz for Frequency Range 1 and 100MHz for Frequency Range 2

RAN further defined mandatory channel bandwidth and signaling support with the following provisions during the NR discussion:

- All bands below 6GHz shall mandatorily support all bandwidths listed in TS 38.101-1 v15.0.0 Table 5.3.5-1 for single component carriers.

- Future channel bandwidths and UE types supporting different mandatory bandwidths would be discussed separately.

- RAN2 was tasked to introduce signaling for maximum supported DL/UL bandwidth per band and per SCS, ensuring forward compatibility for future bandwidth additions.

Accordingly, RAN2 introduced a bitmap to indicate supported UE channel bandwidths, even though all bandwidths below the maximum are nominally mandatory. After the first release, RAN4 initiated a channel bandwidth basket work item to accommodate operator requests for new CBWs in existing NR bands. However, the support level for these new bandwidths—whether mandatory, optional, or mandatory from a specific release—has been decided on a case-by-case basis without consistent principles, leading to prolonged and inefficient discussions.

In addition, with the mandatory mechanism, UE must support a maximum design which cause a lot of work when introducing new commutation capability for different device type.

Observation 1-2: The introduction of new channel bandwidths in NR lacks clear guidelines regarding mandatory support and release applicability.

Proposal 1-2: Study in 6GR the establishment of an improved framework and procedure for the consistent and efficient introduction of new channel bandwidths in future releases.

Relative channel bandwidth

In NR, the mandatory channel bandwidth for UEs and the agreement on maintaining constant maximum output power reduction (MPR) across channel bandwidths raised concerns among companies regarding whether the increased relative bandwidth in NR would allow MPR to remain consistent. The maximum uplink relative channel bandwidth is defined as:

Max relative channel bandwidth = 2*maxBWChannel / (FUL_low + FUL_high)

In Rel-15, it was agreed that if the maximum UL relative channel bandwidth is ≤ 3% for FDD bands and ≤ 4% for TDD bands, the general MPR requirements would apply. This guideline remained in effect until Rel-17, when uplink 30 MHz was introduced for the 700 MHz band (n28), leading to the adoption of delta MPR as a solution.

For 6GR, it is anticipated that even larger channel bandwidths may be supported in certain operating bands. It remains to be examined whether traditional limiting factors, such as achievable Q factor in output matching, power amplifier efficiency, and other RF performance constraints, will continue to be relevant given advances in both passive and active RF component technologies.

Proposal 1-3: Study in 6GR whether the maximum relative channel bandwidth criteria remain valid or require updates in light of advancements in RF component technologies.

A recently identified issue in NR related to relative channel bandwidth involves TDD asymmetric channel bandwidth configurations. Band n50 remains the only TDD band in NR designed to support asymmetric channel bandwidth, a requirement enforced by relative CBW criteria at the time of its introduction in Rel-15. However, due to spectrum utilization constraints that will be discussed subsequently, the NRB of uplink and downlink channel bandwidths in this band ended up with asymmetric parity.

This configuration conflicts with RAN1 and RAN2 constraints for TDD bands, which mandate center frequency alignment for both BWP and carrier bandwidth. Since most TDD bands assume symmetric channel bandwidth, supporting such asymmetric deployment is considered infeasible in practice. Although this issue has been resolved for NR as an exception, it underscores the importance for 6GR to prevent similar inconsistencies across working groups from the earliest stages of 6G design.

Spectrum utilization

Higher SU than 5G

Spectrum utilization (SU) was a major focus in NR Rel-15. Significant efforts were devoted by companies to exhaustive evaluations through both simulations and measurements. These efforts resulted in a substantial improvement in NR SU compared to LTE (90% SU across all channel bandwidths except 1.4MHz). Notably, the maximum SU in NR reaches 98.3% for a 100MHz channel bandwidth with 30kHz SCS.

In the past RAN meeting, an agreement was reached regarding the SU target for 6GR [3]:

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

Table below listed the SU for all supported NR channel bandwidths.

Table 1: Maximum transmission bandwidth configuration NRB and SU ratio

| SCS (kHz) | 3 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 35 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) |
| 15 | 15 | 90.0% | 25 | 90.0% | 52 | 93.6% | 79 | 94.8% | 106 | 95.4% | 133 | 95.8% | 160 | 96.0% | 188 | 96.7% |
| 30 |  |  | 11 | 79.2% | 24 | 86.4% | 38 | 91.2% | 51 | 91.8% | 65 | 93.6% | 78 | 93.6% | 92 | 94.6% |
| 60 |  |  |  |  | 11 | 79.2% | 18 | 86.4% | 24 | 86.4% | 31 | 89.3% | 38 | 91.2% | 44 | 90.5% |
| SCS (kHz) | 40 |  | 45 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
|  | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) | NRB | Y (%) |
| 15 | 216 | 97.2% | 242 | 96.8% | 270 | 96.5% |  |  |  |  |  |  |  |  |  |  |
| 30 | 106 | 95.4% | 119 | 95.2% | 133 | 96.5% | 162 | 97.2% | 189 | 97.2% | 217 | 97.7% | 245 | 98.0% | 273 | 98.3% |
| 60 | 51 | 91.8% | 58 | 92.8% | 65 | 94.0% | 79 | 94.8% | 93 | 95.7% | 107 | 96.3% | 121 | 96.8% | 135 | 97.2% |

As shown in the SU table above, spectrum utilization for smaller channel bandwidths remains relatively low compared to larger bandwidths. This is primarily due to implementation constraints, particularly in channel filter design, where appropriate guard bands should be considered to meet spectrum related requirements. Improving SU in 6GR, especially for smaller channel bandwidths, is critically important, given that low-frequency bands with narrower bandwidths can offer enhanced coverage, which is essential for 6GR deployment.

Observation 2-1: SU for NR smaller channel bandwidths (e.g.≤20 MHz) remains comparatively low, generally below 95% for most such bandwidths.

Observation 2-2: 6GR has a clear objective to achieve improved SU relative to NR.

Proposal 2-1: Study in 6GR to further enhance SU in line with 6G targets, particularly for channel bandwidths with SU<95%.

To thoroughly evaluate enhanced spectrum utilization (SU) in 6GR, several technical aspects should be examined within RAN4. These include waveform design, applicable PA models, assumed spectrum related requirements, and feasible implementation capabilities, particularly filtering performance, no matter implemented in the time or frequency domain.

Proposal 2-2: RAN4 to discuss evaluation assumptions and applicable requirements for the initial SU study.

SU issue identified in 5G

As observed in the maximum transmission configuration table for NR, the selection of NRB numbers (whether odd or even) and the corresponding SU for the supported channel bandwidths exhibit a degree of randomization. This has led to a non-monotonic progression of guard band sizes across the NR channel bandwidths.

Table 2: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

This mismatch leads to difficulties for a 30MHz UE when attempting to access a 40MHz network in the 700MHz band, as discussed in Rel-17. To address this issue, a 10kHz channel raster was introduced after lengthy discussion.

To prevent recurrence of such problems in 6G, it is recommended to consider either a monotonically increasing guard band in relation to channel bandwidth or a fixed spectrum utilization ratio across all CBWs.

Proposal 2-3: Study in 6GR appropriate SU and associated guard bands to prevent non-monotonic guard band issues that may cause access difficulties in real networks.

Channel raster and sync raster

Channel raster

Channel raster definitions in NR include both 100kHz/10kHz and SCS-based options, which are specified using ARFCN. For NR bands that share the same frequency range, duplex mode, and band index as those defined in LTE, the channel raster remains 100 kHz or is enhanced to 10 kHz. However, for bands newly introduced in NR, particularly those in higher frequency ranges, an SCS-based channel raster has been defined to support more flexible configurations for wider bandwidths. Typical examples include bands such as n41 (2.6 GHz), n77, n78 (3.5 GHz), n79 (4.9 GHz), n104 (U6G), and various FR2 bands.

Observation 3-1: For bands introduced in NR, especially with higher frequency range, SCS-based channel raster is defined to enable more flexible configuration for wider bandwidth.

In 6GR, it is highly likely that new bands will be studied and specified in novel frequency ranges, such as around 7 GHz, with support for wider bandwidths. Given the absence of coexistence issues with legacy bands, the SCS-based channel raster represents an appropriate choice.

It is also expected that some legacy NR bands will be carried forward into 6GR, including those previously used for LTE. According to recent discussion in both RAN and RAN1, multi-RAT spectrum sharing is likely to be considered only between 6GR and NR. In this context, whether to retain the 100 kHz / 10 kHz channel raster remains an open question worthy of further study.

Proposal 3-1: Study whether 6GR should retain the 100 kHz / 10 kHz channel raster or adopt an SCS-only channel raster for bands evolved from NR in low and mid-frequency ranges.

Sync raster

In NR, the synchronization raster is defined using the Global Synchronization Channel Number (GSCN), which is sparser than the channel raster. This design helps reduce the blind detection effort for SSB. Due to the different granularities between the GSCN-based synchronization raster and the ARFCN-based channel raster, the SSB is not always located at the center of the carrier.

Furthermore, NR defines various SSB patterns (Case A to Case E) to facilitate spectrum sharing with LTE. These patterns are closely tied to physical layer (RAN1) design, and the granularity of the synchronization raster also depends on the SSB periodicity defined in RAN1. For aspects relevant to RAN4, it is advisable to await further progress within RAN1.

Observation 3-2: Certain aspects of the synchronization raster depend on further studies within RAN1.

In NR, 15 kHz and/or 30 kHz SCS are specified for different operating bands. This can lead to inconsistent sync raster configurations between frequency-adjacent bands. A notable example is band n38, for which only 15 kHz SSB SCS was defined in Release 15. In contrast, the adjacent band n41—which operates in a similar frequency range—supports both 15 kHz and 30 kHz SSB SCS. It is well known that 30 kHz was widely adopted for both data and SSB in commercial n41 deployments, which also preceded those of n38. The inconsistency in SSB SCS hindered the deployment of n38, as its development typically builds upon that of n41. As a result, RAN4 invested significant effort in later releases to introduce 30 kHz SSB SCS support for n38.

Observation 3-3: Inconsistent SSB SCS designs can delay the deployment of certain bands, as observed in 5G.

Proposal 3-2: 6GR should aim for consistent SSB design across supported bands, particularly for adjacent or similarly banded frequencies, to facilitate harmonized deployment and avoid fragmentation.

Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, different nominal channel spacings are defined relevant to the channel raster adopted for the supported NR band.

For intra-band contiguous CA with a 100kHz or 10kHz channel raster

For NR operating bands without a 100kHz channel raster (with SCS-based)

Based on the nominal channel spacing, intra-band non-contiguous carrier aggregation can be differentiated from contiguous CA. In intra-band non-contiguous CA, the channel spacing between two NR component carriers located in different sub-blocks exceeds the predefined nominal channel spacing, a situation that may lead to deployment challenges, as identified in Release 18. Similar issues have been discussed in the context of intra-band EN-DC and CA in unlicensed scenarios.

A simple example is when an operator’s spectrum holding aligns exactly with a contiguous CA configuration, but the UE only reports supporting non-contiguous CA. This raises the question: could there be an issue where a UE that supports non-contiguous configurations fails to support contiguous ones?

On the other hand, the concept of minimum channel spacing for CA was previously discussed in LTE. For certain intra-band combinations, it is feasible to calculate a channel spacing smaller than the nominal value without violating the 100 kHz channel raster placement rule. In such cases, with allowable guard band on each side of the aggregated channel bandwidth could enable the use of narrower spectrum segments for CA. From an implementation perspective, this was technically feasible even in LTE, though it may have been limited by ambiguities in the specification.

Observation 4-1: Differentiation of intra-band non-contiguous and contiguous CA based on the nominal channel spacing, which may fail to accommodate certain real-world spectrum allocation used by operators.

Proposal 4-1: Study in 6GR to develop a proper definition of channel spacing for intra-band CA to support flexible operator deployment scenarios.

Flexible channel bandwidth

In existing LTE/NR specification, a fixed set of regular channel bandwidths are defined, e.g. to fulfil the operator’s request, for FR1 there are 15 channel bandwidths defined for NR up to Rel-16.

FR1: {5 MHz, 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz}

But still in certain countries the channelization plan results in spectrum blocks allocated to an operator that do not exactly correspond to the specified regular channel bandwidth, e.g. GSM, CDMA refarming bands. In order to improve the spectrum utilization in this case, RAN4 had some study in Rel-14 for LTE (TR 36.740) and Rel-17 for NR (TR 38.844). 4 schemes (i.e. Overlapping CA, Combined UE CBW, Overlapping UE CBW and Larger CBW) have been studied in the Rel-17 SI, the background and comparison between different schemes can be found in TR 38.844. Based on the operators’ requests, 3 MHz CBW and 7 MHz CBW was introduced in Rel-19 and 6 MHz CBW is being discussed in Rel-20. In normal situation all specified channel bandwidths are mandate to support for UE. Too many bandwidth types increase the design and test/verification workload and implementation complexity for UE implementation. Hence from the UE implementation and test coverage perspective, only a finite set of channel bandwidth would be possible. And among these schemes, overlapping UE CBW and larger CBW has less impact to UE complexity, however, the UE throughput is limited to legacy CBW in UL or both DL and UL, which need more study for 6G. And on BS side, the channel bandwidth set is declared by the vendors, hence it would be acceptable to support the irregular channel bandwidth for the specific band and operator’s spectrum holding. In general, for the RF requirement definition, we should aim to define the RAN4 requirements as many as possible in a BW independent/scalable manner. And for scalable CBW framework, we can consider to define a finite set of channel bandwidths in the specification, and for other channel bandwidth in between two standard channel bandwidths is supported by scalable bandwidth. BS RF requirements are based on corresponding channel bandwidth filter is implemented, and UE RF requirements are based on next wider channel bandwidth.

Observation 5-1: Irregular spectrum blocks may exist in some specific bands, e.g. GSM, CDMA refarming bands.

Observation 5-2: From the UE implementation and test coverage perspective, only a finite set of channel bandwidth would be possible.

Observation 5-3: On BS side it would be acceptable to support the irregular channel bandwidth for the specific band.

Proposal 5-1: it is proposed to study scalable CBW framework from 6G day 1.

UE implementation assumptions (xTyR)

The UE supported Tx/Rx number is an important parameter for the system performance and has been discussed and enhanced through the whole 5G NR time line. For 6GR, we also need to decide how many #Tx/#Rx to be studied considering following aspects.

- Enhanced TPR and new services

- Technology evolution

- Additional new spectrum range on top of the existing bands

Enhanced TPR and new services

RAN#109 discussed the ITU IMT-2030 TPRs, and the target of peak spectral efficiency is decided as 2x of IMT-2020 [4]. The average spectral efficiency is decided as 3x of IMT-2020 assuming 7GHz spectrum. To meet the system performance target higher than 5GR, increasing #Tx/#Rx needs to be considered as a potential method.

From service point of view, the 6GR is expected to support immersive communication and AI since day-1 [5]. The new services will lead to a boost of traffic from single user. UEs supporting more #Tx/#Rx could be adopted to properly provide the services to users.

Proposal 6-1: Consider higher number of #Tx/#Rx as a potential method to meet the ITU IMT-2030 TPR and new services supported by 6GR.

Technology evolution

To decide the target of 6GR, the experience of 5G NR could be taken as a reference. The #Tx/#Rx specified by RAN4 is summarized as below table.

Table 3: Summary of UE’s #Tx/#Rx in 5G NR

| Frequency rage | UE formfactor | 5G maximum capability |  |
| --- | --- | --- | --- |
| <1GHz | Handheld | 3T | 4R |
|  | CPE | 4T | 4R |
| ~2GHz | Handheld | 3T | 6R |
|  | CPE | 4T | 8R |
| ~4GHz | Handheld | 3T | 6R |
|  | CPE | 4T | 8R |
| All frequency rage | RedCap | 1T | 2R |
| Note: In 5G NR, 4Rx is mandatory for bands n7, n38, n41, n48, n77, n78, n79, n104 (>=2.6GHz) |  |  |  |

The highest capability is 3T6R for Handheld UE and 4T8R for CPE. This represents the state-of-art UE implementation with regards to hardware performance and antenna design. We think this should be the start point for the 6GR design study for the maximum achievable #Tx#Rx since day-1. In 5G NR, the baseline capability is 4Rx for bands >= 2.6GHz. 2Tx is optional in 5G NR, but is generally implemented by high end UEs. In 6GR, the baseline capability needs further study considering of the device type discussion.

Moreover, the #Tx/#Rx has been continually enhanced in 5G NR. Taking CPE UE as an example, below table shows the enhancement in recent releases.

Table 4: The enhancement of CPE UE’s #Tx/#Rx

|  | Rel-15 | Rel-16 | Rel-18 | Rel-19 |
| --- | --- | --- | --- | --- |
| Tx | 2T >1.7GHz | 2T < 1GHz | 4T | 3T |
| Rx | 4R on >1.7GHz | - | 4Rx on < 1GHz 8Rx > 2GHz | 6R on >2.5GHz |

Considering the first release of 6GR might be frozen around 2029, it’s reasonable to expect there could be further evolution in the UE design and implementation. Therefore, we think a higher maximum achievable #Tx/#Rx for high end UE than 5G NR needs to be considered since the day-1 of 6GR. Meanwhile, the baseline should consider the typical UE implementation.

Proposal 6-2: Higher #Tx/#Rx than 5G NR could be studied since the day-1 of 6GR.

New spectrum range

7GHz is an important spectrum range for 6GR. A reasonable target of 7GHz is to achieve comparable coverage of 5G NR on 3.5GHz spectrum. Comparing with 5G NR on 3.5GHz spectrum, 7~10dB gain needs to be provided to compensate the additional path loss and penetration loss. Higher number of #Tx/#Rx than 3.5GHz spectrum could be a potential method to address the issue. On the other hand, the possibility of increasing #Tx/#Rx might be restricted by the overall number of bands supported by the UE.

Proposal 6-3: For 7GHz spectrum, considering higher maximum achievable #Tx/#Rx than 5GNR 3.5GHz spectrum.

UE device types

The RAN WG study item on “Study on 6G Radio” [1] includes an objective of “g) Target scalable and forward compatible design for diverse device types.” RAN1 and RAN have already discussed the objective and made some progress. This section aims to provide some consideration from RAN4 perspective.

In RAN#109, companies brought proposals on potential devices types and the key parameters defining the device types [6]. The key parameters seem more related to service and RAN1 implementation, such as UE bandwidth, peak data rate, coverage etc. We think it’s worth studying if any RAN4 related parameters need to be added to the list.

In 5G NR, some key RAN4 RF requirements are closely related to various UE characteristics as listed below:

Formfactor

- MPR: PC1.5 MPR is different for smartphone and FWA;

- REFSENS: 4Rx requirement is different for smartphone and FWA;

- Overall Tx/Rx number: 4Tx and 8Rx are supported by UEs of large form factor such as FWA/CPE. XR UE is allowed to support 2Rx on 4Rx mandatory bands.

Cost

- The Redcap as a low-cost UE type supports up to 1T2R.

- The Ambient IoT device as an ultra-low-cost UE type supports 1T1R.

Different UE antenna implementation

- For ATG UE, the power related requirements are specified differently, including power class, minimum output power, ACLR, etc.

- Vehicular UE is allowed to support 2Rx on 4Rx mandatory bands considering the external mounted antennas.

Power consumption

- Power class. PC1 UE is not targeted for smartphone form factor.

Regulation

- Aerial UE has specific requirements for AMPR and Additional spurious emissions.

Proposal 7-1: RAN4 is suggested to study whether to include following parameters for defining device types: formfactor, cost, UE antenna implementation, power consumption, regulation.

From our point of view, the definition of device type should not scatter too much to keep the standards concise and neat, at the same time the definition should be flexible enough to address potential new UE types boosting in the future. Having that in mind we would like to check the parameters mentioned above one by one.

Formfactor: The formfactor is a key factor for determining the maximum supported Tx/Rx number, and therefore has major impact on UE’s RF performance and also the performance of peak data rate and coverage We think this should be included in the device type definition. It’s worth noticing that form factor is not equal to the actual area for antenna design, while the latter is more related to the supported Tx/Rx number. Please note that depending on RAN1/RAN decision of the device types, it’s possible that one device type could support different formfactors. For example, if there is a device type called eMBB UE, it could have formfactor of “handheld UE”, “FWA UE” or “XR UE” and serves different level of performance.

Cost: Based on current RAN1 and RAN discussion, it’s likely that the low-cost UEs have a separate device type, i.e. IoT. Moreover, further subdivision might be needed to distinguish Redcap-like, NB-IoT-like and AIoT-like devices. In this case any specific restriction such as #Tx, #Rx, etc. could be directly tied to the device types.

Antenna implementation: Currently RAN4 has different sets of RF requirements for omni-directional antenna (default) and antenna array (ATG). In Rel-20 the requirements for antenna array might be extended to UAV devices. Currently in 5G specifications we have to introduce separate sections for ATG and UAV, therefore leaves some redundancy in the specification. In 6G some enhancement could be considered, such as having a general framework for defining the RF requirements of antenna array, and specifying power classes when a new type of UE needs to be introduced. For example, Power Class AA1 (antenna array 1) for ATG, and Power Class AA2 for UAV. In this case, the antenna type needs to be included as part of the device type definition.

Another example is Vehicular UE, which is allowed to support lower number of #Rx due to the external mounted antennas. Considering there are no separate RF requirements, it seems not necessary to have a dedicated UE type from RAN4 point of view.

Power consumption: If form factor is included as parameter for defining device types, the current way of restricting supported power class per form factor would work well. No need to additionally introduce the status of power consumption into the parameters.

Regulation: Some regulatory requirements only apply to certain type of UE, such as Aerial UEs. Considering other RF requirements are the same with normal UEs, it’s possible the regulatory requirements are handled by some signalling indication method. It seems not necessary to have a dedicated UE type from RAN4 point of view.

Proposal 7-2: At least Formfactor and antenna implementation (omni-directional antenna or antenna array) need to be included as parameter for defining device types.

Conclusion

This contribution provides our consideration on system parameters for 6G.

Channel bandwidth

- Min/Max channel bandwidth per band

Observation 1-1: Channel bandwidth and related parameters are being discussed across RAN4, RAN1 and the RAN plenary.

Proposal 1-1: RAN4 conduct studies on channel bandwidth and related parameters (e.g., numerology) from an implementation perspective, tracking progress within RAN1 and RAN. All decisions should be closely coordinated with relevant working groups and the RAN plenary to ensure consistency and feasibility.

- Improved procedure to introduce new CBW in future releases

Observation 1-2: The introduction of new channel bandwidths in NR lacks clear guidelines regarding mandatory support and release applicability.

Proposal 1-2: Study in 6GR the establishment of an improved framework and procedure for the consistent and efficient introduction of new channel bandwidths in future releases.

- Relative channel bandwidth

Proposal 1-3: Study in 6GR whether the maximum relative channel bandwidth criteria remain valid or require updates in light of advancements in RF component technologies.

Spectrum utilization

- Higher SU than 5G

Observation 2-1: SU for NR smaller channel bandwidths (e.g.≤20 MHz) remains comparatively low, generally below 95% for most such bandwidths.

Observation 2-2: 6GR has a clear objective to achieve improved SU relative to NR.

Proposal 2-1: Study in 6GR to further enhance SU in line with 6G targets, particularly for channel bandwidths with SU<95%.

Proposal 2-2: RAN4 to discuss evaluation assumptions and applicable requirements for the initial SU study.

- SU issue identified in 5G

Proposal 2-3: Study in 6GR appropriate SU and associated guard bands to prevent non-monotonic guard band issues that may cause access difficulties in real networks.

Channel raster and sync raster

- Channel raster

Observation 3-1: For bands introduced in NR, especially with higher frequency range, SCS-based channel raster is defined to enable more flexible configuration for wider bandwidth.

Proposal 3-1: Study whether 6GR should retain the 100 kHz / 10 kHz channel raster or adopt an SCS-only channel raster for bands evolved from NR in low and mid-frequency ranges.

- Sync raster

- Observation 3-2: Certain aspects of the synchronization raster depend on further studies within RAN1.

- Observation 3-3: Inconsistent SSB SCS designs can delay the deployment of certain bands, as observed in 5G.

- Proposal 3-2: 6GR should aim for consistent SSB design across supported bands, particularly for adjacent or similarly banded frequencies, to facilitate harmonized deployment and avoid fragmentation.

Channel spacing

Observation 4-1: Differentiation of intra-band non-contiguous and contiguous CA based on the nominal channel spacing, which may fail to accommodate certain real-world spectrum allocation used by operators.

Proposal 4-1: Study in 6GR to develop a proper definition of channel spacing for intra-band CA to support flexible operator deployment scenarios.

Flexible channel bandwidth

Observation 5-1: Irregular spectrum blocks may exist in some specific bands, e.g. GSM, CDMA refarming bands.

Observation 5-2: From the UE implementation and test coverage perspective, only a finite set of channel bandwidth would be possible.

Observation 5-3: On BS side it would be acceptable to support the irregular channel bandwidth for the specific band.

Proposal 5-1: it is proposed to study scalable CBW framework from 6G day 1.

UE implementation assumptions (xTyR)

Proposal 6-1: Consider higher number of #Tx/#Rx as a potential method to meet the ITU IMT-2030 TPR and new services supported by 6GR.

Proposal 6-2: Higher #Tx/#Rx than 5G NR could be studied since the day-1 of 6GR.

Proposal 6-3: For 7GHz spectrum, considering higher maximum achievable #Tx/#Rx than 5GNR 3.5GHz spectrum.

UE device types

Proposal 7-1: RAN4 is suggested to study whether to include following parameters for defining device types: formfactor, cost, UE antenna implementation, power consumption, regulation.

Proposal 7-2: At least Formfactor and antenna implementation (omni-directional antenna or antenna array) need to be included as parameter for defining device types.

- References

[1] RP-251881, New SID: Study on 6G Radio

[2] RP-252963, Moderator's summary for 6G - system design aspects (numerology, spectrum)

[3] RP-252950, Moderator summary on spectrum utilization requirements

[4] RP-252883 Moderator's summary for RAN led 6G SI Key performance indicators

[5] RP-252943 Moderator's summary for RAN led 6G SI Requirements of new and existing services

[6] RP-252873, Summary of 6G Diverse Device Types (including Minimum UE Bandwidth), Apple

[7] RP-252882 TR 38.914 v0.2.0