3GPP TSG-RAN WG4 Meeting #116bis	R4-2513309

Prague, Czech Republic, Oct 13 – 17, 2025

Title: 	Consideration on demodulation study for 6G

Source: 	Huawei, HiSilicon

Agenda item:	8.8

Document for:	Discussion

Background

6G SID was approved in RAN#108, where some objectives are related to demodulation and performance aspects:

- Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

This contribution provides our general views.

- Discussions

Channel model

In NR and LTE, TDL + correlation channel model was always used for demodulation and CSI performance definition. In Rel-19, a study item on spatial channel model was conducted where rCDL-C1 and xTDL-C1 are finally agreed. In Rel-20, rCDL-C1 channel was agreed for PDSCH requirements definition with consideration of limitation identified in SI. Based on the progress listed above, CDL channel could be considered as one candidate for 6GR RAN4 demodulation requirements definition. Therefore, a more comprehensive study is needed in the study item. Except for the issues identified in Rel-19 SI, some new issues are identified here.

First, the target frequency for 5G CDL is 6GHz, may not be aligned with the 6G frequency range.

Second, only CDL-C is studied in 5G, all CDL-A/B/C/D/E should be covered in 6G.

There are very limited number of cases for simulation alignment (Only one MCS and one doppler), which may not be sufficient to help align the CDL implementation between companies.

Therefore, we propose the following:

Study 6G CDL channel covering CDL-A/B/C/D/E based on 6G frequency range

The CDL channel should be based on latest version of 38.901

To ensure the alignment of CDL implementation, channel properties such as PSD, time correlation coefficient and frequency correlation coefficient should be aligned apart from SNR points alignment.

Proposal 1: Study 6G CDL channel considering following objectives.

- Study 6G CDL channel covering CDL-A/B/C/D/E based on 6G frequency range

- The CDL channel should be based on latest version of 38.901

- To ensure the alignment of CDL implementation, channel properties such as PSD, time correlation coefficient and frequency correlation coefficient should be aligned apart from SNR points alignment.

Meanwhile, one limitation for CDL channel is it is non-configurable. For CDL channel, all clusters have fixed angular resulting fixed spatial properties, hence it is possible that some features are not valuable under specific CDL channel, but they are valuable under TDL channel with configurable correlation matrix.  For example, one advanced receiver having small gain compared to baseline receiver under CDL-C channel could have sufficient gain under TDL channel by configuring correlation. xTDL channel is more configurable and flexible, the number of clusters and power ratio between the clusters could be adjusted to satisfy the specific test purpose.  Hence, it’s would be beneficial to specify extension TDL channel in 6G to give RAN4 more opportunity to satisfy the test purpose.

Observation 1: CDL channel is non-configurable which may cause the risk that the performance of some features under CDL channel cannot be adjusted to the desired level for testing by modifying the channel parameters. E.g. The performance gain of R-ML can be adjusted by configuring different correlation under TDL channel but cannot be adjusted under CDL channel.

Proposal 2: RAN4 shall include extension TDL agreed in Rel-19 SCM SI for demodulation performance definition in 6G.

- Conclusion

In this contribution, we provide our views on 6G demodulation requirements, the proposals and observations are:

Proposal 1: Study 6G CDL channel considering following objectives.

- Study 6G CDL channel covering CDL-A/B/C/D/E based on 6G frequency range

- The CDL channel should be based on latest version of 38.901

- To ensure the alignment of CDL implementation, channel properties such as PSD, time correlation coefficient and frequency correlation coefficient should be aligned apart from SNR points alignment.

Observation 1: CDL channel is non-configurable which may cause the risk that the performance of some features under CDL channel cannot be adjusted to the desired level for testing by modifying the channel parameters. E.g. The performance gain of R-ML can be adjusted by configuring different correlation under TDL channel but cannot be adjusted under CDL channel.

Proposal 2: RAN4 shall include extension TDL agreed in Rel-19 SCM SI for demodulation performance definition in 6G.