3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2513322

Prague, Czech Republic, 13th – 17th October, 2025

Agenda Item:	4.2.8

Source: 	Murata Manufacturing Co Ltd.

Title: 	MSD analysis for UL CBW 25MHz of n71

Document for:	Approval

Introduction

In the RAN4#116 meeting, a WF [1] on optional CBW has been approved. In that, it was agreed to mandate symmetrical UL 25MHz / DL 25MHz CBW for n71 and study on PC3 CA_n5A-n71A and PC3/PC2 CA_n71A-n85A CBI MSD due to BCS4/5 new greatest CBW. In this paper, we proposed MSD for these two band combinations.

Discussion

Review WF [1]

Analysis

CA_n5A-n71A

The gap between n71 UL and n5 DL is 171MHz as shown in Figure 1. Thanks to this wide gap, the Tx noise of n71 falling into n5 DL does not increase even change the n71 UL CBW from 20MHz to 25MHz.

Figure 1. CA_n5A-n71A frequency relationship

Observation: The Tx noise of n71 falling into n5 DL do not increase even change the n71 UL CBW from 20MHz to 25MHz.

CA_n71A-n85A

The gap between n71 UL and n85 DL is only 30MHz as shown in Figure 2. This gap is relatively narrow compared to the UL CBW of n71 (i.e. 20MHz or 25MHz).

Figure 2 CA_n71A-n85A frequency relationship.

As our measured and estimated results, when change the n71 UL CBW from 20MHz to 25MHz, the Tx noise of n71 UL falling into n85 DL increased by 2.4dB and the MSD for PC3 increased by 1.9dB due to Tx noise increase.

Regarding PC2, the MSD can be derived according on the LUT in the WF [2]. The ΔMSD is 3dB for 1Tx architecture, 6dB for 2Tx architecture.

MSD proposal

Proposal 1: RAN4 to consider the MSD value highlighted in the following table 1 for PC3 CA_n5A-n71A and PC3 CA_n71A-n85A for n71 UL CBW 25MHz.

Proposal 2: RAN4 to consider the MSD value highlighted in the following table 2 PC2 CA_n71A-n85A for n71 UL CBW 25MHz.

Table 1. PC3 MSD for CA_n5A-n71A and CA_n71A-n85A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 685.5 | 25 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.0 | >ACLR2 |
| … | … | … | … | … | … | … | … | … | … |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 10.16 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 8: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |  |

Table 2. PC2 MSD for CA_n71A-n85A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 13.14,6 16.14,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

Conclusion

In this paper, we discussed the Tx noise increase when mandate the UL CBW 25MHz of n71 for CA_n5A-n71A and CA_n71A-n85A and proposed the MSD value for these two band combinations.

Observation: The Tx noise of n71 falling into n5 DL do not increase even change the n71 UL CBW from 20MHz to 25MHz.

Proposal 1: RAN4 to consider the MSD value highlighted in the following table 1 for PC3 CA_n5A-n71A and PC3 CA_n71A-n85A for n71 UL CBW 25MHz.

Proposal 2: RAN4 to consider the MSD value highlighted in the following table 2 PC2 CA_n71A-n85A for n71 UL CBW 25MHz.

Table 1. PC3 MSD for CA_n5A-n71A and CA_n71A-n85A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 685.5 | 25 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.0 | >ACLR2 |
| … | … | … | … | … | … | … | … | … | … |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 10.16 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 8: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |  |

Table 2. PC2 MSD for CA_n71A-n85A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 13.14,6 16.14,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

References

[1] R4-2511779, “WF on optional channel bandwidth”, Skyworks/Huawei/Murata, 3GPP TSG-RAN WG4 Meeting #116, Aug 25th – 29th, 2025

[2] R4-2505143, “WF on HPUE MSD simplification”, Skyworks/Qualcomm/Murata/…, 3GPP TSG-RAN WG4 Meeting #114bis, Apr 7th – 11th, 2025