3GPP TSG RAN WG4 Meeting#116bis                                R4-2513328

Prague, CZ, 13th – 17th, Oct. 2025

Agenda item:	8.10

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6G MRSS study

Document for:	Approval

Introduction

In RAN#108 meeting, a new SID on Study on 6G Radio [1] was approved. In this contribution, we would like to share some initial understandings on 6G MRSS study.

Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Discussion

## 2.1. Timeline of 6G MRSS study

As shown in the following Figure 2.1-1, the MRSS study in RAN1 will start from 2026, Q1 with 1Q behind of RAN4 MRSS study (2025,Q4). From our understanding, the dynamic spectrum sharing between 5G and 6G is mainly specified from the physical layer perspective (e.g. dynamic PRB/partial PRB level spectrum sharing). In other words, in 2025,Q4, we could either postpone the discussion until 2026, Q2 with sufficient progress reached in RAN1, or RAN4 could start the some initial discussions from RF requirement for MRSS BS, spec impacts and also the carrier orthogonality/numerology between 5G and 6G.

Figure 2.1-1. The timeline for 6G MSS study in both RAN1 and RAN4

Proposal 1: for MRSS study in RAN4 prior to 2026, Q1, RAN4 focus on the RF requirements of MRSS BS, spec impacts and also carrier/numerology between 5G and 6G.

## 2.2. Background of 5G DSS

As shown in the following figure 2.2-1, 4G-5G DSS was initially specified in Rel-15 with the three options and with further enhancement in the following Rel-16/17 release to address other leftover problems or optimizations to minimize the throughput impacts (e.g. Rel-17 RAN4 led CRS-IM solution to replace the RAN1 network rate matching solution). It could be easily found that lots of 5G DSS design or improvement was focusing on how to handle the LTE carrier specific reference signal (CRS) interference in filed. For 5G-6G MRSS, there will be no cell specific signal transmission (CRS) anymore from 5G NR BS, such kind of solution specific for LTE CRS rate matching or mitigation is not applicable for 6G MRSS anymore due to the nature of 5G DMRS design.

For regions that have only deployed 4G thus far, it may be possible to implement a migration strategy that involves deploying 6G without first deploying 5G, however whether to consider the 4G-6G MRSS is up to RAN-P decision instead of WG’s discussion.

Figure 2.2-1. The overview of 5G DSS standardization

Proposal 2: For MRSS study in RAN4, focus on 5G-6G MRSS according to the 6G objective.

## 2.3. Consideration of MRSS study in RAN4

As shown in the following figure 2.3-1/2/3, we provide some high level illustration of 5G and 6G MRSS for the convenience of the discussions. In order to guarantee the spectrum utilization for the component carrier, the subcarrier’s orthogonality between 5G and 6G is necessary. In other words, the commercialized 5G NR sub-carrier spacing should be the baseline for 6G MRSS BS. However, we should also consider the case of clean reframing to 6G without legacy 5G NR, the detailed analysis and proposals can be found in the companion contribution [2].

In addition, as we know know that, in order to improve the coverage for the certain cell specific broadcast channel (e.g. SSB channel), power boosting is one useful and efficient solution to enable it. For MRSS BS, if the power boosted channel between 5G and 6G is overlapped in the time domain, the maximum power limitation of MRSS BS and to minimize the impacts on other DL channels on the same symbol should be considered.

For MRSS BS, if the power boosted channel between 5G and 6G is not overlapped in the time domain, the potential transition time should be also well considered to minimize the impacts on DL performance degradation.

Figure 2.3-1. The illustration of 5G-6G spectrum sharing for FDD low bands below 1GHz

Figure 2.3-2. The illustration of 5G-6G spectrum sharing for FR1 FDD/TDD middle bands around 2GHz

Figure 2.3-3. The illustration of 5G-6G spectrum sharing for FR1 TDD high bands

Proposal 3: for the MRSS BS, the commercialized NR subcarrier spacing (e.g. FR1 FDD with 15kHz, FR1 TDD with 30kHz and FR2-1 with 120kHz SCS) should be the baseline for 6G BS;

Proposal 4: for MRSS BS, if the power boosted channel between 5G and 6G is overlapped in the time domain, the maximum power limitation of MRSS BS and to minimize the impacts on other DL channels on the same symbol(s) should be considered

## 2.4. RF requirement for MRSS

For MRSS BS supporting both 5G and 6G in the NR refarming bands, it is expected that they may have the RF hardware upgrade and different baseband processing for 5G and 6G. Therefore, it is reasonable to have the unified RF requirement for MRSS BS supporting both 5G and 6G. In addition, considering the applicability of RF requirement for MRSS BS, it is more reasonable to apply the 6GR BS requirement for MRSS BS instead of 5G BS requirement due to the coming detailed 6GR RF requirement discussion (e.g. in-band/out of band blocking requirement) and improvement of 5G BS RF requirement (e.g. co-location relevant requirements etc ).

Proposal 5: for MRSS BS, apply new 6GR BS RF requirements to MRSS BS supporting both 5G and 6G.

Proposal 6: for MRSS BS, propose to consider the TN BS with 5G-6G TN MRSS in the existing TN MSR specification and NTN SAN with 5G-6G NTN MRSS in the new NTN MSR specification.

Conclusions

In this contribution, we shared some initial views on the 6G MRSS BS and proposals are made as following:

Proposal 1: for MRSS study in RAN4 prior to 2026, Q1, RAN4 focus on the RF requirements of MRSS BS, spec impacts and also carrier/numerology between 5G and 6G.

Proposal 2: For MRSS study in RAN4, focus on 5G-6G MRSS according to the 6G objective.

Proposal 3: for the MRSS BS, the commercialized NR subcarrier spacing (e.g. FR1 FDD with 15kHz, FR1 TDD with 30kHz and FR2-1 with 120kHz SCS) should be the baseline for 6G BS.

Proposal 4: for MRSS BS, if the power boosted channel between 5G and 6G is overlapped in the time domain, the maximum power limitation of MRSS BS and to minimize the impacts on other DL channels on the same symbol(s) should be considered.

Proposal 5: for MRSS BS, apply new 6GR BS RF requirements to MRSS BS supporting both 5G and 6G.

Proposal 6: for MRSS BS, it’s proposed to consider the TN BS with 5G-6G TN MRSS in the existing TN MSR specification and NTN SAN with 5G-6G NTN MRSS in the new NTN MSR specification.

References

- RP-251881, New SID: Study on 6G Radio, Approved.

- R4-2513269, Views on 6G system parameter, ZTE Corporation.