3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513331

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	8.7

Source:	Ericsson

Title:	Discussion on 6G RRM requirements

Document for:	Discussion

# Introduction

In this contribution, we provide an overview of potential directions for procedure enhancements and the user experience improvements in 6G RRM.

# General

# Methodology of 6G RRM requirement design

In 6G, the need and support for new scenarios introduces additional costs and complexities, making it essential to first conduct a comprehensive background analysis for each feature and function. This analysis should address the conditions or assumptions under when it is required, and what level of requirements needed.

During each RRM requirement discussion, the first objective should be whether the designed features or functions that operate with a baseline requirement or a relaxed requirement with the conditions. Additionally, the strict conditions can be optionally leveraged to enhance the feature or function when available.

This strategy can be tailored to specific UE requirements under different conditions. For instance, RAN4 can define a baseline requirement to consider typical use cases. Furthermore, RAN4 can also introduce a strict requirement which meets some specific conditions based on some real field deployments if there is a performance benefit and the preconditions for such a requirement can be provided with reasonable cost and complexity.

Robustness and resilience will be crucial for many 6G features and functions. On the one hand, a more relaxed requirement leads to a feature or a function that can work in more deployment configurations which enables the UE to have a flexible design. On the other hand, a feature or a function with strict level of requirement can guarantee the better UE performance in some advanced scenarios. However, RAN4 is sometimes eager to define the requirements about corner cases which can make the requirement overly relaxed and meaningless in typical scenarios.

Therefore, we made the following proposals.

- RAN4 should consider both the baseline requirement and the strict performance requirement with conditions in real field.

- RAN4 should define reasonable requirements based on the typical scenarios other than focus on minimum requirement in corner cases.

- RAN4 should strive for the quantifiable requirement to avoid the vague specification.

Proposal 1: When RAN4 defines the RRM requirement,

- RAN4 should consider both the baseline requirement and the strict performance requirement based on  real field conditions without UE capability.

- RAN4 should define reasonable baseline requirements based on the typical scenarios other than focus on minimum requirement for corner cases.

- RAN4 should strive to establish quantifiable requirements to avoid the vague specification.

# UE architecture

The RRM requirements are strongly connected to UE architecture. Historically in 3GPP, the balance between the use cases of a feature and what a certain UE architecture can and cannot do has been well studied. In each early new generation discussion, UE architecture was often part of the general RRM and RF requirement discussion.

From our understanding, there are several examples:

- Single or multiple LNA in RF frontend.

- Single or multiple FFT and IFFT in baseband.

- The number of searchers in baseband.

A single FFT drives all carriers towards a fraction of CP. A single LNA handling several carriers might work better of aligned at UE RX, limiting MRTD, so CP time period can be used to control LNA gain on one carrier without perturbing data throughput on the other, but this limits site to site distance for non-colocated deployments, on the other hand. The RF retuning time is also highly impacted the interruption performance. The number of FFT will impact most of the performance for L1 level procedure, such as L1 inter-frequency measurement. Another important aspect contributing to RRM requirement is the searcher assumption. In NR, RAN4 requirements assume that one searcher is dedicated to the PCell frequency, while the second searcher handles all other intra-frequency carriers. This constraint limits the UE’s ability to perform parallel measurements across multiple carriers or RATs.

Observation 1: Aspects of UE architecture like FFT, searcher and LNA implementations put conditions on what kind of requirements UE can be achieved.

Because of this interplay between UE architecture and feature use case requirements we think it would benefit 6G standardization if we could settle aspects of UE architecture early in the form of a reference UE architecture for RRM. The intention is not to constrain implementations to follow the particular reference architecture, but rather to allow a framework where standards discussions can meaningfully take place.

Proposal 2: RAN4 to discuss UE reference architectures in early stage of 6G. The RRM requirements should be compatible with the UE reference architecture.

In NR, several advanced features—such as multi-RX chain reception, TDCP, and CJTC reporting for non-ideal backhaul—have been introduced across different releases. However, there remains a noticeable gap between the feature design in other WGs, and the corresponding requirements defined in RAN4. When other WG develops a feature, it often assumes that the UE can support a specific architecture necessary for the feature to function. In contrast, RAN4 discussions frequently rely on Rel-15 baseline UE assumptions, focusing on worst-case or basic UE implementations.

We would like to respectfully emphasize that not all features are expected to be supported by all UEs. For 6G, it would be valuable to define requirements that enable a feature to operate effectively. If there are concerns regarding a feature or the UE architecture needed to support it, these should ideally be raised during the WI approval phase. This would help to avoid situations where substantial effort is invested in other WG to design a feature, only for it to be challenged later in RAN4 due to misaligned architecture assumptions and impractical requirements.

In many cases, RAN4 discussions begin with whether requirements should be defined at all, rather than focusing on how they should be defined. Addressing these gaps earlier in the process would lead to more coherent and implementable specifications.

Proposal 3: RAN4 should define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature.

# Timing and synchronization

# MRTD

Deployments can be colocated and non-colocated. Non colocated deployments, adds flexibility when it comes to cell planning, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Figure 1: Large MRTD and path loss difference use case.

Observation 2: Deployments can be colocated and non-colocated. Non colocated deployments, adds flexibility, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Consider whether the synchronization requirements of a feature or function really need to be specified in the standard. Often, a sync requirement cannot be adequately captured by a single figure, as there generally are complex and varying input parameters associated with it. In the worst-case scenario, specifying a value based on the most challenging dimensioning can lead to increased costs and complexity, which will not be necessary in most scenarios. If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Let us discuss an important example. In Figure 2, we know that:

RTD= TAE+ ∆RF_Propagation+Channel_dispersion

If the RAN4 requirement is specified as Maximum Receive Time difference, MRTD then it is possible to trade TAE and ∆RF_Propagation, within a total budget.

Figure 2: Receive time difference example

For requirement clarity link each requirement to specific features or functions (which may be optionally supported), clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints, like robustness—and, where applicable, to specific UE capabilities.

Proposal 4: When feasible, in timing requirement, use a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Proposal 5: When feasible, specify MRTD (RRM) as a total budget and avoid stating TAE (BS RF) between ARP.

# TDD Cell Phase Synchronization

TDD Cell Phase Synchronization time uncertainty budgets are depicted in Figure 3. The current requirement of 3 µs is based on a budget consisting of three terms, the synchronization input error, the operational limit and the holdover margin. The input error represents the uncertainty in the external clock source. The operational limit is the performance when the external clock source is running with normal operation base station uncertainties added. Finally the holdover margin is e.g. how far the clocks can drift from the operational limit until the TDD Cell Phase Synchronization Requirement is violated, i.e. ± 1.5 µs (relative 3 µs).

Observation 3: part of the current 3 µs TDD Cell Phase Synchronization requirement is for holdover margin.

For a network with a base level of TDD Cell Phase Synchronization we can conclude that:

- A large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network.

- A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics.

- We get a higher level of redundancy as more alternative synchronization sources can be used (including GNSS independent sources). This leads to a more robust network.

- It´s impact on overhead in a TDD system is very small

For a network with a strict level of TDD Cell Phase Synchronization we can conclude that we get:

- Lower holdover margin and a less robust network when it comes to temporary loss of synchronization.

- A strict requirement on input synchronization limits deployment flexibility..

- We get a low level of redundancy, since few or no alternative synchronization sources can be used, resulting in a less robust network.

Figure 3: System robustness and resilience in relation to Cell Phase Sync

Observation 4: A large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network. A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics. We get a higher level of redundancy as more alternative synchronization sources can be used. This leads to a more robust network.

Observation 5: A lower holdover margin makes the network less robust when it comes to temporary loss of synchronization. A strict requirement on input synchronization lowers the probability that the base stations and the network can operate in certain deployments. The number of possible base station configurations will be more limited regarding how network components are connected with switched fronthaul. We get a low level of redundancy, since few or no alternative synchronization sources can be used, resulting in a less robust network.

As highlighted above, we consider robustness, deployment flexibility, and the ability to leverage existing infrastructure to be more important than marginal TDD overhead reductions. Therefore, we propose to maintain the TDD Cell Phase Synchronization requirement at the same level as in NR NR, i.e., 3 μs. Similarly, we believe that synchronization enhancements are better utilized to improve overall system robustness rather than to achieve the aforementioned minor TDD overhead reductions.

Proposal 6: Keep TDD Cell Phase Synchronization requirement the same as in NR NR.

# NTA_offset

The RRM requirementin RRM specification is related to Cell Phase Synchronization and TDD transient times in RF specifications through the formula:

NTA offset≥Cell_Phase_Synchronization+Transmitter transient period

It also needs to accommodate sufficient time for TX<=>RX switching. This means that, Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together.

Observation 6: NTA offset (RRM) , Cell Phase Synchronization (RRM) and TDD transient times (RF) are inter dependent and has to be discussed together, since NTA offset≥Cell_Phase_Synchronization+Transmitter transient period.

In existing NR specification (TS 38.133, Table 7.1.2-2) there are several coexistence cases listed, since the RRM requirement NTA offset affects UL timing. There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Observation 7: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

# ANR

ANR and PCI optimization are basic functions that enables optimization during initial planning and deployment phase as well as any changes in the network. CGI reading helps resolving PCI confusion in the network and facilitates network planning for the operators. This feature is useful for the operators to have from day one when the 6G networks are deployed, introducing  them in later releases adds little value. Both ANR and PCI optimization solutions require the input from the UE to create the neighbour relation table by CGI reading. In NR, RAN4 involved the CGI discussion in a very late stage and no time to define any CGI reading requirements in Rel-15.

Hence, in order to implement the functionality, it is vital that UE supports the ANR feature from the beginning of 6G and RAN4 should define the CGI reading requirement in the first release.

Proposal 7: RAN4 should define the CGI reading requirement in 6G first release.

# Interruptions

At a very high level, it is very desirable from the performance point of view that the amount of interruptions in 6G UE due to different procedures is reduced, which is a trade-off between the UE complexity/capability and a plurality of procedures, supported bands and bandwidths, and carrier frequencies and cells under operation.

Proposal 8: RAN4 shall strive for reducing the amount of interruptions in UEs.

Proposal 9: The interruptions can be specified at a symbol level.

Proposal 10: The existing interruption requirements may need to be revisited for 6G, if the 6G UEs can achieve a better performance.

# Procedure enhancement from NR

# IDLE/INACTIVE mode mobility

Referring to field data analyses in NR, it is observed that a significant number of UEs camp on cells that are not intended to serve them after entering RRC_CONNECTED state. For example, currently, when UE enters the IDLE mode, the UE is highly possible to cell reselect and camp on a FR1 cell even though there is a good quality FR2 cell. In practice, this means that once UEs transition to RRC_CONNECTED state, the serving cell may soon initiate a handover to another cell that is better aligned with network configuration and service objectives. Such redundant mobility activities could be avoided if UEs initially camped on the cells preferred by the network.

The current priority-based cell reselection scheme, although reflecting the network’s preferences through priorities, has certain limitations. On the one hand, there is a higher risk of mobility once NW configures a lower SnonIntraSearchP, SnonIntraSearchQ thresholds to only monitor the higher priority cells. On the other hand, UE shall search and measure all the inter-frequencies of higher, equal or lower priority and ignore the higher priority configured by NW if NW configures a higher SnonIntraSearchP, SnonIntraSearchQ thresholds. Generally, UE needs to spend merely one minute to finish all configured frequencies measurement based on the defined evaluation requirement shown in Table 1. It’s highly possible UE to ignore the configured cell reselection priority and camp on a lower priority cell. As an example, we observed in real field that UE is highly possible to perform cell reselection and camp on a low-band FR1 cell even though NW configured a mid-band FR1 cell with higher priority and having good quality.

Figure 4: An example of cell reselection priorities configuration with 3 frequencies

Table 1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |

For 6G, it is expected that the priority-based cell reselection scheme will be refined with clear design targets derived from practical field observations. RAN4 shall study the potential enhancement and requirements to improve UE camping behaviour.

Proposal 11: RAN4 shall study possible solutions and requirements to improve UE camping behavior and reduce redundant mobility activities.

# CONNECTED mode mobility

In NR, various mobility procedures have been standardized across different releases. Rel-15 introduced Layer 3 handover as the baseline. Rel-16 added conditional handover to improve robustness and DAPS to minimize interruption time. Rel-18 introduced lower-layer triggered mobility (LTM) for reduced latency, and Rel-19 further enhanced robustness with conditional LTM.

Both low interruption time and high robustness are essential and should be considered from the outset in 6G. UL and DL pre-synchronization, as defined in LTM, accelerates handover execution and significantly reduces interruption time. This mechanism is beneficial and should be part of the baseline design in 6G. With advanced UE capabilities such as ltm-FastUE-Processing-r18, interruption time can be reduced to as low as 10 ms. Achieving such fast processing and shorter interruption should be a design goal for 6G, and any necessary system-level support to enable this should be included in the baseline.

Currently, RAN4 requirements rely on explicit L1-RSRP configurations for DL synchronization during pre-synchronization. We believe this imposes unnecessary constraints on network configuration. In 6G, UE design should aim to maintain DL synchronization without requiring explicit L1-RSRP configuration.

For UL synchronization, if it is completed significantly ahead of the handover or cell switch command, the network may need to trigger multiple RACH preambles. This is an inefficient approach for both UE and network. RAN4 should study mechanisms to reduce this overhead and improve the overall handover efficiency.

Proposal 12: As a baseline to 6G connected mode mobility

- RAN4 should target <=10 ms handover interruption as a baseline system design for 6G and study the necessary assistance information required.

- RAN4 should aim for low signalling overhead mechanisms to maintain DL and UL synchronization.

# Measurement

# Measurement framework

When we designed the NR NR measurement requirements, many of the foundational principles were inherited from LTE — a system conceptualized nearly two decades ago. As we now look ahead to 6G, it’s important to recognize that this generation will serve the industry for the next twenty years or more. Over the past two decades, UE processing capabilities have evolved significantly, and we expect applications and use cases to continue evolving rapidly. Therefore, it may not be optimal to use NR (and by extension, LTE) as the baseline for 6G measurement design.

In NR, the measurement delay requirements consist of three key components:

- Cell Search and Identification Delay

- SSB Measurement Period

- SSB Index Reading Time

For the purpose of this discussion, we focus on FR1. The relevant inter-frequency measurement delays are defined as follows (exact scaling factors are not considered here for simplicity):

- Cell Search and Identification Delay: max (600 ms, 8×SMTC period) ×Scaling Factor

- SSB Measurement Period: max (200 ms, 8×SMTC period) ×Scaling Factor

The RAN4 requirements are defined as minimum performance criteria, assuming that the UE shall meet them even under worst-case conditions. However, practical observations from the field data show that most UEs are designed to meet only the minimum requirements, even when operating under better conditions. This conservative UE behavior, while technically compliant, can lead to inefficiencies in the NW implementation.

Observation 8: In NR, the measurement requirements are too relaxed, even in good channel conditions.

Fast measurement

As illustrated above, the current measurement requirements are too relaxed. One contributing factor for this relaxed measurement requirement is the lower bound inherited from LTE, which may no longer be suitable for 6G scenarios. Another important factor we need to revisit is the searcher assumption for 6G. Given the significant advancements in UE hardware and processing capabilities, some enhancements have been studied in Rel-19 by considering use of a third searcher under certain configurations and conditions.

Increasing the number of searchers can enable:

- Parallel measurement processing, reducing latency in reporting

- Faster carrier aggregation setup, especially in connected mode

- Improved responsiveness to bursty traffic and dynamic network conditions

- Enhanced support for network-driven measurement prioritization

RAN4 should study the measurement enhancement based on the new searcher assumption.

Proposal 1310: For 6G, RAN4 shall define requirements assuming multiple (e.g., more than three) searchers as a baseline.

Rx Beam sweeping

In NR, RAN4 agreed beam sweeping factor as 8 for FR2 measurements. It was based on the assumption that a large number of beams would be implemented on the network side, and that UEs would be equipped with multiple antenna panels on all four sides of the device, requiring the UE to sweep across the entire spherical coverage.

However, over time in practice, this assumption did not hold. In later releases, additional UE capabilities were introduced to reduce the RX beam sweeping factor in different releases under different scenarios and features.

We suggest that RAN4 shall revisit the default RX beam sweeping factor, taking into account practical deployment realities and updated UE implementation capabilities.

Proposal 1410: RAN4 shall revisit the default RX beam sweeping factor design for FR2.

Another issue we would like to highlight is the treatment of wide and narrow beams and their associated RX beam sweeping factors in the RAN4 specifications. These assumptions are currently reflected only as a fixed value (N=8) in the requirements, without a clear link to actual UE behavior. In performance testing, this aspect was not thoroughly evaluated, leaving significant margin in the gain assumptions used in test case design. As a result, RAN1, RAN2, and product implementation teams have limited visibility into actual UE behavior. At the end, the delay was scaled but we are not sure whether UE has really performed any RX beam sweeping as it was not visible in the reported L1-RSRP or L3-RSRP.

Observation 9: The RAN4 specification does not define any UE behavior for RX beam sweeping in the context of L3 and L1 measurements, apart from prescribing a fixed numerical value in the requirements.

NW aided measurement prioritization

Currently, RAN4 requirement is defined as minimum requirement with periodic measurement. In other words, the requirement aims to treat all the target measurement objects with equal priority during the measurement. However, current minimum requirement sometimes didn’t fulfil the issues in the real field. To illustrate this, let’s consider a scenario we’ve encountered in our networks. Suppose four carriers are configured. According to RAN4, the UE is considered compliant with RAN4 requirements if it reports measurements for all four carriers within 3200 ms — that’s 600 ms for cell search and 200 ms for measurement per carrier. However, in practice, we’ve seen some UEs wait the full 3200 ms before reporting anything (in other words, we observe that UE schedule the measurements based on the RAN4 minimum requirements). This delay has real consequences: it slows down the network’s ability to initiate carrier aggregation in connected mode. It can also lead to the network to assumes that a carrier is out of coverage and triggers unnecessary reconfigurations. In certain cases, we have observed that when the SCell is configured in a deactivated state and the reporting configuration includes A1/A2 events, some UEs do not report these events at all. As illustrated in the figure below, one test configuration includes four cells—one PCell and three SCells. The figure clearly shows that some UEs fail to report A1/A2 events.

Percentage of UEs

(ms)

We consider another example to highlight the issue with current measurement behaviour.

Measurements play a critical role not only in mobility but also in carrier activation (e.g., SCell activation). There are two primary approaches can be considered for a carrier activation:

- Known carrier activation: The network configures the measurement object, waits for a measurement report to assess the cells quality before activating it.

- Unknown carrier activation: The network activates the carrier/cell without waiting for a measurement report or the measurement report was before last 5 seconds, assuming the carrier is likely to be suitable. If the carrier coverage is not good, then the carrier activation may fail, and NW may need to activate the other carrier.

In NR, in scenarios where multiple SCells are activated simultaneously, the activation delay for an unknown SCell includes:

- TFirstSSB_MAX_multiple_scells

- TSMTC_MAX_multiple_scells

- TRS × N1 + TRS + 5 ms

SCell activation involves AGC, L3 measurements, and L1 measurements. In a simplified case with a single beam and no L1 measurement, and assuming an SSB and SMTC period of 20 ms per carrier, the UE can activate all four SCells in approximately: 40 ms (AGC) + 20 ms × 4 + 20 ms + 5ms= 145 ms.

However, when the same carrier is configured as a neighboring frequency for measurement, the UE may take up to 3200 ms to report measurements. In other words, there is a different behaviour from the measurements point of view when the carrier is activated, and the same carrier is configured to be measured before activating it. This discrepancy can lead the network to incorrectly assume the carrier is out of coverage, delaying or preventing carrier activation.

Additional Observations:

- SCell activation conditions typically include higher SNR side conditions (e.g., ≥ -2 dB).

- Cell search may only require a single SSB occasion under the SCell activation side condition.

Despite good channel conditions, when the carrier is configured as a measurement object, UEs still delay reporting, reinforcing the need to revisit the current measurement strategy.

Thus, we believe that currently defined minimum requirement cannot meet the real world scenarios described above. Towards this end, in Rel-18, RAN2 introduced the measurement order based indication, which allows the network to recommend the order in which intra/inter-RAT and intra/inter-frequency measurements are performed based on the NW’s current requirement.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not configured, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

In 6G, RAN4 should define beyond the baseline minimum measurement requirements and incorporate adaptive measurement behavior based on the priority or order indicated by the network. While preserving an enough flexibility for the typical UE measurement operations, it is equally essential to enable the UE to adaptively reschedule its measurement tasks based on network preference or evolving real-time conditions.

This flexibility becomes especially critical in scenarios such as carrier aggregation for bursty traffic, where timely and prioritized measurements can significantly enhance responsiveness and overall network performance. By enabling the network to guide measurement priorities and allowing the UE to adjust accordingly, it can be better to support the dynamic and heterogeneous nature of 6G traffic patterns.

Proposal 1510: RAN4 should study the measurement requirements based on NW aided measurement priority on top of the baseline requirement.

Measurement model for L1 measurements:

Another aspect that may warrant reconsideration is the current measurement framework for L3 and L1 measurements. In RAN4, the prevailing assumption is that L3 measurements are performed using wide beams, while L1-RSRP is measured using narrow (fine) beams. Additionally, it is assumed that QCL acquisition occurs during L1-RSRP measurement.

However, this assumption does not fully align with UE behavior in practice. UEs typically do not store QCL properties for all reference signals (RS) measured, but only for those associated with activated TCI states. As a result, measuring L1-RSRP using fine beams across all cells may not be meaningful unless the corresponding TCI states are activated. This issue becomes more apparent when L1-RSRP is configured for candidate cells. Since TCI states for these cells are not yet activated, there is limited value in using fine beams for their measurement. In such cases, wide beam measurements may be sufficient until the activation occurs.

That said, one potential benefit of using fine beams for L1-RSRP, even for candidate cell, is that it can accelerate TCI state activation. For the candidate cells, TCI state activation delay may not be that critical. Therefore, a more context-aware and flexible measurement strategy may be needed, balancing efficiency with responsiveness based on the activation status and network intent. We suggest RAN4 to study the measurement model for L3 and L1 measurements.

Proposal 1610: RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement.

Impact of SSB periodicity on the connected mode measurements

There is significant interest among operators in improving network energy efficiency (NW EE) by increasing the SSB periodicity. In NR, the typical SSB periodicity is 20 ms. One potential approach to reduce energy consumption is to extend the SSB periodicity. However, increasing the SSB periodicity may impact measurements behaviors.

RAN4 and other RAN WG should evaluate the optimal SSB periodicity that balances energy savings without degrading mobility performance (e.g., HOF, RLF due to late handovers and the time spent in best cell, etc.). Specifically, when assessing HO performance, it is important to consider the inter-frequency measurement delay—i.e., whether the number of measurement samples can remain at 8 or be reduced under certain SNR conditions. Additionally, RAN4 should analyze the impact on measurement accuracy based on the new SSB design by other WGs.

Proposal 1710: RAN4 should study the measurements delay and accuracy requirements based on the new SSB design.

Measurement assumptions for 7 to 15 GHz

With the introduction of a new frequency range (7 to 15 GHz) for 6G services, it is important for RAN4 to discuss assumptions regarding UE beamforming in this band. Aligning UE reference architecture for this spectrum prior to defining requirements would be beneficial. Based on our understanding, the UE antenna architecture may not rely on analog beamforming using antenna panels (as assumed in FR2) across the entire 7 to 15 GHz range. At least for frequencies below 10 GHz, the UE architecture may instead resemble that of FR1—i.e., with a digital Rx port per antenna element. Therefore, we consider that RX beam sweeping is not necessary for measurements in this frequency range, particularly below 10 GHz. For the Frequency range above 10 GHz, RAN4 should align the assumptions of UE antenna architecture.

Proposal 1810: RAN4 to discuss the UE reference architecture for new frequency range of 7 to 15 GHz before discussing the related RRM requirements.

Measurements quantities

In NR, L3-RSRP, L3-RSRQ, and L3-SINR measurement requirements are defined for both L3 measurements and L1-RSRP and L1-SINR are defined for L1 measurements. Among these, L1-SINR was introduced in Rel-16. Although RTD measurements have been discussed across multiple releases, they have not yet been introduced. We believe SINR and RTD are critical for network operation, and to maximize their utility, all devices should support them from the first release of 6G.

Currently, L1-SINR is supported only for serving cells, and not for candidate cells. However, we consider support for candidate cells essential, particularly when the network relies on L1 measurements for HO decisions.

We understand that RAN1 are the responsible groups for introducing new measurement quantities. However, RAN4 can study and recommend these measurements to other WGs.

Proposal 19: RAN4 to discuss and identify measurement quantities to introduce in 6G from Day 1. The measurements shall include at least SINR and RTD measurements.

UE measurements capabilities

UE measurement capability is a scarce resource, while it is important for the entire network performance that the UE is capable of performing many measurements during the same time period. With the growing measurement demand, it is very much expected that the measurements demand in 6G will be at least not less than that in NR. Hence, RAN4 will have to discuss, agree on, and specify minimum requirements on 6G UE capability in terms of:

- the number of parallel measurements, and

- the number of simultaneously measured carriers, cells, and beams.

Proposal 20: RAN4 to discuss, agree on, and specify minimum requirements on 6G UE capability in terms of:

- the number of parallel measurements, and

- the number of simultaneously measured entities (e.g., carriers, cells, beams, etc.).

# Unified Measurement Gap

In NR Rel-15, RAN4 developed a conservative measurement gap mechanism similar to LTE. To provide flexibility in UE design, RAN4 introduced more than 20 MGPs to accommodate different UE implementations. In later releases, RAN4 developed multiple measurement gap mechanisms; however, most of these solutions are independent and complex. As a result, both UE and NW vendors have been reluctant to support these later enhanced measurement gap mechanisms.

Measurement Gap Pattern

In Rel-15, a large number of measurement gap patterns were defined. UE vendors only support few of the gap patterns, such as only the mandatory gap patterns. On the other hand, the measurement gap is mainly due to RF retuning and is not related to any measurement gap repetition period(MGRP). Thus, to simplify the gap patterns, RAN4 can consider decoupling the MGL with the MGRP configuration.

Observation 10: In NR, too many measurement gap patterns are defined but are seldom used in real field.

Proposal 21: RAN4 to study how to simplify the MGP design to avoid unnecessary MGPs.

Measurement Gap

In NR, a typical measurement gap length(MGL) is 6ms. One reason for introducing such a long MGL is that the UE doesn’t have a spare RF chain for measurements. In latter releases, RAN4 developed two types of small gap mechanisms, one is small gap controlled by UE(NeedForGaps) and the other is small gap controlled by NW(NCSG).

In 6G, RAN4 should aim to design a simplified and enhanced gap mechanism. For example, the shorter gap length mainly caused by RF retuning should be supported. At the same time, RAN4 needs to identify the gapless scenarios. We also admit that, in some extreme scenarios, UE may still need a longer gap for measurement. Therefore, a unified measurement gap mechanism to adapt different gap types should be studied.

Another possible enhancement we would like to highlight is the measurement gap control mechanism. In NR, the gap is mainly controlled by RRC. It is difficult to adapt different scenarios with different MGL and MGRP. In Rel-17, pre-configured gap is introduced for dynamic gap configuration, but the use of pre-configured gap is limited to some specific procedures, such as BWP switching and SCell activation/deactivation. In Rel-19 XR, a more dynamic gap scheduling method is introduced, but it merely focuses on the gap cancellation. Thus, when RAN4 designs the 6G measurement gap, we should consider dynamic solution to allow NW to activate/deactivate the gap on demand.

Proposal 22: RAN4 to study a unified measurement gap design to consider the following dimensions:

- Measurement without spare RF chain, Measurement with spare RF chain and gapless measurement

- Both UE controlled and NW controlled measurement gap

- Both static and dynamic gap activation/deactivation/cancellation mechanism

When RAN4 introduced NCSG and NeedForGaps in NR, UE will adaptively report the gap status based on band combination. It would place a higher burden on the network side if hundreds of UEs report different status. In such case, NW cannot make an optimal decision for a specific UE, since it must take all the UEs within the coverage into account. In other words, frequent and detailed UAI reporting may be better for one UE, but it would prevent the overall measurement design from achieving its intended objectives in real flied application. Therefore, RAN4 should study the effective UAI reporting to support the adapted MG framework at the early stage of the gap design.

Proposal 23: For gap design, RAN4 should firstly study how to design the UAI to avoid unnecessary information exchange with NW.

# Spectrum aggregation

Fast Carriers Setup

In NR, the fast carrier setup was introduced in Rel-16 which enables early measurements and reporting during Idle/Inactive mode. This fast carrier setup procedure was further enhanced in both Rel-18 and Rel-19.

During Rel-18 discussion, some possible measurement enhancement solutions before UE entering the RRC CONNECTED mode were discussed. However, UE vendors had different opinions to update the measurement behaviours in IDLE/INACTIVE mode. Thus, UE enhancement behaviour during IDLE/INACTIVE mode was not excluded in the work scope.

We understand that IDLE mode is one of the most important areas for UE power saving and it’s very difficult to change the UE behaviour in later releases in IDLE/INACTIVE mode. In 6G, to avoid later design change and back compatible issues, the fast carrier setup mechanism needs to be studied in the first release along with the potential new spectrum aggregation framework.

Spectrum aggregation

In NR, the spectrum aggregation was realized through the carrier aggregation framework which was partially inherited from LTE. Carrier Aggregation enhancements were carried out in almost every NR releases.

For 6G we see a clear benefit to support different inter-site/ inter-gNB etc spectrum aggregation that will increase the flexibility of network deployment to lower the cost and to achieve better performance. We see the need to relax the requirement on timing alignment between different carriers. RAN4 needs to further study if different carriers are not tightly synchronized, the impact on the carrier activation time. The well balance between rapid carrier accessibility and spectrum aggregation flexibility is necessary for 6G from day1. spectrum aggregation flexibility is necessary for 6G from day1.

Flexible DL/UL pairing

In many cases, the best frequency band for the downlink may not be the best frequency band for uplink. For example, a UE can easily be in a location where the 3.5 GHz band provides very good downlink performance while the uplink is better served by the 800 MHz band and not the 3.5 GHz band. To achieve better uplink coverage and throughput in 6G, flexible paring scheme of DL and UL carriers will be more important.

Therefore, a unified UL-DL spectrum activation requirements for spectrum aggregation and flexible paring of the DL and UL carriers should be studied in RAN4.

Proposal 24: RAN4 to study and support fast carrier set up in 6G Day-1.

Proposal 25: RAN4 to study the feasibility to define a unified UL-DL carrier activation requirement based on spectrum aggregation scheme and flexible DL UL carrier pairing.

Proposal 26: RAN4 to study the potential relaxation of the requirement on timing alignment between carriers to increase deployment flexibility, for example inter-site aggregation.

# Others

# RRM related Energy Efficiency

# Network energy saving

Today in NR, SSB is designated as an always-on transmission on the PCell. During initial cell search, when a UE attempts to discover and connect to its first cell, SSBs in coverage cells are required to be transmitted every 20 ms. For 6G, a sparser baseline configuration may be introduced to allow SSB transmission periodicities of up to 160 ms to enable energy savings. However, such sparse transmissions may negatively impact performance, and various solutions to address these issues are under development.to 160 ms to enable energy savings. However, such sparse transmissions may negatively impact performance, and various solutions to address these issues are under development.

Figure 5: NW energy consumption for SSB periodicities equal to or larger than 20 ms

Proposal 27: RAN4 shall evaluate the new SSB and relevant designs and assess their impact on concerning RRM requirements.

During the later stages of NR standardization (Release 18 and 19), several promising network energy-saving techniques were further developed. However, the NW energy savings features to some extent cannot be achieved in NR due to certain limitations. Firstly, all these mechanisms depend on UE capabilities and the number of UEs that support such features may be very small. Secondly, techniques that could negatively impact legacy UEs were excluded. For 6G, we need to support the network energy saving techniques from Day-1 to avoid back-compatible issues in NR. For example, in Rel-19, OD-SSB was introduced only for Scell which limits the usage of the network energy saving technology.-compatible issues in NR. For example, in Rel-19, OD-SSB was introduced only for Scell which limits the usage of the network energy saving technology.

In 6G, to avoid dense SSB transmission, OD-SSB should be studied to assist more high-performance procedures for UEs in both Idle/Inactive and CONNECTED mode for any carrier. To allow the 6G design to optimally cater for both IDLE and CONNECTED mode mobility needs, separate SSB for IDLE and CONNECTED should be considered. For example, a sparse SSB can be used in IDLE mode. An OD-SSB can be applied when it is needed, such as for connection establishment, CONNECTED mode operation and CONNECTED mode mobility.

Proposal 28: RAN4 to investigate different scenarios and requirements for sparse SSB and/or OD-SSB for IDLE/INACTIVE mode mobility, Connection establishment, CONNECTED mode operation and CONNECTED mode mobility.

SSB-less SCell

In NR Rel-15, an intra-band SSB-less Scell capability has already introduced and RAN4 defined the related Scell activation requirements. In NR Rel-18, an inter-band co-located SSB-less Scell capability was introduced. Such SSB-less Scells can achieve better energy saving for network. UE obtains timing reference from the reference cell or an intra-band SpCell/SCell. Thus the SSB-less SCell does not need to transmit SSB. In this case, UE can perform a fast SCell activation without receiving SSB. With a new WI for fast SCell activation in Rel-20 to support data scheduling prior to the valid CSI reporting, the overall activation delay may be further reduced. However, the SSB-less features have not yet been commercialized, since the effectiveness would be limited if some UEs do not support the feature. Therefore in 6G, SSB-less deployment should be considered as baseline from day 1.

Proposal 29: The SSB-less based carrier activation should be considered as a mandatory feature in 6G.

# UE power saving

In NR, many features were introduced to support UE power saving from Rel-16 to Rel-19. However, these later introduced features are all based on capabilities. To support legacy Rel-15 UE, it’s difficult for network and operators to support these enhanced UE power saving features in NR. To make 6G market success, it is imperative to provide a good end-user experience for device battery life considering different UE types, such as low power IoT devices in Day-1.

In the existing specifications, different measurement delay requirements are applied for NR, RedCap, HST, NTN, ATG, and IoT UEs. Each category further includes variants arising from diverse carrier configurations, enhancement/relaxation criteria. For 6G, it is necessary to establish a unified and scalable measurement requirement framework that can accommodate different UE types in a consistent manner.

Proposal 3019: RAN4 shall study and define a scalable set of measurement requirements in 6G to ensure compatibility across different UE types and configurations and conditions.

From Rel-16 to Rel-19, a large number of UE energy saving technologies were developed. For example, RRM relaxation due to low mobility and cell centre was introduced from Rel-16. Further enhancement for stationary UE and RLM relaxation were introduced in Rel-17. In Rel-19, a new type of low power receiver with further RRM relaxation is introduced. However, both network vendors and operators may be reluctant to activate these features or tend to configure the parameters in conservative manner in commercial networks.

Proposal 3119: RAN4 to define a simple unified RRM relaxation solution for UE power saving.

Most UEs spend long durations not actively transmitting/receiving data but still need to monitor PDCCH in CONNECTED mode or Paging in IDLE mode frequently. An efficient way to reduce the monitoring time is to request the network to wake up the UE only when needed by sending a wake-up signal (WUS). Different wake-up signals were developed in NR different releases, such as DCP, PEI and LP-WUS etc.

In NR Rel-19, a new wakeup signal is designed for both IDLE/INACTIVE and CONNECTED modes which are detected by a dedicated wake up receiver (WUR). The unified signal is based on an OOK waveform overlaid with OFDM sequences which can be detected by both OOK WUR and OFDM WUR. In RAN4, we defined two different types of requirements to support both LP-SS and SSB. The OFDM WUR provides significantly better coverage compared to the OOK WUR. Moreover, for synchronization and measurements in Idle/Inactive mode, the OOK WUR requires a dedicated low power synchronization signal (LP-SS) while the OFDM WUR can re-use the existing SSBs. Considering NW energy consumption, we should avoid the additional OOK transmission on top of SSB transmission.

In NR, the neighbour cell SSB transmission is mainly based on 20ms, which reliefs the LP-WUS drawbacks for mobility detection. UE can perform the neighbour cell measurement based on the dense SSB periodicity within DRX cycle.

However, in 6G, due to the sparse SSB transmission, a new LP-WUS receiver should be investigated, either to enhance the LP-WUR capability to measure the neighbour cell’s SSB directly or to wake up the main receiver more efficiency to support the mobility measurement and pre-synchronization for CONNECTED procedures.

At the same time, to achieve better NW energy saving, NW also doesn’t need to transmit the dense periodic signals if not need to wake up the UE, such as SSB. Instead of a dense periodic signal, the on-demand signals can be transmitted to assist UE together with the wake-up signals. Therefore, when RAN4 discusses the LP-WUR receiver from UE side, the network energy efficiency needs to also be considered. RAN4 needs to evaluate the OFDM-based LP-WUS receiver together with the network energy efficiency in IDLE and CONNECTED mode.

Proposal 32: RAN4 to study and evaluate an OFDM-based LP-WUS/WUR mobility performance together with NES in both IDLE and CONNECTED mode in 6G.

Furthermore, For specific use cases with low traffic activity, such as IoT, extending battery life largely relies on the use of DRX in RRL_IDLE and RRC_INACTIVE mode. This allows the device to deactivate its receiver for extended periods. With the introduction of eDRX and its enhancements, eDRX cycle up to ~3 hours in RRL_IDLE and RRC_INACTIVE mode, as seen in NB-IoT, LTE-M, Rel-18 NR and Rel-18 RedCap. Therefore, 6 GR should support eDRX in idle/inactive mode as a baseline.

Proposal 33: RAN4 should introduce eDRX based requirement in IDLE/INACTIVE mode as a baseline for 6G IoT.

# NTN

From Rel-17 to Rel-19, RRM requirements for NTN have been introduced in alignment with the evolution of TN requirements with necessary adaptations for NTN-specific aspects. In 6G, NTN requirements shall continue to follow the TN RRM requirement framework while considering necessary updates to account for the specific features, characteristics, and aspects of NTN. In particular, unified measurement framework shall be applied for NTN.

Proposal 34: RAN4 shall ensure that NTN RRM requirements remain aligned with the TN RRM framework in 6G, while incorporating updates to address NTN-specific aspects (which have already been introduced in earlier releases or may be introduced in 6G).

GNSS-resilient NTN operation has been initiated for study in Release 20, while GNSS-less, i.e., no GNSS, operation is under discussion in the 6G scope. Accordingly, RAN4 needs to evaluate the impacts on the corresponding NTN RRM requirements, such as downlink/uplink timing and measurements, and study the solutions where applicable.

Proposal 35: RAN4 shall study valid and effectivet NTN RRM requirements remain under both GNSS-resilient and GNSS-less operation.

# Positioning

ITU-R is discussing positioning requirements for IMT-2030, which is going to result in a set of requirements for positioning. Hence, the 6G technology must deliver positioning performance not worse than NR technology, while also meeting the ITU-R requirements already from the first 6G release. The amount of work on positioning in RAN4 will depend on the positioning architecture, selection of positioning methods to be used for 6G, whether dedicated positioning RS will be specified in the first release of 6G, and whether and how much in common positioning will have with radio sensing. Given a variety of positioning methods specified in NR, it may also be so that inter-RAT NR positioning measurements may need to be provided for the positioning server in a Rel-20 6G network (inter-RAT E-UTRAN positioning measurements have also been supported from Rel-15 NR). In either case, the RAN4 scope on Rel-20 6G positioning heavily depends on decisions to be made in other groups (e.g., RAN, RAN1, RAN2).

Observation 11: Rel-20 6G radio communications networks have to provide some positioning support, e.g., to meet ITU-R requirements for IMT-2030.

Observation 12: RAN4 cannot start the work on positioning requirements until the supported positioning scenarios and methods are decided by RAN/RAN1/RAN2.

# Summary

In this contribution, we initially discussed the 6G RRM-related aspects, including lessons learnt from NR, potential procedure enhancements from NR and new user experience improvements in 6G. Based on these discussions, we have made the following proposals and observations:

Observation 1: Aspects of UE architecture like FFT, searcher and LNA implementations put conditions on what kind of requirements UE can be achieved.

Observation 2: Deployments can be colocated and non-colocated. Non colocated deployments, adds flexibility, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Observation 3: part of the current 3 µs TDD Cell Phase Synchronization requirement is for holdover margin.

Observation 4: A large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network. A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics. We get a higher level of redundancy as more alternative synchronization sources can be used. This leads to a more robust network.

Observation 5: A lower holdover margin makes the network less robust when it comes to temporary loss of synchronization. A strict requirement on input synchronization lowers the probability that the base stations and the network can operate in certain deployments. The number of possible base station configurations will be more limited regarding how network components are connected with switched fronthaul. We get a low level of redundancy, since few or no alternative synchronization sources can be used, resulting in a less robust network.

Observation 6: NTA offset (RRM) , Cell Phase Synchronization (RRM) and TDD transient times (RF) are inter dependent and has to be discussed together, since NTA offset≥Cell_Phase_Synchronization+Transmitter transient period.

Observation 7: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Observation 8: In NR, the measurement requirements are too relaxed, even in good channel conditions.

Observation 9: The RAN4 specification does not define any UE behavior for RX beam sweeping in the context of L3 and L1 measurements, apart from prescribing a fixed numerical value in the requirements.

Observation 10: In NR, too many measurement gap patterns are defined but are seldom used in real field.

Observation 11: Rel-20 6G radio communications networks have to provide some positioning support, e.g., to meet ITU-R requirements for IMT-2030.

Observation 12: RAN4 cannot start the work on positioning requirements until the supported positioning scenarios and methods are decided by RAN/RAN1/RAN2.

Proposal 1: When RAN4 defines the RRM requirement,

- RAN4 should consider both the baseline requirement and the strict performance requirement based on real field conditions without UE capability.

- RAN4 should define reasonable baseline requirements based on the typical scenarios other than focus on minimum requirement for corner cases.

- RAN4 should strive to establish quantifiable requirements to avoid the vague specification.

Proposal 2: RAN4 to discuss UE reference architectures in early stage of 6G. The RRM requirements should be compatible with the UE reference architecture.

Proposal 3: RAN4 should define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature.

Proposal 4: When feasible, in timing requirement, use a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Proposal 5: When feasible, specify MRTD (RRM) as a total budget and avoid stating TAE (BS RF) between ARP.

Proposal 6: Keep TDD Cell Phase Synchronization requirement the same as in NR NR.

Proposal 7: RAN4 should define the CGI reading requirement in 6G first release.

Proposal 8: RAN4 shall strive for reducing the amount of interruptions in UEs.

Proposal 9: The interruptions can be specified at a symbol level.

Proposal 10: The existing interruption requirements may need to be revisited for 6G, if the 6G UEs can achieve a better performance.

Proposal 11: RAN4 shall study possible solutions and requirements to improve UE camping behavior and reduce redundant mobility activities.

Proposal 12: As a baseline to 6G connected mode mobility

- RAN4 should target <=10 ms handover interruption as a baseline system design for 6G and study the necessary assistance information required.

- RAN4 should aim for low signalling overhead mechanisms to maintain DL and UL synchronization.

Proposal 13: For 6G, RAN4 shall define requirements assuming multiple (e.g., more than three) searchers as a baseline.

Proposal 14: RAN4 shall revisit the default RX beam sweeping factor design for FR2.

Proposal 15: RAN4 should study the measurement requirements based on NW aided measurement priority on top of the baseline requirement.

Proposal 16: RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement.

Proposal 17: RAN4 should study the measurements delay and accuracy requirements based on the new SSB design.

Proposal 18: RAN4 to discuss the UE reference architecture for new frequency range of 7 to 15 GHz before discussing the related RRM requirements.

Proposal 19: RAN4 to discuss and identify measurement quantities to introduce in 6G from Day 1. The measurements shall include at least SINR and RTD measurements.

Proposal 20: RAN4 to discuss, agree on, and specify minimum requirements on 6G UE capability in terms of:

- the number of parallel measurements, and

- the number of simultaneously measured entities (e.g., carriers, cells, beams, etc.).

Proposal 21: RAN4 to study how to simplify the MGP design to avoid unnecessary MGPs.

Proposal 22: RAN4 to study a unified measurement gap design to consider the following dimensions:

- Measurement without spare RF chain, Measurement with spare RF chain and gapless measurement

- Both UE controlled and NW controlled measurement gap

- Both static and dynamic gap activation/deactivation/cancellation mechanism

Proposal 23: For gap design, RAN4 should firstly study how to design the UAI to avoid unnecessary information exchange with NW.

Proposal 24: RAN4 to study and support fast carrier set up in 6G Day-1.

Proposal 25: RAN4 to study the feasibility to define a unified UL-DL carrier activation requirement based on spectrum aggregation scheme and flexible DL UL carrier pairing.

Proposal 26: RAN4 to study the potential relaxation of the requirement on timing alignment between carriers to increase deployment flexibility, for example inter-site aggregation.

Proposal 27: RAN4 shall evaluate the new SSB and relevant designs and assess their impact on concerning RRM requirements.

Proposal 28: RAN4 to investigate different scenarios and requirements for sparse SSB and/or OD-SSB for IDLE/INACTIVE mode mobility, Connection establishment, CONNECTED mode operation and CONNECTED mode mobility.

Proposal 29: The SSB-less based carrier activation should be considered as a mandatory feature in 6G.

Proposal 30: RAN4 shall study and define a scalable set of measurement requirements in 6G to ensure compatibility across different UE types and configurations and conditions.

Proposal 31: RAN4 to define a simple unified RRM relaxation solution for UE power saving.

Proposal 32: RAN4 to study and evaluate an OFDM-based LP-WUS/WUR mobility performance together with NES in both IDLE and CONNECTED mode in 6G.

Proposal 33: RAN4 should introduce eDRX based requirement in IDLE/INACTIVE mode as a baseline for 6G IoT.

Proposal 34: RAN4 shall ensure that NTN RRM requirements remain aligned with the TN RRM framework in 6G, while incorporating updates to address NTN-specific aspects (which have already been introduced in earlier releases or may be introduced in 6G).

Proposal 35: RAN4 shall study valid and effectivet NTN RRM requirements remain under both GNSS-resilient and GNSS-less operation.

# References

[1]. RP-251881, New SID: Study on 6G Radio, NTT DOCOMO