3GPP TSG-RAN4 Meeting #116-bis	R4-2513337

Prague, CZ, Oct. 13 – 17, 2025

Source:	Nokia

Title:	OTA and testability for 6G

Agenda item:	8.12

Document for:	Discussion

Introduction

In the last RAN plenary meeting #108 in Prague, the 6G study work items in [1] were endorsed. In this paper, we discuss these OTA-related aspects regarding the 6GR core and performance requirements.

- 6GR core and performance requirements

- General RF aspects [RAN4]

    - Intra-3GPP co-existence studies

    - Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

    - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

    - Study how to improve 6G BS core,  conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

    - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

    - Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

    - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

    - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

    - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

    - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

    - Handling irregular channel bandwidths including the definition [RAN4, RAN1]

    - Definition of ‘frequency range(s)’[RAN4]

Discussion

UE OTA enhancement

Based on the current OTA WI in Rel-19 and the market demand, we propose the following high-level scope for SI on UE OTA enhancement:

- The performance requirements of UE OTA, including TRP/TRS and MIMO OTA, for new 6G bands

  - 2Tx/4Tx, around 7GHz (6G frequencies)

  - TRP/TRS Requirement for single bands around 7GHz for 6G spectrum.

- UL MIMO OTA test methods

  - There is no OTA test for Tx MIMO so far.

- Closer collaboration between RAN4 and RAN5 for MIMO OTA test requirements and test methods development.

- Test requirements and test method for the folded UE type should be specified in the SI.

  - CTIA folded phone OTA test methods as reference

  - Define the test plan related to measurement setup, test procedure, and preliminary MU

- 6G UE categories, test methodology based on new UE categories

- Use the AIML BM and mobility OTA test methods as an example and extend them to other AI features.

### 2.1.1 OTA test requirement for new 6G frequency bands

The studies of channel properties between 7GHz to 24 GHz, which is considered as 6G new band (mid-band), have been recorded in 38.901. It is fundamental for 6G wireless communications. The related studies for 6G new bands have been unfolded in 3GPP towards 6G.

In the current specification, there is no OTA test requirement related to 6G spectrum. Considering that around 7 GHz is most mentioned in 6G, the UE OTA requirement around 7GHz is the priority. We should start to verify whether the current OTA test methodology can be reused for 6G bands around 7GHz. If needed, new OTA test methodology can be introduced.

There are no OTA test requirements and OTA test methodologies related to 6G spectrum that have been defined.

In the 5G standard, the UE RF architecture with 1Tx/2Tx/4Tx and 2Rx/4Rx is well studied and defined. For 6G UEs, a similar RF architecture can be reused and prioritized. The related TRP/TRS and MIMO OTA requirements should be studied first, and the corresponding OTA test methods should be prioritized.

The study for OTA requirements around 7GHz frequencies should be started. Verify if the current OTA test methodology can be reused for 6G bands around 7GHz and new OTA test methodology can be introduced if necessary.

TRP/TRS and MIMO OTA requirements for 1Tx/2Tx/4Tx and 2Rx/4Rx RF architecture should be studied first.

### 2.1.2 UL MIMO OTA methods

- In the current 3GPP specifications (38.151 and 38.827), the MIMO OTA test requirements for downlink (DL) MIMO use cases are defined. However, the corresponding OTA test specifications for uplink (UL) MIMO use cases have not yet been developed.

- According to the 5G UE RF architecture specified in 38.101-1 and 38.101-2, the industry has already studied and defined 2Tx/3Tx/4Tx configurations across different UE types and user scenarios. Building on this foundation, it would be logical to prioritize the development of MIMO OTA requirements for UL MIMO use cases, aligning them with the established 5G NR UE types.

OTA test specification related to UL MIMO use cases has not been developed yet in 38.151.

We can prioritise completing the UL MIMO OTA requirements for 2Tx/3Tx/4Tx use cases based on 5G NR UE types.

### 2.1.3 Closer collaboration between RAN4 and RAN5 for MIMO OTA test requirements and test methods development.

- The OTA test specification for MIMO was introduced in TS 38.151 from Rel-18. In Rel-19, RAN5 completed the work item “NR_MIMO_OTA_enh-UEConTest” [3], which implements RAN5’s UE-conformance test methods for those Rel-18 MIMO OTA requirements. Therefore, further OTA test methods related to RAN4-specific OTA test requirements completed in Rel-19 need to be developed.

- Although UEs with multiple Tx and Rx MIMO implementations have been commercial for years, MIMO OTA conformance testing is not yet routinely deployed prior to market release. After the MIMO OTA test development in RAN5 for UEs with multiple Rx MIMO implementation, we therefore urge closer collaboration between RAN4 and RAN5 to accelerate the development of UL and Tx MIMO OTA test requirements and test methods so they become standardised and available for use before UEs are widely marketed.

UEs with multiple Tx and Rx MIMO implementations have been commercial for years, MIMO OTA conformance testing is not yet routinely deployed prior to market release.

We suggest a closer collaboration between RAN4 and RAN5 for MIMO OTA test requirements and test methods development.

### 2.1.4 OTA test requirement for Folded UE

- The folded UE has become popular in the market. The CTIA has defined the OTA specification for both folded mode and unfolded mode since there are use scenarios for both modes.

- In CTIA, testing is required in both the folded and unfolded states, as communication functionality may be utilized in either configuration. Hand phantom placements differ depending on the device in the unfolded state and folded state. If the unfolded device dimensions exceed the allowable size of the hand phantom (e.g., approximately double the standard size), testing in that state shall be conducted only in free space. If the folded phone is half of the size 1 UE, then it should be placed at the top of the hand phantom [2].

- 3GPP has not discussed these details related to the folded phone OTA test. It only specifies that testing should be based on the UE declaring a primary mechanical mode. In principle, it does not exclude the requirement for testing in either the folded or unfolded state, but only one of the two needs to be selected. However, specific details such as the hand phantom placement for the folded configuration have not yet been incorporated.

Folded phones are in high demand in the market. CTIA has developed the OTS test specification for a folded phone based on the unfolded and folded states. However, there is no folded phone OTA test specification defined yet in 3GPP.

Refer to the CITA folded phone OTA test plan, study and develop a folded phone OTA test plan related to measurement setup, test procedure, and preliminary MU for 3GPP.

### 2.1.5 New 6G UE categories definition and corresponding OTA test requirements

- The absence of specific UE categories initially (in Rel-15) made it challenging to manage the smooth introduction of a new device type with distinct capabilities, such as eMBB, RedCap UE types. In the current 5G NR UE definition, lack of solid reference and baseline led to proceeding in standard with devices requirements and capabilities scattered across different domains (RAN1, RAN4, RAN2), with challenging identification of a device type, making it impossible to recognize actual support of a baseline and/or complete set of device capabilities. However, instead of narrowing device design, this wide scope drove the development of an increasingly varied set of device categories (RedCap, eRedCap, XR, CPE, V2X, UAV), each shaped by distinct constraints in power, form factor and connectivity.

- For OTA testing, this implies that corresponding test method adjustments may be necessary to properly address the new 6G UE category definitions. However, this work can only proceed once the 6G UE type framework has been clearly defined. Until then, it may be more practical to defer the start of OTA work to align with the finalized 6G UE category definitions.

New 6G UE category definition will be studied, addressing the development of an increasingly varied set of device categories. OTA test requirements and test methods should be correspondingly adjusted.  However, we can wait until those works are done in other parts of 3GPP.

### 2.1.6 AIML OTA test methodologies generalization

- The development of OTA test methods for AI/ML-based beam management and mobility is already under discussion in Rel-19, and further enhancements are expected to continue in Rel-20.

- Specifically, the OTA test method for beam management (BM) has been developed in relation to FR2 use cases, while the OTA test method for mobility has been developed in relation to FR1 use cases. These two test methods can serve as flagship examples and provide reference baselines for their respective frequency ranges (FR1 and FR2).

- Accordingly, they can be leveraged as reference points for the continued development of OTA test methods for AI/ML use cases in future releases.

The OTA test methods developed for AIML mobility is for FR1 use case, and AIML BM is for FR2 use case. They can be leveraged as reference points for the continued development of OTA test methods for AI/ML use cases in future releases.

Conclusion

The following Proposals were made on the OTA topics:

- There are no OTA test requirements and OTA test methodologies related to 6G spectrum that have been defined.

- The study for OTA requirements around 7GHz frequencies should be started. Verify if the current OTA test methodology can be reused for 6G bands around 7GHz and new OTA test methodology can be introduced if necessary.

TRP/TRS and MIMO OTA requirements for 1Tx/2Tx/4Tx and 2Rx/4Rx RF architecture should be studied first.

OTA test specification related to UL MIMO use cases has not been developed yet in 38.151.

We can prioritise completing the UL MIMO OTA requirements for 2Tx/3Tx/4Tx use cases based on 5G NR UE types.

UEs with multiple Tx and Rx MIMO implementations have been commercial for years, MIMO OTA conformance testing is not yet routinely deployed prior to market release.

We suggest a closer collaboration between RAN4 and RAN5 for MIMO OTA test requirements and test methods development.

Folded phones are in high demand in the market. CTIA has developed the OTS test specification for a folded phone based on the unfolded and folded states. However, there is no folded phone OTA test specification defined yet in 3GPP.

Refer to the CITA folded phone OTA test plan, study and develop a folded phone OTA test plan related to measurement setup, test procedure, and preliminary MU for 3GPP.

New 6G UE category definition will be studied, addressing the development of an increasingly varied set of device categories. OTA test requirements and test methods should be correspondingly adjusted.  However, we can wait until those works are done in other parts of 3GPP.

The OTA test methods developed for AIML mobility is for FR1 use case, and AIML BM is for FR2 use case. They can be leveraged as reference points for the continued development of OTA test methods for AI/ML use cases in future releases.

- References

- RP-251881; New SID: Study on 6G Radio

- CTIA, Test Plan for Wireless Device Over-the-Air Performance, v3.8.2, April 2019.

- R5-247967, Device Positioning for FR2 MIMO OTA Testing.