3GPP RAN4 Meeting #116 bis		R4-2513338

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	8.12

Source:	Ericsson

Title:	Discussion on testability and OTA in 6G

WI/SI:	FS_6G_Radio

Release:	Rel-20

Document for:	Approval

# Introduction

During RAN #108 a new SID [1] on study on 6G Radio was agreed. Some of the objectives of the SI are shown below:

This contribution presents testability and OTA related issues that should be studied for 6G (f).

# Discussion

## Background & Motivation

- 6G introduces:

- Operation at 7-15 GHz frequency range and wider bandwidths in the mmWave frequency band, and novel channel behaviours

- Extremely large antenna arrays (ELAA), distributed arrays, conformal and shared-aperture antennas.

- ISAC functions, where sensing and communications share spectrum/hardware.

- Sub-Band Full Duplex (SBFD) and other hybrid duplex modes.

- AI/ML-based control loops (beam prediction, self-healing, power management).

- NTN in 6G will likely expand into higher frequencies, new orbits/platforms including VLEO, seamless terrestrial-NTN integration, massive IoT support, and native resilience.

- Tight time and reliability requirements for time-sensitive applications (TSN/DetNet over wireless).

- Reconfigurable Intelligent Surfaces (RIS), holographic surfaces, and integrated apertures are also some potential 6G technologies

- OTA and testability must address device- / subsystem-level RF performance, array/beam performance, system-level E2E KPIs (throughput/latency/reliability/sensing accuracy), security, production testability, and lifecycle assurance (in-field self-test and continuous verification).

- Existing 3GPP OTA methods (CATR, reverberation chambre, etc.) require extension and new methods to handle near-field/large-array, spatially-distributed radiation, and AI-in-the-loop behaviours.

Observation 1. 6G will introduce new 7-15 GHz frequency range and wider bandwidths in mmWave frequency bands, and novel channel behaviours.

Proposal 1. Study OTA testing for the new 7-15 GHz frequency range and the wider bandwidths in mmWave frequency band with the appropriate channel models.

Proposal 2. Study enhancements in existing OTA testing method and associated requirements for frequencies below 7 GHz.

## Objective

Study the following aspects for device OTA and testability:

- Definitions and taxonomy of OTA test scenarios for 6G.

  - OTA scenarios in 6G must capture new frequency ranges, near-field effects, NTN mobility, ISAC, SBFD, etc.. A taxonomy can be built across frequency, environment, topology, and use case dimensions. This ensures reproducibility and standard language when comparing lab setups and field trials.

- Standardized OTA KPIs and measurement methods (PHY → Service).

  - KPIs must span PHY-level (EVM, SNR, beam accuracy), link-level (throughput, latency, BLER), and service-level (XR QoE, sensing resolution). Standardized methods—CATR, channel sounding, and protocol-aware traffic—map low-level impairments to user-experienced quality. This traceability enables multi-layer validation from hardware to application.

- Testbed and equipment requirements (CATR, near-field scanners, channel emulators, hybrid setups).

  - 6G OTA testbeds need compact ranges (CATR) for far-field emulation, near-field scanners for ELAA, and channel emulators for reproducible mobility and Doppler. Hybrid setups combining physical chambers with emulated multipath will be essential.

- Methods for testing AI-native features, ISAC, holographic MIMO, ELAA/near-field arrays, SBFD, and RIS.

  - Novel features demand new test approaches: AI-native functions need closed-loop robustness testing; ISAC must jointly measure communications KPIs and sensing accuracy; RIS setups require controllable phase-profile validation. Near-field scanning enables holographic MIMO/ELAA evaluation, while SBFD tests focus on self-interference cancellation depth and full-duplex stability.

- Conformance and interoperability test procedures for device & network elements.

  - Conformance verifies devices meet spectrum, protocol, and OTA requirements, while interoperability ensures multi-vendor systems work together under standardized scenarios. Tests must cover RF compliance, MAC/RRC behaviour, mobility, and slice/service consistency. Plugfests and certification harnesses can formalize cross-vendor validation.

- Robustness against non-3GPP emissions testing.

  - 6G devices must withstand interference from Wi-Fi, radar, and industrial emitters. OTA tests should introduce controlled adjacent-channel, pulsed, or broadband interferers and measure degradation in KPIs. Robustness ensures coexistence in crowded spectrum environments.

- Test data reporting and APIs for reproducible automated testing.

  - Standardized data schemas and APIs enable automated, repeatable test execution. Metadata (scenario ID, DUT version, calibration state) must accompany raw traces and KPIs. APIs should support orchestration, remote monitoring, and CI/CD integration for continuous validation.

- The potential for improving demodulation requirement testing

  - Currently, OTA demodulation testing is based on “cable replacement”; i.e., replacing the cables between the device tester TX and UE RX with an anechoic OTA link. Also, a large AWGN signal is transmitted such that the test is well above the device noise floor. There are several issues with this approach; for example, it is not possible to create independent fading channels on the same polarization (i.e., only 2RX can be tested) and for high SNR, it may not be possible to achieve the power needed to achieve the high SNR with the added AWGN. Studying could consider whether the testing could be improved for high SNR testing (even for example if the AWGN level would be closer to the device noise floor or not present), or alternative and more realistic setups that could emulate multiple fading channels on the same polarization.

Some of the items to be tested are:

- RF front-end and antenna characterization (EVM, ACLR, spurious emissions, Tx/Rx performance, OTA antenna efficiency and patterns, Self-Interference Cancellation (SIC) for SBFD and ISAC).

- Beam & array performance (gain, beamwidth, sidelobe, spatial consistency, beam-switching dynamics).

- Near-field / Far-field transition and measurement methods for ELAA/holographic arrays.

- OTA validation of ISAC performance (sensing accuracy, joint KPI trade-offs).

- OTA testing for SBFD (self-interference cancellation, residual interference, cross-link interference).

- AI/ML validation and testability (training/test data, offline/online validation, reproducibility).

- OTA testing for new NTN use-cases, frequency bands, etc.

- Time-synchronization testing and measurement.

- Energy efficiency / thermal aspects under OTA operation (studied as performance KPIs; outside strict 3GPP conformance, but relevant for evaluation and certification).

Some conducted requirements from the previous generations, ex. Pout, sensitivity, spurious emissions, EVM, Maximum Sensitivity Degradation (MSD), and MIMO verification, are not sufficient for guaranteeing device good performance in network operation. For example, MSD for CA is currently tested using conducted setups, where the UE is connected via RF cables to a tester that provides controlled downlink signals while forcing uplink transmission with antennas disabled. This method is highly repeatable and isolates the effect of the internal RF chain, but it ignores the real antenna environment. A key drawback is that conducted tests do not capture antenna mutual coupling, radiation leakage, housing effects, or user interaction, all of which contribute to real-world desensitization. They also assume ideal impedance matching and duplexer isolation, which are rarely true in practice. By shifting MSD to OTA testing, these missing effects would be included, leading to results that better reflect end-to-end device performance. OTA testing would reveal degradations caused by antenna placement, device form factor, and near-field coupling, giving operators a more realistic view of UE behaviour in the field. It would also allow assessment of self-interference paths through radiation, not just circuit leakage. It would also indicate whether simultaneous Rx-Tx can be supported for a band combination.

Observation 2. Some conducted requirements from the previous generations, e.g. Pout, sensitivity, spurious emissions, EVM, MSD and MIMO verification, are not sufficient for guaranteeing device good performance in network operation. OTA testing provides more realistic device-level performance under integrated antenna/beamforming conditions.

Proposal 3. Study which conducted tests in the previous generations can be moved or complemented with OTA test in 6G, e.g. Pout, sensitivity, spurious emissions, EVM, MSD, UL MIMO, etc.

Observation 3. 6G will introduce AI/ML functionalities such as beam prediction, mobility, positioning, and CSI compression/prediction. Robust testing will be required to ensure reliable operation and interoperability of AI/ML based features across vendors, which may necessitate new test setups. Moreover, testing time and reproducibility will be important considerations, and standardized approaches will be critical, as inadequate validation could limit adoption of certain AI-native features.

Proposal 4. Study OTA testing for the added AI/ML functionalities with the appropriate channel models as Spatial Channel Model (SCM). 6G study shall take the achievements expected in 5G AI/ML OTA testing into account.

Observation 4. 6G will introduce ISAC, SBFD, and other new duplexing modes.

Proposal 5. Study OTA testing for the added ISAC functionalities and SBFD, including joint validation of communication and sensing KPIs, evaluation of self-interference cancellation and residual effects, and performance under realistic mobility and multipath conditions.

Proposal 6. Study OTA testing for 6G NTN for different use-cases, bands including VSAT terminals operating in Ku and Ka bands, etc.

Proposal 7. Study OTA testing for the new antenna types ELAA and if considered for 6G RIS, holographic arrays, etc.

In the current 3GPP conformance framework for FR1, conducted requirements are defined under an assumption that simplifies the role of the antenna. Specifically, TS 38.101-1 states that “for UEs with an integral antenna only, a reference antenna with 0 dBi gain is assumed for each antenna port”. This approach provides a neutral baseline for conducted testing, but it effectively decouples the RF performance of the device from the realities of antenna efficiency and radiation characteristics. Since typical FR1 handset antennas rarely achieve 0 dBi effective gain and often exhibit efficiencies around –5.5 dB, it is worth considering whether conducted requirements could be more meaningfully related to OTA performance by factoring in realistic antenna efficiency values. This would help bridge the gap between laboratory conducted tests and real-world device behaviour.

Observation 5. In FR1 conducted TS 38.101-1, “for UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s)”. It is worth discussing if the conducted requirement could be related to the OTA requirement by at least assuming a reasonable antenna efficiency, e.g. a typical FR1 antenna efficiency is -5.5 dB.

Proposal 8. Discuss a more realistic assumption on antenna efficiency for FR1 band tests, e.g. -5.5 dB.

Proposal 9. For the bands that have TRP/TRS requirements, use the value of TRP to derive the real antenna efficiency.

# Conclusions

This contribution provides our views related to OTA and testability. The contribution makes the following observations and proposals.

Observation 1. 6G will introduce new 7-15 GHz frequency range and wider bandwidths in mmWave frequency band, and novel channel behaviours.

Proposal 1. Study OTA testing for the new 7-15 GHz frequency range and the wider bandwidths in mmWave frequency band with the appropriate channel models.

Proposal 2. Study enhancements in existing OTA testing method and associated requirements for frequencies below 7 GHz.

Observation 2. Some conducted requirements from the previous generations, e.g. Pout, sensitivity, spurious emissions, EVM, MSD and MIMO verification, are not sufficient for guaranteeing device good functioning. OTA testing provides more realistic device-level performance under integrated antenna/beamforming conditions.

Proposal 3. Study which conducted tests in the previous generations can be moved or complemented with OTA test in 6G, e.g. Pout, sensitivity, spurious emissions, EVM, MSD, UL MIMO, etc.

Observation 3. 6G will introduce AI/ML functionalities such as beam prediction, mobility, positioning, and CSI compression/prediction. Robust testing will be required to ensure reliable operation and interoperability of AI/ML based features across vendors, which may necessitate new test setups. Moreover, testing time and reproducibility will be important considerations, and standardized approaches will be critical, as inadequate validation could limit adoption of certain AI-native features.

Proposal 4. Study OTA testing for the added AI/ML functionalities with the appropriate channel models as Spatial Channel Model (SCM). 6G study shall take the achievements expected in 5G AI/ML OTA testing into account.

Observation 4. 6G will introduce ISAC, SBFD, and other new duplexing modes.

Proposal 5. Study OTA testing for the added ISAC functionalities and SBFD, including joint validation of communication and sensing KPIs, evaluation of self-interference cancellation and residual effects, and performance under realistic mobility and multipath conditions.

Proposal 6. Study OTA testing for 6G NTN for different use-cases, bands including VSAT terminals operating in Ku and Ka bands, etc.

Proposal 7. Study OTA testing for the new antenna types ELAA and if considered for 6G RIS, holographic arrays, etc.

Observation 5. In FR1 conducted TS 38.101-1, “for UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s)”. It is worth discussing if the conducted requirement could be related to the OTA requirement by at least assuming a reasonable antenna efficiency, e.g. a typical FR1 antenna efficiency is -5.5 dB.

Proposal 8. Discuss a more realistic assumption on antenna efficiency for FR1 band tests, e.g. -5.5 dB.

Proposal 9. For the bands that have TRP/TRS requirements, use the value of TRP to derive the real antenna efficiency.

# References

[1]	RP-251881, “New SID: Study on 6G Radio”, 3GPP RAN #108.