3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513355

# Prague, Czech Republic, 13th – 17th October, 2025

Title:	Discussion on IoT-NTN PC1.5 Single Tx

Source:	Samsung

Agenda item:	7.6.3

Document for:	Approval

Introduction

In last RAN #109 meeting, the meeting approved the WID [1] for further enhancement of both NR-NTN and IoT-NTN. In this agenda, it includes three major aspects: HPUE for NTN, HD-FDD for NTN and DL intra-band intra-SAN CA. And in this paper, we will focus on the NR-NTN objectives as follows:

HPUE for NTN Core Part

- Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types

  - Example bands are Band 256 and 255

  - Necessary Tx requirements, at least including

    - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

  - Necessary Rx requirements, at least including

    - Specify the RSD requirements on the example bands, if necessary

  - Specify the necessary signaling to support the above objectives

- The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

Co-existence results and ACLR

In the co-existence studies carried out in Rel-19 and documented in [2], the power class 2 and power class 1 NB IoT-based IoT NTN study results indicate it’s feasible to re-use the PC3 ACLR to meet the coexistence criteria. The co-existence results are given below

## 6X.5 Summary of co-existence study for NTN HPUE in FR1 The averaged interpolate ACLR values for each power class of NR-NTN and IoT-NTN in Scenario 4b are presented in Table 6X.5-1 below. It should be noted that only co-ex study results assuming 1.5km isolation distance are taken into account in this summary. Table 6X.5-1: Average ACLR values for NTN HPUE in FR1 Power classes; PC 2; PC 1.5; PC 1. ACLR values (dBc); NR-NTN; <20; <20.6; <21.6. IoT-NTN; <22.8; Note 2; <24.7. Note 1: Some company’s results meet performance metrics at certain starting ACIR/ACLR level they used, e.g. 20dBc. These results are listed as smaller than those starting level. Note 2: The power class 1.5 for IoT-NTN is not studied in this version.. With the above averaged values from co-existence study and recognizing the characteristics of NTN HPUE in FR1 to the state of art and their future developments; and the protection to the operators in adjacent channel, the agreed ACLR of NR-NTN and IoT NTN HPUE are given in Table 6X.5.1-2. Table 6X.5.1-2: ACLR and ACS of NTN HPUE in FR1 Power classes; PC 2; PC 1.5; PC 1. ACLR (dBc); NR-NTN; 31; 31; 343. NB-IoT based IoT-NTN; 371; Note 2; 371. eMTC based IoT-NTN; 31; Note 2; [34]3. Note 1: NB-IoT based IoT-NTN PC2 and PC1 ACLR reused from NB-IoT based IoT-NTN PC3 UTRA_ACLR. Note 2: The power class 1.5 for IoT-NTN is not studied in this version. Note 3: It is observed in some company co-existence results that the 5%-ile throughput degradation is slightly higher than 5% with zero km isolation distance assumption. It was however agreed to progress with this ACLR as it faciliates UE RF implementation..

Proposal 1: For NB-IoT based IoT-NTN, the ACLR requirement can be 37 dBc, same as PC3, PC2 and PC1 as indicated by co-existence study in TR.

Given the WID only mention NB-IoT based IoT-NTN, our understanding is that the eMTC-based IoT-NTN is out of scope. But we would like to further check if this is common understanding from the rapporteur point of view.

Proposal 2: Given only NB-IoT based IoT-NTN is mentioned in WID, RAN4 to clarify eMTC-based IoT-NTN is out of scope for this release.

SEM

For SEM requirements, our view is that it should be quite straightforward to re-use the SEM for PC1.5.

Proposal 3: To re-use SEM for PC1.5.

MPR

Once ACLR was agreed, the evaluation work of MPR should be carried out with certain assumptions. And given the previous work of NB-IoT NTN on PC2 and PC1, in order to be consistent, we believe the previous agreements in WF [3] can be taken as starting point as below:

- [3] dB post-PA insertion losses for calibration purpose - For PC2, PC3 NR PA can also be considered for reference architecture - Calibration point for PC2 (PC3 NR PA) (further update based on offline common view): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 dBc - CIM3: 60 dBc - NR ACLR: 30dBc, other number is not precluded - Calibration based on NB-IOT signal is not precluded. - For PC1, consider adding [-34] dBc IQ-image and LO-leakage and 60 dBc CIM3 as another option for evaluation - Calibration points for PC1 (based on NR PC2 PA) (further update based on offline common view): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 / [-34]dBc - CIM3: 60 dBc - NR ACLR: 31dBc, other number is not precluded - Calibration based on NB-IOT signal is not precluded.

Observation 1: For PC1 NB-IoT NTN, the Rel-17 discussion adopted NR PC2 PA to derive MPR. And it is from the consideration of achieving higher Tx power for the benefits of NTN link and also the comparison from GSM PA.

For PC1.5 IoT-NTN, the consideration should be the same, which is to achieve best power gain for NTN uplink. And from the same consideration and comparison, we believe the meeting should be able to ready to adopt NR PC2 PA for MPR evaluation.

Proposal 4: Adopt the following as starting point for MPR evaluation assumption for PC1.5 NB IoT based IoT-NTN:

    - Calibration points for PC1.5 (based on NR PC2 PA) (further update based on offline common view):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 dBc

      - CIM3: 60 dBc

      - NR ACLR: 31dBc

    - Calibration based on NB-IOT signal is not precluded.

Power boosting

In Rel-19, a power boosting scheme was introduced to NB-IoT based IoT-NTN PC2 UE. And in Rel-20, if feasible, we could also consider a possible power boosting for single tone case.

Proposal 5: To study if power boosting is feasible for PC1.5 single tone.

Conclusion

In this contribution, we provide following proposals on NB-IoT based IoT-NTN:

Proposal 1: For NB-IoT based IoT-NTN, the ACLR requirement can be 37 dBc, same as PC3, PC2 and PC1 as indicated by co-existence study in TR.

Proposal 2: Given only NB-IoT based IoT-NTN is mentioned in WID, RAN4 to clarify eMTC-based IoT-NTN is out of scope for this release.

Proposal 3: To re-use SEM for PC1.5.

Observation 1: For PC1 NB-IoT NTN, the Rel-17 discussion adopted NR PC2 PA to derive MPR. And it is from the consideration of achieving higher Tx power for the benefits of NTN link and also the comparison from GSM PA.

Proposal 4: Adopt the following as starting point for MPR evaluation assumption for PC1.5 NB IoT based IoT-NTN:

    - Calibration points for PC1.5 (based on NR PC2 PA) (further update based on offline common view):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 dBc

      - CIM3: 60 dBc

      - NR ACLR: 31dBc

    - Calibration based on NB-IOT signal is not precluded.

Proposal 5: To study if power boosting is feasible for PC1.5 single tone.

Reference

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[2] TR 38.863

[3] R4-2508122, WF on IoT-NTN HPUE, vivo