3GPP TSG-RAN WG4 Meeting #116bis									R4-2513357

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	4.20.1.1

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements A-IOT devices.

- Discussion

2.1 Tx requirements

- Backscattering power

Last meeting agreements are listed as below:

Issue 3-1-1: Transmit output power Agreement: - Define requirements only at the peak antenna gain direction. - When input CW level is -27dBm, the backscatter loss<=10dB for OOK and <=6dB for BPSK - When input CW level is-10dBm, the backscatter power is larger than -25dBm for OOK, and -21dBm for BPSK respectively.

Last meeting, it was proposed to define max backscatter power upper limit considering unwanted emisison related requirements are only applicable for corresponding max backscatter power and when backscatter power is larger than the power upper limit, the unwanted emission would exceed corresponding limit. The opponent thinks the power limit has been implicitly defined as testing input power and A-IoT use licensed band which don’t need to limit power upper bound.

From our point of view, it’s better not limit the upper bound and leave enough space for real deployment. Besides, it’s very hard to define one uniform upper limit. For example, the max backscatter power = input CW level - loss. In spec, we only define minimum loss requirements but commercial devices will have better loss performance. One issue is which loss should be used, if we use the minimum loss requirements(15dB) which means actual backscatter power must be larger than the backscatter power upper bound. Otherwise if we use ideal loss (6dB for OOK) to derive backscatter power upper limit, it means most device can’t achieve such power and it is not reasonable to use it for unwanted emission, then it seems meaningless for such power limit. So it seems very hard to define power upper limit.

Observation 1: it seems hard to define one uniform backscatter power upper limit. Proponent are suggested to propose some reasonable value for this power limit.

2.1 Rx requirements

- REFSENSE

Issue 3-4-1: Reference sensitivity Agreement: - EIS at peak antenna gain direction is -34dBm. Additional peak EIS levels can be added in the future if needed - EIS spherical coverage requirement is defined as sensitivity over partial sphere, i.e. XdBm at Y solid angle range partial sphere . Y is suggested to be +-45 degree with respect to the bore sight direction (or UE declared direction), X is suggested to be 5.5dB worse than peak EIS. - For testing - 90% success rate. - Use CFRA for REFSENS test procedure - Allow set time + SFO for the device response time. - Leave number of repeats to reach 90% success rate for the RAN5 to determine. - Further discuss if false alarm test can be considered in demod. - FRC is listed as below (note: it does not mean UE should be tested for all specified FRC) SCS; 15; 15; 15; 15. PRB; 1; 2; 3; 4. SIP; Bit length; 8; 8; 8; 8. M_SIP; 4; 4; 4; 4. OOK. OFDM; 2; 2; 2; 2. CAP; Bit length; 4; 4; 4; 4. M; 2; 2; 2; 2. OOK. OFDM; 2; 2; 2; 2. PRDCH; TBS; Depending on the size of the MAC PDU of A-IoT paging message indicating CFA. CRC; 16; 16; 16; 16. Line encoding. OOK. M; 6; 6; 6; 6. postamble; Bit length; 4; 4; 4; 4. M; 6; 12; 2; 2. chip number except for SIP, padding; 228; 228; 228; 228. Padding; 6; 12; 2; 2.

- The remaining issues for REFSENSE is M value, the PRB configuration and FRC PRDCH TBS.

- For the M value, at first they are the same for CAP, PRDCH, postamble. Following list our simulation results for different M value. The SNR corresponding to 0.1 BLER is 3dB difference between M=2 and M=4, and 1.5dB difference between M=4 and M=6. M =24 is the worst case but is not the typical value for REFSENSE which will only used when device is at lowest data rate. from reducing testing time point of view, M =6 or 8 are both OK for us

- Observation 2: M=6 seems OK for REFSENSE testing.

- For the PRB configuration, the R2D signal waveform will have better signal performance as larger PRB configuration with the same M value. From this point of view, it’s OK from our side to only maintain 1PRB and delete 2/3/4PRB configurations.

- Proposal 1: RAN4 can only retain 1PRB and delete 2/3/4PRB configurations for measurement channel.

- For TBS, annex lists RAN2 agreements for reference. The TBS include following parts. So the total TBS is suggested as 224bits based on current agreements. But as listed in RAN2 conclusion, they needs to further check whether additional information from SA3 is needed or not. So RAN4 is suggested to wait for final conclusion from RAN2

- R2D message Type: 3 bits

- R2D TBD + AT: 8 bits

- Paing ID length: 8 bits

- Paging ID: 177 bits

  - The AIoT device permanent ID include following 5 parts:

    - -ID Type

    - -PLMN ID (optional)

    - -NID (Optional)

    - -Third Party Identifer (Optional)

    - -Identification Information

  - It’s suggested to list all above information as in TBS calculation. So the total AIoT device permanent ID is 177bits.

- D2R scheduling info: 24 bits

- Fill Bits: 4bits

- Observation 3: based on current RAN2 conclusion, TBS is 224bits totally.

- Proposal 2: wait for RAN2 conclusion of whether SA3 information should be added or not before concluding TBS.

# 3. Conclusions

In this contribution, RF requirements for device 1 are discussed with following observations and proposals.

Observation 1: it seems hard to define one uniform backscatter power upper limit. Proponent are suggested to propose some reasonable value for this power limit.

- Observation 2: M=6 seems OK for REFSENSE testing.

- Proposal 1: RAN4 can only retain 1PRB and delete 2/3/4PRB configurations for measurement channel.

- Observation 3: based on current RAN2 conclusion, TBS is 224bits totally.

- Proposal 2: wait for RAN2 conclusion of whether SA3 information should be added or not before concluding TBS.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

Annex: RAN2 conclusion

From 38.300:

For CFA, the A-IoT device shall use the dedicated resource provided in A-IoT paging message to send the D2R Upper Layer Data Transfer message as shown in Figure 16.x.5.3-1(b). The A-IoT device does not need to monitor the Access Trigger message.

From 38.391:

For CFA, the following fields are further included:

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bit. The length of the field is 8 bits.

-	Paging ID: This field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). This field is of variable length.

D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 24 bits.

-	Fill Bits: This field is of variable size, and can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the MAC entity shall ignore the values of this field.

From R2-2505006:

From chairman notes: