# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513369

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.15.2.2

Source:	China Telecom

Title:	Discussion on RRM performance requirements for Adaptation of common signal/channel transmission

Document for:	Discussion

- Introduction

In RAN4 #116 meeting, RRM performance requirements for Adaptation of common signal/channel transmission were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for Adaptation of common signal/channel transmission captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD

For SSB adaptation, test cases for L1 measurement with SSB adaptation and L3 measurement with SSB adaptation can be designed.

Proposal 1: RAN4 to design the following SSB adaptation test cases

- L3 intra-frequency measurement

- L1-RSRP

- L1-SINR

- CBD

Status in the WF [1]:

Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

For general principles, FR1 and FR2 can be considered for test case design. Besides, the principles on only testing non-DRX mode, without SSB time index detection can be considered. And the test cases for SSB adaptation can be defined in RRC_CONNECT mode only, and test case for SMTC and SSB periodicity change is defined based on DCI 2-9.

Proposal 2: RAN4 to design the SSB adaptation test case with the following principles

- Both FR1 and FR2

- Only test non-DRX mode

- Without SSB time index detection

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9

- Conclusion

The contribution presents our views on RRM performance requirements for Adaptation of common signal/channel transmission, with the following proposals:

Proposal 1: RAN4 to design the following SSB adaptation test cases

- L3 intra-frequency measurement

- L1-RSRP

- L1-SINR

- CBD

Proposal 2: RAN4 to design the SSB adaptation test case with the following principles

- Both FR1 and FR2

- Only test non-DRX mode

- Without SSB time index detection

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9

- References

- R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #116, 25 – 29 August, 2025.

- R4-2509063, Topic summary for [116][220] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #116, 25 – 29 August, 2025.