# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513371

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.16.3

Source:	China Telecom

Title:	Discussion on RRM performance requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #116 meeting, RRM performance requirements for LP-WUS/WUR were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for LP-WUS/WUR captured in the WF [1] and summary [2].

- Discussion

LP-WUR test case design

Status in the summary [2]:

Issue 3-2-1: Test case scenarios - Proposals - P1: Consider test case for the following scenarios: 1: LP-WUR monitoring related test case; 2: RRM offloading related test case; 3: RRM relaxation related test case (CATT China telecom vivo) - P2: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.(Ericsson Nokia) Recommendations:

In the core part, issues including LP-WUR monitoring, RRM offloading and RRM relaxation are mainly discussed in RAN4 RRM meetings, related test cases need to be designed.

Proposal 1: It’s proposed to design test cases for LP-WUR monitoring, RRM offloading and RRM relaxation.

Status in the summary [2]:

Issue 3-2-2: Test case for OOK based LR and OFDB based LR - Proposals - P1: RAN4 to consider defining separate test cases for PSS/SSS based LR and LP-SS based LR. (Apple oppo CMCC China telecom vivo Huawei Nokia) - P2: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE (Ericsson) - P3: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB.(Ericsson) - P4: Discuss how to define a test case / configuration for OFDM capable UE (FG-62-1a) which is only configured with LP-RSRP & LP-RSRQ (no SS-RSRP, SS-RSRQ configuration). (Nokia) Recommendations: Suggest to agree: PSS/SSS based LR and LP-SS based LR should be verified separately in some scenarios.

In the core part, LP-WUR measurement requirements are defined based on PSS/SSS and LP-SS, in order to verify LP-WUR measurement requirements based on the two signals, it’s proposed to introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR.

Proposal 2: RAN4 to consider defining separate test cases for PSS/SSS based LR and LP-SS based LR.

Status in the summary [2]:

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3 - Proposals - Entry/exit condition evaluation verification - Option 1: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions (Apple CMCC vivo Huawei QC) - Option 2: Define test case for entry scenario (oppo Nokia) - Option 2-1 (oppo) - Scenario 1: UE enter Case 1 from legacy and exit to legacy - Scenario 2: UE enter Case 3 from legacy and exit to legacy - Scenario 3: UE enter Case 1 from Case 3 - Option 2-2: Use test mode / procedure for LR entry measurements (Nokia) - Option 3: Not to define test cases for the evaluation requirements for Entry/Exit between different cases (Case#1 and Case#3).(MTK) - Detail on exit condition evaluation verification: - P1: Define following tests for exit criteria evaluation (CMCC) - LR exit criteria evaluation test when exit from LP-WUS monitoring - LR exit criteria evaluation test when exit from Case 1 - FFS exit criteria evaluation test when exit from Case 3, wait the RAN2’s conclusion of exit condition - P2: Define test cases for evaluation of exit conditions for the following cases. (Huawei) - Case 1: UE exist Case 1 to legacy - Case 2: UE exist Case 3 to legacy - P3: Introduce the following test cases where the exit conditions can be tested (QC) - UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode - UE meets the cell resection requirements with initial conditions being WUR operating in the relaxed measurement mode. Recommendations: Suggest to agree the following based on majority view: only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions

For entry/exit conditions for case 1/case 3, when the entry conditions are met, it’s up to UE whether to enter the state, therefore, entry/exit conditions for case 1/case 3 are not testable. It’s proposed to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions.

Proposal 3: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions.

Status in the summary [2]:

Issue 3-2-4: Test case design for RRM relaxation - Proposals - P1: For RRM relaxation, the following tests can be considered: (CATT oppo CMCC vivo Huawei Ericsson) - intra-frequency cell re-selection - inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection - inter-RAT cell re-selection - P1-1: whether to introduce all the former 3 test cases for each LR type, i.e., the LP-SS based LR or the SSB based LR, could be further discussed. (vivo) - P2: In case 3, the test cases for cell reselection shall be considered and the specific number of test cases could be discussed in this meeting (ZTE) - P3: Introduce reselection test case(s) for PSS/SSS and LP-SS considering intra-frequency, inter-frequency and both, MR relaxation / offloading cases. - P4: RAN4 to discuss whether Rel-16 test cases for RRM relaxation based on cell reselection can be applied for Case#3. (MTK) Recommendations:

In our views, it’s meaningful to define test cases to verify cell re-selection performance for RRM relaxation, including intra-frequency cell re-selection, inter-frequency cell re-selection, and inter-RAT cell re-selection.

Proposal 4: For RRM relaxation, the following test cases can be considered:

- intra-frequency cell re-selection

- inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

- inter-RAT cell re-selection

- Conclusion

The contribution presents our views on RRM performance requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: It’s proposed to design test cases for LP-WUR monitoring, RRM offloading and RRM relaxation.

Proposal 2: RAN4 to consider defining separate test cases for PSS/SSS based LR and LP-SS based LR.

Proposal 3: RAN4 to consider only define test cases for exit conditions, do not define dedicated test cases for evaluation of entry conditions.

Proposal 4: For RRM relaxation, the following test cases can be considered:

- intra-frequency cell re-selection

- inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

- inter-RAT cell re-selection

- References

- R4-2512139, WF on RRM requirements for NR_LPWUS, vivo, RAN WG4 Meeting #116, 25 – 29 August, 2025.

- R4-2509065, Topic summary for [116][222] NR_LPWUS, vivo, RAN WG4 Meeting #116, 25 – 29 August, 2025.