3GPP TSG-RAN WG4 Meeting #116bis	R4-2513373

Prague, Czech Republic, October 13th – 17th, 2025

Source:	Tejas Networks

Title:	Discussion on RRM measurement prediction

Agenda item:	7.12.3

Document for:	Discussion

Introduction

The study on AI/ML mobility in NR were concluded in RAN4#116 in the scope of Rel-19 SI. The study outcome is captured in TR 38.744 V1.2.0 [3].

Discussion

The RRM specification impacts are captured in TR 38.744 section 6, and few of the potential impacts are discussed in detail below.

LCM functionality management

From Rel-19 study conclusions: - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note 2: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

The work item objective states that the LCM frame work should be, as far as possible, be based on the Rel-19 AI/ML for NR air interface work. And Rel-19 AI/ML for air interface work item considered two different types of approach for LCM procedures. Functionality based LCM and model-based LCM. AI/ML mobility study item concluded to consider only functionality-based LCM, that is, model-based LCM is not considered. In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models 	may not be identified at the Network, and it is up to UE’s implementation to decide on model related choices.

Observation 1: Only functionality-based LCM is considered for LCM procedures.

Observation 2: Model related choices are up to UE implementation.

Rel-19 study on AI/ML based mobility concluded with the recommendations of RRM and measurement event predictions for normative work. Furthermore, the following scenarios and/or sub-cases are recommended for normative work:

- For UE sided model (RRM and measurement event prediction), intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case;

- For network sided model (RRM prediction), at least RRM sub-case 2 of intra-frequency temporal domain case A for inference input report and all scenarios and sub-cases for data collection.

Proposal 1: RAN4 shall specify the handling of LCM for all agreed scenarios and/or sub-cases.

Cluster-based prediction

Rel-19 Study conclusions: - It is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach, RRM sub-use cases and OW length - Both cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) were used by different companies. Both approaches are valid implementations.

As per the Rel-19 study, using a group of measurement values as input for prediction can improve RRM measurement accuracy compared to using a single measurement. And the Rel-19 study concluded that, both cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) are valid implementation. Furthermore, it is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach.

Cluster approach example as per TR 38.744,

  - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

In line with the above conclusions, for a UE configured to predict the measurement values for a specific cell or frequency, the AI/ML model may use actual measurement values from another cell or frequency along with predicted measurement values from another cell or frequency as inputs to the model. Cluster based approach can benefit from incorporation of both actual and predicted measurement results, as a result enhance the model’s ability to generalize and improve performance in predicting the target measurements.

Proposal 2: Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values.

In scenarios where cluster-based approach is applied, measurement prediction may depend on inputs derived from other cells or frequencies, which can include either actual measurement values or predicted measurement values. Therefore, how to handle LCM in such cases needs to be clearly defined to ensure consistency of prediction and to avoid unnecessary fallback or model switching.

Proposal 3: RAN4 to specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: Only functionality-based LCM is considered for LCM procedures.

Observation 2: Model related choices are up to UE implementation.

Proposal 1: RAN4 shall specify the handling of LCM for all agreed scenarios and/or sub-cases.

Proposal 2: Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values.

Proposal 3: RAN4 to specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference.

- References

- RP-251968, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, Beijing, China, Sept. 15-18, 2025.

- RP-251965, Status Report to TSG, Beijing, China, Sep. 15-18, 2025.

- 3GPP TR 38.744 V1.2.0 (2025-9)