3GPP TSG-RAN WG4 Meeting #116bis	R4-2513374

Prague, Czech Republic, October 13th – 17th, 2025

Source:	Tejas Networks

Title:	Discussion on measurement event prediction

Agenda item:	7.12.4

Document for:	Discussion

Introduction

The study on AI/ML mobility in NR were concluded in RAN4#116 in the scope of Rel-19 SI. Agreements achieved during this meeting are captured in TR 38.744 V1.2.0 [3].

Discussion

The RRM specification impacts are captured in TR 38.744, and some of the relevant agreements are given below.

From the outcomes of Rel-19 study: UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each reported cell in the measurementReport message: - For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported; - For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported; - For inter-frequency prediction, the latest predicted measurement result is reported. It is up to UE’s implementation to decide on choice between indirect and direct event prediction methodology. On top of inference parameters captured in section 6.1.2.1.2, event-related parameters e.g. event type are part of inference configuration of measurement event prediction. UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information. When UE is configured with intra-frequency temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact. For indirect event prediction, RSRP difference can be used as performance monitoring metric. NOTE 1: The spec impact captured in section 6.1.2.1 is applicable for measurement event prediction unless otherwise described explicitly in this section. NOTE 2: A single framework is aimed for direct and indirect event prediction. NOTE 3: There is no consensus on the feasibility of performance monitoring of direct event prediction. it should be concluded before proceeding with normative work.

Measurement prediction use cases

AI/ML based mobility was extensively studied in Rel-19, and the conclusions are captured in TR 38.744. The study concluded that the UE can be configured with periodic or event-triggered reporting of predicted and/or actual measurement results. For intra-frequency temporal domain case A, one or more predicted measurement results across a prediction window can be reported, while case B allows reporting of the latest predicted or actual result. For inter-frequency prediction, the latest predicted result is reported. RRM measurement prediction can be performed using either UE-sided model or network-sided model. To support measurement event prediction, specification enhancements can be considered only for UE-sided model. How to predict measurement event using a network sided model is up to network implementation without specification impact.

Observation 1: In temporal domain case A, more than one predicted measurement result across a window can be reported.

UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). In the case of temporal domain Case A prediction, this configuration may include multiple predicted measurement reports within a defined time window. Such predictive reporting enables the network to anticipate future events, which enhances the accuracy and timeliness of measurement reporting.

This proactive approach allows the network to prepare in advance for mobility related actions, significantly reducing handover delays and failures. Rel-19 study on AI/ML mobility have shown that predicting future measurement instances benefits handover performance. Furthermore, anticipation of measurement events can help network to make informed and proactive mobility decisions, leading to improved robustness, fewer unnecessary handovers, and better user experience.

Furthermore, the prediction of future events can also reveal which target cell is likely to be optimal at a future time. This information can be valuable not only for mobility management but also for efficient resource preparation at the network side. RAN4 may consider extending this further to include prediction of future events, as this can enable shortlisting of potential target cells in advance. And UE may provide additional information to the network (e.g., candidate cells, confidence levels) to further optimize handover.

Observation 2: Event prediction for future time instances improves handover performance by improving robustness, fewer unnecessary handovers, and better user experience.

Observation 3: Prediction of future events can provide information about the future target cell.

Proposal 1: Based on future event prediction, RAN4 shall consider shortlisting potential target cells for handover.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: In temporal domain case A, more than one predicted measurement result across a window can be reported.

Observation 2: Event prediction for future time instances improves handover performance by improving robustness, fewer unnecessary handovers, and better user experience.

Observation 3: Prediction of future events can provide information about the future target cell.

Proposal 1: Based on future event prediction, RAN4 shall consider shortlisting potential target cells for handover.

- References

- RP-251968, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, Beijing, China, Sept. 15-18, 2025.

- RP-251965, Status Report to TSG, Beijing, China, Sep. 15-18, 2025.

- 3GPP TR 38.744 V1.2.0 (2025-9)